PROJECT JARVIS

THE COMPLETE ARCHIVE

Build · Wire · Code · Troubleshoot · Exhibit

4-Board Edge-Computing Android · 4-Foot Boxy Humanoid

Wood & Reinforced Cardboard Build

Lead Architect: Vikhyat Gupta

Fabrication Team: Aayush Agarwal · Shresth Adukia · Adhyansh Kumar · Rudraksh Sharma

Architect Division · St. Anthony's School · Target: September 16th

Anyone holding this document has everything needed to build, wire, code & exhibit Jarvis.

# CONTENTS

Project Jarvis — Master Reference · 131 pages

How to Use This Document  — Safety rules, structure guide, beginner notes	4

Fabrication Roadmap  — Month-by-month timeline from materials to exhibit day	5

Wiring Overview Diagram  — System block diagram, power domains, colour convention	6

Quick Reference Card  — Startup/shutdown sequences, LED states, fast fixes — print & laminate	8

PART 1  — Philosophy & System Architecture	10

1.1–1.2  Engineering Philosophy & Mechanical Anatomy	10

1.3–1.5  Power Architecture, Data Routing & Multi-Core Brain	11

1.6–1.8  LED Vibe System, Safety Architecture & Fail-Safes	12

PART 2  — Complete Build	13

Phase 0  Complete Materials & Tools	13

Phase 1–6  Core Mechanical Assembly	16

Phase 7  Electronics Mounting	18

Phase 8  Painting & Finishing	18

Phase 9  Physical Build Verification	19

PART 3  — Code & Wiring	20

Phase 10  Raspberry Pi Setup	20

Phase 11  Arduino Uno & Mega Code Upload	22

Phase 12  Raspberry Pi Pico Code	25

Phase 13  Brain Code on Pi (jarvis.py)	27

Phase 14  Complete Wiring Reference	31

Phase 15  First Power-Up & Testing	34

Phase 16  Pre-Exhibit Checklist	36

Phase 17  WebSocket Control Panel & Workshop HUD  — NEW	38

PART 4  — Deep Reference	49

Component Deep Dive  — B1–B13	49

Electricity Fundamentals  — A1–A4	57

Theory  — H1–H5	59

Tool School  — C1–C5	62

Code Explained Line by Line  — D1–D5	65

PART 5  — Exhibit Operations	70

F1–F16  Troubleshooting Guide	70

G1–G7  Exhibit Day Survival	78

FAQ  — Everything a Beginner Needs to Know (40+ questions)	83

Glossary  — Every technical term defined A–Z	90

End of Archive  — Final page of the complete document	131

# How to Use This Document

This master reference compiles all Project Jarvis documentation into one complete guide — from the first screw to the live exhibit. It merges four source documents with all redundancy removed, and adds a complete WebSocket control interface section.

If you have never built electronics before — this document is written for you. Every concept is explained from first principles. You do not need prior experience with Raspberry Pis, Arduinos, soldering, or code. You need attention, care, and the willingness to re-read a section if something is unclear.

### How This Document Is Organised

| Part | What It Covers | Who Needs It |
| --- | --- | --- |
| PART 1 — Philosophy | Why every design decision was made. Read first to understand WHY before starting. | Everyone. Understanding the why prevents the most common mistakes. |
| PART 2 — Build (Phases 0–9) | Physical construction: materials, tools, and step-by-step assembly of every part. | Fabrication team. Follow sequentially — each phase assumes the previous is complete. |
| PART 3 — Code & Wiring (Phases 10–17) | Setting up all four boards, writing code, connecting every wire, first power-up, and WebSocket control. | Lead Architect and anyone handling electronics. |
| PART 4 — Deep Reference | Full explanations of every component, electricity concept, tool, and code line. | Read when you want to understand something deeply, or when troubleshooting. |
| PART 5 — Exhibit Operations | Troubleshooting guide, exhibit day procedure, visitor Q&A scripts, and full glossary. | Everyone on exhibit day. |

### Safety Rules — Non-Negotiable

⚠ WARNING RULE 1: NEVER connect the 10000mAh power bank (motor domain) to the L298N until Phase 15 explicitly instructs you to. RULE 2: ALWAYS use a multimeter to check continuity before powering any new circuit. RULE 3: NEVER pull the power bank from a running Raspberry Pi — always run 'sudo shutdown -h now' first. RULE 4: Incorrect wiring destroys boards in milliseconds. Check twice before connecting anything. RULE 5: The 12V positive rail from the power bank or boost converter NEVER connects to the Pi, Uno, Mega, or Pico. Only grounds connect.

### If You Have Never Used These Tools Before

Do not panic. The Tool School section in Part 4 covers every tool you will use, including how to hold it correctly, the most common mistakes, and how to avoid injury. Read Tool School before Phase 1.

# Fabrication Roadmap

Month-by-month timeline from materials procurement to exhibit day. Each milestone assumes all previous milestones are complete.

| Month / Week | Milestone | Key Deliverable |
| --- | --- | --- |
| Month 1, Week 1–2 | Procurement & Tool School | All materials purchased. Tools ready. Team trained on jigsaw, drill, soldering. |
| Month 1, Week 3–4 | Phase 0–2: Base & Legs | Chassis cut and assembled. Drive motors mounted. Spine installed. Robot rolls. |
| Month 2, Week 1–2 | Phase 3–6: Body, Arms, Head, Arc Reactor | Torso built. Arms attached. Head mounted on spine. NeoPixel ring tested. |
| Month 2, Week 3 | Phase 7–9: Electronics Mounting & Painting | All boards mounted on standoffs. Cable pre-routes done. Paint and finish applied. |
| Month 2, Week 4 | Phase 10–13: All Code Loaded | Pi setup. All 4 boards programmed. jarvis.py created. Gemini key installed. |
| Month 3, Week 1 | Phase 14–15: Full Wiring & First Power-Up | All wires connected. Multimeter verified. All 4 function tests passed. |
| Month 3, Week 2–3 | Phase 16–17: Final Checks & WebSocket HUD | Pre-exhibit checklist passed. WebSocket server running. Workshop HUD live. |
| Month 3, Week 4 | Exhibit Day | Full demo. All team roles briefed. Oh-Shit kit packed. Jarvis ready. |

# Wiring Overview Diagram

### System Block Diagram — Two Power Domains

╔══════════════════════════════════════════════════════════════╗ ║ CLEAN POWER DOMAIN (5V) ║ ║ ║ ║ 20,000mAh Power Bank (5V USB) ║ ║ │ ║ ║ ▼ ║ ║ Raspberry Pi 3B+ ◄── USB Webcam (Port 4) ║ ║ │ │ │ ║ ║ │ │ └── USB Port 3 ──► Raspberry Pi Pico ║ ║ │ │ │ ║ ║ │ │ └──► NeoPixel Ring ║ ║ │ │ ║ ║ │ └── USB Port 2 ──► Arduino Mega (The Muscle) ║ ║ │ │ PWM/DIR ║ ║ │ ▼ ║ ║ │ L298N ◄──── DIRTY POWER ║ ║ │ (See below) ║ ║ │ ║ ║ └── USB Port 1 ──► Arduino Uno (The Bouncer) ║ ║ │ Pin 2 ║ ║ └──► Pi GPIO 17 (Wake signal) ║ ╚══════════════════════════════════════════════════════════════╝ ╔══════════════════════════════════════════════════════════════╗ ║ DIRTY POWER DOMAIN (12V) ║ ║ ║ ║ 10000mAh Power Bank ──► DC-DC Boost (5V→12V) ──► Kill Switch ──► L298N ║ ║ │ ║ ║ OUT1/2 ──► Left Johnson Motor ║ ║ OUT3/4 ──► Right Johnson Motor║ ║ ║ ║ COMMON GROUND BRIDGE: ║ ║ 12V Battery (–) ──┐ ║ ║ ├──► L298N GND terminal ║ ║ Arduino Mega GND ─┘ ║ ╚══════════════════════════════════════════════════════════════╝

### Wire Colour Convention

| Colour | Meaning | Examples |
| --- | --- | --- |
| Red | Positive power (+) | 5V to sensor VCC, battery (+) to kill switch |
| Black | Ground (–) | All GND connections |
| Orange | Wake-up / PWM signals | Uno Pin 2 → Pi GPIO 17; Mega Pin 3 → L298N ENA |
| Yellow | Ultrasonic TRIG (out) | Arduino Pin 9 → HC-SR04 TRIG |
| Green | Ultrasonic ECHO (in) | HC-SR04 ECHO → Arduino Pin 10 |
| White | Motor direction (IN1/IN2) | Mega Pin 4/5 → L298N IN1/IN2 |
| Purple | Motor direction (IN3/IN4) | Mega Pin 6/7 → L298N IN3/IN4 |
| Data | NeoPixel data wire | Pico GP28 → NeoPixel DIN |

ℹ NOTE The two power domains share ONLY the common ground wire. The 12V positive rail NEVER touches any 5V component.

# Quick Reference Card

PRINT THIS PAGE · LAMINATE IT · KEEP WITH JARVIS AT ALL TIMES

### STARTUP SEQUENCE (Always in this order)

| Step | Action | Wait for... |
| --- | --- | --- |
| 1 | Connect 20,000mAh power bank to Pi micro-USB | 60–90 seconds for Linux boot |
| 2 | SSH into Pi from laptop | 'pi@jarvis:~$' prompt |
| 3 | Run: ls /dev/ttyACM* | 3 ports: ttyACM0, ttyACM1, ttyACM2 |
| 4 | Connect 10000mAh power bank (12V output) to L298N via kill switch | Motors now live |
| 5 | Run: python3 jarvis.py | 'Jarvis is ready. Waiting for wake signal...' |

### SHUTDOWN SEQUENCE (Always in this order)

| Step | Action | Why |
| --- | --- | --- |
| 1 | Disconnect 10000mAh power bank from L298N (or flip kill switch OFF) | Prevents accidental movement |
| 2 | Ctrl+C in SSH terminal to stop jarvis.py | Stops all active processes |
| 3 | Run: sudo shutdown -h now | Linux safely closes filesystem |
| 4 | Wait 15–20 seconds for green Pi LED to stop blinking | Confirms disk writes complete |
| 5 | Disconnect power bank | Only after LED activity stops |

### LED STATES — Arc Reactor Colour Guide

| Command | Colour | Meaning | When |
| --- | --- | --- | --- |
| B | BLUE | Listening | Microphone active — Jarvis is hearing |
| G | GOLD | Thinking | AI processing — Gemini API call in progress |
| W | WHITE | Speaking | TTS active — Jarvis is responding |
| H | PINK | Secret ♥ | Special command triggered |
| X | OFF | Idle | Waiting for wake signal |

### FAST FIX GUIDE

| Symptom | Most Likely Cause | 60-Second Fix |
| --- | --- | --- |
| Robot won't move | 10000mAh power bank disconnected, not charged, or motors in kill-zone | Reconnect battery; check nothing within 30cm of front sensor |
| LEDs stuck/wrong colour | jarvis.py crashed or Pico port wrong | SSH: Ctrl+C, then python3 jarvis.py |
| No voice recognition | Wi-Fi dropped or microphone issue | Check ping google.com; reposition webcam microphone |
| Jarvis won't stop talking | pyttsx3 stuck | SSH: sudo pkill python3, restart jarvis.py |
| Complete freeze | Pi crashed | SSH: sudo reboot (tell visitors: 'updating neural pathways') |
| Motor battery dead mid-demo | 10000mAh power bank discharged or output not set to 12V | Recharge or swap the 10000mAh power bank; verify 12V output before reconnecting |
| Pi won't boot | SD card issue or cable | Re-seat microSD; check cable and port (must be 2.1A) |

### EMERGENCY SHUTDOWN — HARDWARE ONLY

If SSH is unavailable and Jarvis is misbehaving: press the RED KILL SWITCH on the back panel. This physically cuts all motor power. If Pi also needs stopping, pull the power bank cable from Pi's micro-USB port — but only as a last resort (may corrupt SD card).

# PART 1 — PHILOSOPHY & SYSTEM ARCHITECTURE

## 1.1 — Engineering Philosophy

Project Jarvis exists to demonstrate that a functional, intelligent, interactive robot can be built by students using accessible materials and open-source software. Every design decision prioritises: reliability over features, safety over speed, and understanding over complexity.

The design philosophy is summarised in three rules: (1) Every subsystem must be independently testable. (2) Every safety mechanism must be hardware-level, not software-level. (3) Every design choice must be explainable to a visitor in one sentence.

## 1.2 — Mechanical Anatomy

Jarvis is a boxy humanoid approximately 4 feet (122cm) tall. The skeleton is a 25mm PVC spine — a single vertical column from which everything hangs. This is inspired by vertebrate anatomy: the spine carries load and routes cables.

| Section | Material | Purpose |
| --- | --- | --- |
| Base chassis | 12mm plywood | Carries total robot weight. Rigid platform for drive motors and casters. |
| Leg boxes (2) | 6mm plywood | Structural bridging between base and torso. Houses kill switch and 10000mAh power bank. |
| Spine | 25mm PVC pipe, 90cm | Central load-bearing column. All body sections attach to this via PVC flange. |
| Torso | Laminated cardboard (3 layers) | Lightweight body shell. Cable-routed before assembly. |
| Arms (2) | 6mm plywood + foam | Cosmetic. Fixed. Adds humanoid silhouette. |
| Head | 6mm plywood box | Houses camera eye, decoy eye, speaker. Mounted on top of spine. |
| Arc reactor | NeoPixel ring + diffuser | Chest-mounted LED indicator. Shows system state in real time. |

### Six-Point Tripod Stability

Two drive wheels (rear) + four ball casters (front corners and rear corners). Drive wheels are set 1–2mm lower than caster ball contact height. This means all weight rests on drive wheels when level; casters catch Jarvis if it tips. This is the zero-wobble secret.

## 1.3 — Power Architecture

Jarvis uses TWO completely isolated power domains. This is the most important architectural decision in the build:

| Domain | Source | Powers | Why Isolated |
| --- | --- | --- | --- |
| Clean (5V) | 20,000mAh USB power bank | Raspberry Pi → Uno, Mega, Pico, Webcam via USB | Logic boards need stable, noise-free power. |
| Dirty (12V) | 10000mAh power bank (12V output or with boost converter) | L298N motor driver → Johnson Motors only | Motors generate electrical noise that corrupts sensor readings. |
| Common Ground | Single wire: Mega GND ↔ L298N GND ↔ Power bank/boost converter GND (–) | Signal reference for both domains | Without this, motor commands from Pi read as garbage. |

⚠ WARNING The 12V positive rail NEVER connects to any 5V component. Only ground wires bridge the two domains. Connecting 12V to the Pi, Uno, Mega, or Pico destroys the board instantly — there is no recovery.

## 1.4 — Data Routing: USB Star Topology

The Raspberry Pi is the central hub (star). Every other board connects directly to the Pi via USB — there is no daisy-chaining.

| Pi USB Port | Connected Board | Purpose |
| --- | --- | --- |
| Port 1 | Arduino Mega (ttyACM0) | Receives motor commands (F/B/L/R/S) |
| Port 2 | Arduino Uno (ttyACM1) | Sends wake signal to Pi GPIO 17 |
| Port 3 | Raspberry Pi Pico (ttyACM2) | Receives LED colour commands (B/G/W/H/X) |
| Port 4 | USB Webcam | Provides camera and microphone input |

## 1.5 — Multi-Core Brain: Why 4 Boards

Linux (the Pi's OS) is non-real-time: it can be interrupted by background processes at any moment. Arduino has no OS and cannot be interrupted. This is the fundamental reason for multiple boards:

| Board | Its Only Job | Why Dedicated? |
| --- | --- | --- |
| Raspberry Pi 3B+ | AI, speech recognition, coordination | High-level reasoning needs a full OS. Cannot guarantee real-time response. |
| Arduino Mega | Motor control + 30cm kill-zone | Real-time hardware safety. Kill-zone must respond in <5ms — always. |
| Arduino Uno | 1-metre wake-up sentinel (24/7) | Ultra-low power. Never sleeps while Pi can idle to save battery. |
| Raspberry Pi Pico | LED animation only | Frees Pi CPU. LED timing is exact — no OS jitter. |

## 1.6 — LED Vibe System

The NeoPixel ring on Jarvis's chest communicates system state to visitors in real time. The arc reactor is not decorative — it is a live diagnostic indicator that visitors understand instantly without explanation.

## 1.7 — Safety Architecture

Three independent safety layers protect visitors and hardware:

- Kill-Zone (Hardware): The Arduino Mega's sensor detects any object within 30cm. When triggered, the Mega stops motors and ignores ALL commands from the Pi. The Pi is completely overridden. This is physical — not software.

- Brownout Protection: A 2200µF 25V capacitor recommended (1000µF minimum acceptable) across the L298N motor driver terminals absorbs motor startup current spikes. Without it, motor startup causes voltage sags that reset the Mega mid-command. With Johnson motors drawing up to 3–4A stall, the larger 2200µF value provides meaningfully better protection.

- Physical Kill Switch: A large red latching button on the back panel interrupts the 12V positive lead from the power bank (or boost converter output). Pressing it physically severs all motor power regardless of software state.

## 1.8 — Fail-Safes

- If the 10000mAh motor power bank runs out or disconnects, the 5V brain domain stays alive. jarvis.py detects the motor loss and speaks: 'Lead Architect, my drive systems have lost power.'

- If jarvis.py crashes, the Uno continues monitoring for visitors and the Pico holds its last LED colour — neither crashes silently.

- If Wi-Fi drops, Jarvis cannot access Gemini or Google STT, but can still wake up, greet, and respond with its offline fallback message.

# PART 2 — COMPLETE BUILD

## Phase 0: Complete Materials & Tools

Do not start building until everything on this list is in front of you. Stopping mid-build to hunt for supplies is how mistakes happen.

⚡ JOHNSON MOTORS QUICK REFERENCE

⚡ SPECIFICATIONS 12V rated ¦ 100 RPM at 12V ¦ 0.8–1.5A running ¦ 3–4A stall ¦ metal gearbox 🔧 KEY CHANGES FROM BO MOTORS • FUSE: Upgrade from 5A to 10A inline fuse • PWM: Increase default values from 150 to 180–200 • CAPACITOR: Use 2200µF 25V instead of 1000µF for brownout protection • HEAT: L298N runs hotter — ensure airflow around heatsink tab ⚠ WARNING SIGNS • L298N too hot to touch after 30s → check for stall or add fan • Robot barely moving → increase PWM (try 180–200) • One motor not spinning → L298N channel may be burned out; see F13

### 0.1 — Structural / Physical Materials

| Item | Qty / Spec | Notes |
| --- | --- | --- |
| Plywood 6mm | One 4×4 ft sheet | Structural panels, leg boxes, head box, arm mounts. |
| Plywood 12mm | One 2×2 ft sheet | Base chassis platform only — extra thickness for motor load. |
| Double-wall corrugated cardboard | 10–12 large sheets | Appliance store washing machine boxes — free and ideal. |
| PVC pipe 25mm diameter | 2 metres | Central spine. Buy extra — off-cuts are useful. |
| PVC flange plate 25mm | 1 | Mounts spine to base platform. Must match pipe diameter exactly. |
| 12V 100RPM Johnson motors (metal gears, higher torque) with wheels | 2 | Drive wheels. Confirm wheels are included. |
| Ball casters 20mm | 4 | Must sit 1–2mm higher than drive wheel ground contact — see Phase 1.3. |
| M4 bolts, nuts, washers (assorted) | One full pack | Used everywhere. 40 sets minimum. |
| Wood screws 25mm & 40mm | One box each | Panel-to-panel and structural joins. |
| M3 nylon standoffs 5mm + M3 bolts/nuts | Pack of 20+ | All circuit boards mount on these — prevents shorts. |
| PVA wood glue | 1 large bottle | Primary adhesive for all wood joints. |
| Hot glue gun + 30 sticks | 1 kit | Cardboard panel assembly and temporary fixings ONLY. Not structural. |
| Epoxy adhesive 2-part (Araldite) | 3 tubes | Motor mounts, PVC-to-wood, permanent metal-to-wood joints. |
| Craft foam sheets 3mm | 8 sheets | Panel detailing, neck collar, joint covers, eye frames. |
| Acrylic sheet 3mm clear + Acrylic dome 5cm | 30×30cm + 2 domes | Eye lenses and housings. |
| LED diffuser film | One A4 sheet | Arc reactor face. Without it, LEDs look like 12 separate dots. |
| Spray paint: gunmetal grey, matte black, silver | 2 cans each | Base coat and detail colours. |
| Acrylic craft paint | Assorted small pots | Hand-painted panel detail, weathering effects. |
| Varnish (matte) | 1 can | Seals and protects all painted surfaces. |
| Craft knife + cutting mat + metal ruler | 1 each | Essential for cardboard work. |
| Sandpaper 120-grit and 240-grit | 4 sheets each | Surface preparation for painting. |
| Masking tape | 3 rolls | Paint masking and temporary fixings. |
| Cable ties 100mm and 200mm | 100 of each | Cable management throughout. |
| Heat-shrink tubing assorted | 1 pack | Insulate all soldered joints. |
| PVC primer spray | 1 can | Adhesion promoter for painting PVC spine. |

### 0.2 — Electronics Components

| Component | Qty | Specification & Notes |
| --- | --- | --- |
| Raspberry Pi 3B+ | 1 | The Master Brain. Must be 3B+ — code assumes this USB port layout. |
| Arduino Uno R3 | 1 | The Bouncer. Standard Uno. |
| Arduino Mega 2560 | 1 | The Muscle. Must be Mega — Uno lacks sufficient PWM pins. |
| Raspberry Pi Pico | 1 | The Artist. Standard Pico (NOT Pico W). Runs MicroPython. |
| L298N Motor Driver Board | 1 | Controls 12V motors from 5V logic. Buy pre-mounted with heatsink. |
| NeoPixel LED Ring 12-LED WS2812B | 1 | The arc reactor. Must be WS2812B protocol. ~8cm diameter. |
| HC-SR04 Ultrasonic Sensor | 2 | S1 → Uno (wake-up, 1m range). S2 → Mega (kill-zone, 30cm). LABEL S1 AND S2 IMMEDIATELY. |
| USB Webcam with built-in mic | 1 | Plug-and-play USB. Logitech C270 recommended. |
| USB powered mini speaker | 1 | TTS voice output. 3W minimum for exhibit hall volume. |
| 10000mAh power bank (12V output or with boost converter) | 1 | 10,000mAh (10Ah) power bank — Powers the “Dirty” motor domain at 12V output. Verify the power bank has a 12V DC output port; if it only outputs 5V USB, pair with a 5V-to-12V DC-DC boost converter (3A minimum). |
| 20,000mAh 5V USB power bank | 1 | Powers all four logic boards. Must output 2.1A or more on at least one port. |
| Physical Kill Switch | 1 | Rated for 12V, 3A minimum. Large, red, latching button recommended. |
| MicroSD card 32GB Class 10 | 1 | Samsung EVO or SanDisk Ultra ONLY — cheap cards cause filesystem corruption. |
| 2200µF 25V capacitor recommended (1000µF 25V minimum acceptable) | 1 | Brownout protection across L298N power terminals. Longer leg = (+). 2200µF 25V recommended (1000µF minimum acceptable). Pair with 10A inline fuse for full Johnson motor protection. |
| Resistors 330Ω | 10 | Decoy eye LED current limiting. Buy a mixed pack. |
| Jumper wires male-to-male 20cm | 30 | GPIO connections. |
| USB-A to USB-B cables | 2 | For Arduino Uno and Mega to Pi. |
| Micro-USB cable | 1 | For Raspberry Pi power (Pi's own power port). |
| USB-A to Micro-USB cable | 1 | For Raspberry Pi Pico to Pi. |
| 22AWG hookup wire (red, black, various) | 5m each colour | All board-to-board wiring. Solid core for terminal blocks, stranded for flex points. |

### 0.3 — Tools Required

| Tool | Spec | Critical Use |
| --- | --- | --- |
| Jigsaw | Variable speed, with fine-tooth blade | Cutting all plywood panels |
| Power drill | Variable speed, with Phillips + HSS bit set | All screw holes and bolt holes |
| Soldering iron | 25–40W, 350°C tip, with stand and sponge | All terminal solder joints |
| Multimeter | DC voltage + continuity + resistance | Every wiring check before power-on |
| Wire strippers | Adjustable 20–28 AWG | Preparing all hookup wire |
| Needle-nose pliers | Standard | Nylon standoff installation |
| Phillips screwdrivers | #1 and #2 | All assembly |
| Flat screwdrivers | Small (3mm tip) | L298N terminal blocks — must be tight |
| Spirit level | 30cm minimum | Verifying spine is perfectly vertical |
| G-clamps / F-clamps | 2+ pairs | Holding joints during glue cure |
| Laptop | Any OS, Arduino IDE + Thonny installed | Code upload to all boards |

### 0.4 — Laptop Software (Install Before Build Starts)

- Raspberry Pi Imager — raspberrypi.com/software

- Arduino IDE 2.x — arduino.cc/en/software

- Thonny IDE — thonny.org

- PuTTY (Windows only) — putty.org (for SSH)

- On Pi (install via SSH after OS flash): speech_recognition, pyttsx3, google-generativeai, pyserial, RPi.GPIO, pyaudio

## Phase 1: The Base Chassis

### 1.1 — Cut the Base Platform

From 12mm plywood, cut one rectangle: 30cm × 40cm. This is the floor Jarvis stands on. All weight transfers through this panel.

Before cutting: verify the sheet is flat. Warped plywood produces a rocking chassis that no amount of caster adjustment will fix.

### 1.2 — Mount Drive Motors

- Mark two motor positions centred left-right, 8cm from the rear edge of the base. Mark the motor shaft hole position.

- Drill motor shaft holes (typically 6mm diameter for Johnson motor shafts — measure your motor’s shaft diameter before drilling).

- Use epoxy + M4 bolts to mount each motor bracket. Epoxy alone is insufficient — always bolt through.

- Verify both motor shafts are on exactly the same horizontal axis. Use a ruler across both shafts — they must touch.

### 1.3 — Ball Caster Installation — The 1–2mm Clearance Rule

This is the most important dimensional spec in the entire build. Get this wrong and Jarvis rocks, wobbles, or balances on casters instead of drive wheels:

⚠ WARNING Drive wheels must sit 1–2mm LOWER than ball caster contact height when robot is on level ground. Method: Install drive wheels. Place robot on flat surface. Slide a 1mm and 2mm feeler gauge (or two stacked credit cards) under one caster position. The caster ball should just touch the feeler gauge with the drive wheels bearing the weight. If casters sit lower than drive wheels: Jarvis rolls on casters. Drive wheels spin freely. Robot goes nowhere.

- Position 4 ball casters: two front corners, two rear corners. Bolt through base.

- Verify clearance rule before proceeding.

### 1.4 — Cross-Brace

Cut two strips of 6mm plywood: 4cm × 30cm. Glue and screw diagonally across the underside of the base. This eliminates flex under motor torque.

## Phase 2: Legs and Spine

### 2.1 — Leg Boxes

Cut 4 identical side panels from 6mm plywood: 12cm × 25cm. Assemble into two open-top boxes 12×12×25cm using PVA + 25mm wood screws. Pre-drill all holes — plywood edges split without pilot holes.

### 2.2 — Spine Installation

Cut PVC spine to 90cm. Drill a 26mm hole centred in the base platform. Push the PVC flange plate down from above and bolt it to the base with M4 bolts (4 corners of the flange). The spine drops into the flange and is epoxied.

💡 PRO TIP Use a spirit level against the spine after installation. It must be perfectly vertical. One degree of lean = the entire head is offset. Pack the flange with epoxy + shims before it cures to correct any lean.

## Phase 3: The Torso

### Reinforced Cardboard Lamination Technique

Three laminated layers create a near-rigid composite. Single layers are floppy. Three alternating layers resist bending in all directions:

- Cut three identical panel shapes from double-wall corrugated cardboard.

- Apply a thin, even layer of PVA glue to one sheet.

- Place the second sheet on top, rotating it 90° so internal corrugations are perpendicular to the first layer.

- Repeat for the third layer, rotating back to the original orientation.

- Place under heavy, flat weight (stack of books) and cure for minimum 2 hours.

💡 PRO TIP CRITICAL: Pre-Route Cables BEFORE closing the torso. Run all USB cables and the NeoPixel data wire through the interior before final assembly. Bundle neatly and tie to spine every 15cm. It is nearly impossible to add cables after the torso is closed. Seal all cardboard surfaces with 2 coats of PVA (50:50 water dilution) before spray painting. Unsealed cardboard absorbs paint unevenly and looks rough.

## Phase 4: The Arms

Cut arm shapes from 6mm plywood: two rectangular main arms (8cm × 25cm) and two forearm sections (6cm × 15cm). Sand all edges. Attach with epoxy and small M4 bolts through a corner bracket. Arms are purely cosmetic — they add humanoid silhouette but carry no structural load.

## Phase 5: The Arc Reactor

The NeoPixel ring mounts on the chest face. Cut a circular hole sized to the ring's outer diameter. The LED diffuser film goes behind the ring (between ring and chest wall) to blend individual LED dots into a smooth glow.

⚠ WARNING TEST the NeoPixel ring BEFORE sealing it into the chest. Connect it to the Pico temporarily and run the test code. Once epoxied in, replacing a dead ring requires significant disassembly.

## Phase 6: The Head

### Head Construction

Build the head box from 6mm plywood: 18cm × 14cm × 16cm (width × depth × height). Two circular eye holes: one for the webcam lens (left eye, 28mm diameter) and one for the decoy LED eye (right eye, 10mm diameter). The head attaches to the spine top via a 6mm plywood plate epoxied to the PVC.

### Eye Installation

- Camera eye: Remove webcam PCB from its housing (2 screws). Mount bare PCB + lens flush inside the head using M3 nylon standoffs. The lens must align exactly with the eye hole.

- Decoy eye: 5mm LED, anode through 330Ω resistor to Pi 3.3V (Pin 1), cathode to Pi GND (Pin 6). Sealed behind the acrylic dome.

- Speaker: Mount on the inside of the back head panel pointing outward through a grille cut from craft foam.

## Phase 7: Electronics Mounting

### 7.1 — Internal Layout (Bottom to Top)

Mount boards from the bottom up. Lower = heavier = lower centre of gravity = more stable.

| Tier / Height | Component | Why Here |
| --- | --- | --- |
| Bottom (leg box area) | 10000mAh Power Bank (12V output) + Kill Switch | Lowest possible position for maximum stability. Kill switch accessible on back panel. |
| Lower tier | L298N motor driver | Close to motor wire entry points. Short motor wires = less EMI. |
| Middle-lower tier | Arduino Mega | Short connections to L298N. USB cable runs up to Pi. |
| Middle tier | Arduino Uno | Short USB cable to Pi. |
| Upper-middle tier | 20,000mAh power bank | Accessible via maintenance door for charging/swapping. |
| Upper tier | Raspberry Pi 3B+ | Central position for short USB runs to all boards. Good airflow. |
| Arc reactor height | Raspberry Pi Pico | Adjacent to NeoPixel data wire exit. |

### 7.2 — Board Mounting Method

⚠ WARNING NEVER hot-glue boards directly to the chassis. Motor vibration will cause solder joints to fail within days of demonstration use.

- For each board, cut a plywood mounting plate slightly larger than the board.

- Drill holes and install M3 nylon standoffs. These lift the board 5mm above the plate, preventing short circuits.

- Securely screw each mounting plate to the internal structure (spine or torso walls).

- Bolt each board onto its standoffs using M3 bolts.

- Label every mounting plate: board name, designated Pi USB port, and function. Example: 'Arduino Mega — The Muscle — Pi USB Port 1'.

## Phase 8: Painting & Finishing

### Sequence (Never Skip Steps)

- Sand all plywood with 120-grit, then 240-grit. Clean off all dust.

- Seal all cardboard panels with 2 coats PVA 50:50 dilution. Allow 1 hour between coats.

- Apply PVC primer to the spine. Allow to dry fully.

- Mask off all electronics and lenses. Spray gunmetal grey as base coat on all panels. Two light coats, allow 20 minutes between.

- Hand-paint panel detail lines with matte black acrylic.

- Dry-brush silver acrylic on raised edges for metallic highlight effect.

- Remove masking. Install arc reactor ring behind diffuser film.

- Apply 2 coats matte varnish to all painted surfaces.

💡 PRO TIP Dry-brushing: Load brush, wipe 90% of paint off on paper towel, drag lightly across raised edges. The tiny remaining paint catches only the highest points.

## Phase 9: Physical Build Verification

Complete ALL checks before touching any software. A problem found here costs 20 minutes. A problem found after wiring costs 3 hours.

| Check | How to Verify | Pass Condition |
| --- | --- | --- |
| Chassis level | Place on flat floor. Check with spirit level. | No rocking. Level across all axes. |
| Drive wheel clearance | Lift slightly and spin each drive wheel by hand. | Both spin freely. No binding. |
| Caster clearance rule | Check with feeler gauge or credit cards. | Drive wheels 1–2mm lower than casters. |
| Spine vertical | Spirit level against spine from two perpendicular angles. | Perfectly vertical both ways. |
| All joints rigid | Pull gently on arms, head, leg box panels. | No movement, creak, or flex at any joint. |
| Cable routes clear | Trace each cable from board position to exit point. | No pinch points. No sharp bends. All pre-routed. |
| Eye alignment | Shine torch at camera eye from outside. | Lens clearly visible. No obstruction. No epoxy on lens. |
| Kill switch mounted | Operate kill switch 5 times. | Positive click each way. Accessible without tools. |

# PART 3 — CODE & WIRING

## Phase 10: Raspberry Pi Setup

### 10.1 — Flash the Operating System

- Download Raspberry Pi Imager from raspberrypi.com/software and install on your laptop.

- Insert the 32GB microSD card into your laptop.

- Open Imager. Click 'Choose OS' → 'Raspberry Pi OS (other)' → 'Raspberry Pi OS Legacy (32-bit)'. Use Bullseye, not Bookworm. The speech recognition and audio libraries are more stable on Bullseye.

- Click the settings gear icon (⚙️) before writing. Configure: Set hostname to 'jarvis'. Enable SSH (use password authentication). Set username 'pi' and a password you will remember. Configure Wi-Fi with your network SSID and password. Set locale/timezone. This pre-configures everything so the Pi connects to Wi-Fi and accepts SSH on first boot.

- Click 'Write'. Wait for write and verify to complete (approximately 5 minutes).

### 10.2 — First Boot & SSH Connection

- Insert microSD into Pi. Connect power bank to Pi micro-USB. Wait 60–90 seconds.

- On Mac/Linux: open Terminal, type: ssh pi@jarvis.local

- On Windows: open PuTTY, host = jarvis.local or the Pi's IP address, port 22.

- Accept the SSH fingerprint when prompted. Enter your password. You should see the 'pi@jarvis:~$' prompt.

💡 PRO TIP If jarvis.local doesn't resolve, log into your Wi-Fi router admin page and find the IP address of 'jarvis' in the connected devices list. Then SSH to that IP address directly.

### 10.3 — System Update

sudo apt update sudo apt upgrade -y

Allow 5–10 minutes. This is necessary to ensure all system libraries are current before installing Python packages.

### 10.4 — Install Python Audio Dependencies (Before Python packages)

sudo apt install portaudio19-dev -y sudo apt install python3-pyaudio -y

⚠ WARNING Install these system-level packages BEFORE attempting to install pyaudio via pip. Attempting pip install pyaudio without portaudio19-dev first will fail with a compilation error.

### 10.5 — Install Python Packages

pip3 install SpeechRecognition --break-system-packages pip3 install pyttsx3 --break-system-packages pip3 install google-generativeai --break-system-packages pip3 install pyserial --break-system-packages pip3 install RPi.GPIO --break-system-packages

Verify all installed:

pip3 list ¦ grep -E 'google¦speech¦pyttsx3¦pyserial¦RPi'

### 10.6 — Get Your Gemini API Key

- On your laptop, go to aistudio.google.com. Sign in with Google.

- Click 'Get API Key' → 'Create API Key'. Copy the key string.

- Save it temporarily in a text file on your laptop. You will paste it into jarvis.py.

💡 PRO TIP Gemini free tier is more than sufficient for exhibition use. No payment method needed. If you exceed the free quota during a demo, create a second Google account and get a new key — takes under 5 minutes.

## Phase 11: Arduino Code Upload

⚠ WARNING The Uno and Mega are programmed from YOUR LAPTOP using Arduino IDE, not from the Pi. They communicate with the Pi via USB serial AFTER being installed inside the robot. DO NOT upload code while the 10000mAh power bank (motor domain) is connected.

### 11.1 — Arduino IDE Setup

Download Arduino IDE 2.x from arduino.cc/en/software. Install on your laptop. Accept any driver installs on first run. On Windows, watch Device Manager for new COM ports appearing.

### 11.2 — Board 1: Arduino Uno — The Bouncer

Connect Uno to laptop via USB-A to USB-B cable. In Arduino IDE: Tools → Board → Arduino Uno. Tools → Port → select the COM port that appeared. Paste code below and click Upload.

// BOARD 1: ARDUINO UNO --- The Bouncer // Monitors Sensor 1 (1m wake-up range). Sends HIGH to Pi when visitor detected. int trigPin = 9; // Sends the ultrasonic pulse int echoPin = 10; // Receives the echo back int wakePin = 2; // Connects to Raspberry Pi GPIO 17 void setup() { pinMode(trigPin, OUTPUT); pinMode(echoPin, INPUT); pinMode(wakePin, OUTPUT); Serial.begin(9600); } void loop() { long duration, distance; digitalWrite(trigPin, LOW); delayMicroseconds(2); digitalWrite(trigPin, HIGH); delayMicroseconds(10); digitalWrite(trigPin, LOW); duration = pulseIn(echoPin, HIGH, 30000); distance = (duration == 0) ? 999 : (duration / 2) / 29.1; if (distance > 0 && distance < 100) { digitalWrite(wakePin, HIGH); // Wake the Pi! Serial.println("Wake!"); } else { digitalWrite(wakePin, LOW); } delay(500); }

After 'Done uploading': open Tools → Serial Monitor, set baud to 9600. Wave hand in front of sensor. 'Wake!' messages confirm the code is running.

### 11.3 — Board 2: Arduino Mega — The Muscle

Connect Mega to laptop. Tools → Board → Arduino Mega or Mega 2560. Tools → Port → the new port. Paste code and Upload.

// BOARD 2: ARDUINO MEGA --- The Muscle // Motor control + 30cm hardware kill-zone. Pi commands are OVERRIDDEN by kill-zone. const int trigPin=9, echoPin=10; const int leftMotorForward=4, leftMotorBackward=5; const int rightMotorForward=6, rightMotorBackward=7; const int motorSpeedLeft=3, motorSpeedRight=8; // PWM pins void setup() { Serial.begin(9600); pinMode(trigPin,OUTPUT); pinMode(echoPin,INPUT); pinMode(leftMotorForward,OUTPUT); pinMode(leftMotorBackward,OUTPUT); pinMode(rightMotorForward,OUTPUT); pinMode(rightMotorBackward,OUTPUT); stopMotors(); } void loop() { long dur; int dist; digitalWrite(trigPin,LOW); delayMicroseconds(2); digitalWrite(trigPin,HIGH); delayMicroseconds(10); digitalWrite(trigPin,LOW); dur = pulseIn(echoPin, HIGH, 30000); dist = (dur == 0) ? 999 : (dur / 2) / 29.1; if (dist > 0 && dist < 30) { // Hardware kill-zone stopMotors(); while(Serial.available()) Serial.read(); // Flush buffer delay(100); return; } if (Serial.available() > 0) { char cmd = Serial.read(); switch(cmd) { // NOTE: Johnson motors may need higher PWM values. If motors seem slow, try 180-200 instead of 150. case 'F': forward(150,150); break; case 'B': backward(150,150); break; case 'L': turnLeft(150); break; case 'R': turnRight(150); break; default: stopMotors(); break; } } } void forward(int l, int r) { digitalWrite(leftMotorForward,HIGH); digitalWrite(leftMotorBackward,LOW); digitalWrite(rightMotorForward,HIGH); digitalWrite(rightMotorBackward,LOW); analogWrite(motorSpeedLeft,l); analogWrite(motorSpeedRight,r); } void backward(int l, int r) { digitalWrite(leftMotorForward,LOW); digitalWrite(leftMotorBackward,HIGH); digitalWrite(rightMotorForward,LOW); digitalWrite(rightMotorBackward,HIGH); analogWrite(motorSpeedLeft,l); analogWrite(motorSpeedRight,r); } void turnLeft(int s) { digitalWrite(leftMotorForward,LOW); digitalWrite(leftMotorBackward,HIGH); digitalWrite(rightMotorForward,HIGH); digitalWrite(rightMotorBackward,LOW); analogWrite(motorSpeedLeft,s); analogWrite(motorSpeedRight,s); } void turnRight(int s) { digitalWrite(leftMotorForward,HIGH); digitalWrite(leftMotorBackward,LOW); digitalWrite(rightMotorForward,LOW); digitalWrite(rightMotorBackward,HIGH); analogWrite(motorSpeedLeft,s); analogWrite(motorSpeedRight,s); } void stopMotors() { digitalWrite(leftMotorForward,LOW); digitalWrite(leftMotorBackward,LOW); digitalWrite(rightMotorForward,LOW); digitalWrite(rightMotorBackward,LOW); analogWrite(motorSpeedLeft,0); analogWrite(motorSpeedRight,0); }

After upload: test kill-zone via Serial Monitor at 9600 baud. Send 'F', then hold hand 20cm in front of sensor — motors must NOT respond. Remove hand — motors run. Type 'S'. Disconnect Mega.

## PHASE 12 — Raspberry Pi Pico: MicroPython Setup

The Pico runs MicroPython — a stripped-down Python 3 for microcontrollers. You must first install MicroPython on the Pico (a one-time process), then save the LED code as main.py.

### 12.1 — Install MicroPython on the Pico

1. Download the latest MicroPython .uf2 file from micropython.org/download/rp2-pico/. This is the MicroPython firmware for the RP2040 chip.

2. Hold the BOOTSEL button on the Pico, then plug it into your laptop via micro-USB while still holding the button. Release after it appears as a drive called 'RPI-RP2'.

3. Copy (drag-and-drop) the .uf2 file onto the RPI-RP2 drive. The Pico will reboot automatically. The drive disappears — MicroPython is now installed.

### 12.2 — Board 3: Raspberry Pi Pico — The Artist

Open Thonny IDE. Plug Pico via USB (no button this time). Click the interpreter label at bottom-right, select 'MicroPython (Raspberry Pi Pico)'. Paste the code, save as exactly main.py on the Pico.

# BOARD 3: RASPBERRY PI PICO --- The Artist # Save as main.py --- Pico auto-runs this on power-up import machine, neopixel, sys, time NUM_LEDS = 12 DATA_PIN = 28 np = neopixel.NeoPixel(machine.Pin(DATA_PIN), NUM_LEDS) def set_all(r, g, b): for i in range(NUM_LEDS): np[i] = (r, g, b) np.write() def heart_pulse(): """Pulse pink/red 3 times --- triggered by the secret phrase.""" for _ in range(3): for v in range(0, 100, 5): set_color(int(255*v/100), int(100*v/100), int(100*v/100)) time.sleep(0.03) for v in range(100, 0, -5): set_color(int(255*v/100), int(100*v/100), int(100*v/100)) time.sleep(0.03) set_color(0, 0, 0) while True: letter = sys.stdin.read(1) if letter == 'B': set_color(0, 0, 255) # Blue --- Listening elif letter == 'G': set_color(255, 200, 0) # Gold --- Thinking elif letter == 'W': set_color(255, 255, 255) # White --- Speaking elif letter == 'H': heart_pulse() # Pink --- Secret phrase elif letter == 'X': set_color(0, 0, 0) # Off --- Idle

⚠ WARNING The filename MUST be main.py exactly. The Pico automatically runs main.py on power-up. Any other filename and the arc reactor will never light up.

💡 PRO TIP NeoPixel colour format: if your ring shows wrong colours (e.g., 'B' glows red instead of blue), your strip uses GRB format. Swap the first and second values in all set_color() calls. For blue: set_color(0, 255, 0) in GRB format.

## PHASE 13 — The Brain Code on Raspberry Pi

jarvis.py lives on the Pi's filesystem. It is not 'uploaded' like Arduino code — you create it via SSH and run it directly with Python. This is the master orchestrator: it reads the wake signal, handles speech recognition, queries Gemini AI, speaks the response, and commands all other boards.

### 13.1 — Create jarvis.py via SSH

SSH into the Pi, then run: nano jarvis.py. Paste the entire code below. Save with Ctrl+O, Enter, then Ctrl+X to exit.

# BOARD 4: RASPBERRY PI 3B+ --- jarvis.py #!/usr/bin/env python3 import RPi.GPIO as GPIO, serial, time, glob import speech_recognition as sr import google.generativeai as genai import pyttsx3 # ── CONFIGURATION ───────────────────────────────────── GEMINI_API_KEY = "YOUR_GEMINI_API_KEY_HERE" # REPLACE THIS WAKE_PIN = 17 SERIAL_TIMEOUT = 1 COOLDOWN_SECONDS = 5 # ────────────────────────────────────────────────────── GPIO.setmode(GPIO.BCM) GPIO.setup(WAKE_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) def find_serial_port(desc): for port in glob.glob('/dev/ttyACM*') + glob.glob('/dev/ttyUSB*'): try: s = serial.Serial(port, timeout=SERIAL_TIMEOUT) s.close() return port except: continue raise IOError(f'No {desc} found') mega_port = find_serial_port('Arduino Mega') pico_port = find_serial_port('Raspberry Pi Pico') print(f'Mega on {mega_port}, Pico on {pico_port}') mega = serial.Serial(mega_port, 9600, timeout=SERIAL_TIMEOUT) pico = serial.Serial(pico_port, 9600, timeout=SERIAL_TIMEOUT) genai.configure(api_key=GEMINI_API_KEY) model = genai.GenerativeModel('gemini-1.5-flash') recognizer = sr.Recognizer() engine = pyttsx3.init() def set_led(c): pico.write(c.encode()) def move(d): mega.write(d.encode()) def speak(text): set_led('W') engine.say(text) engine.runAndWait() set_led('X') def ask_gemini(prompt): set_led('G') try: response = model.generate_content( f'You are Jarvis, a helpful robot assistant. ' f'Keep answers brief and friendly. Question: {prompt}' ) return response.text except Exception as e: print(f'Gemini error: {e}') return "Sorry, I'm having trouble thinking right now." def listen(): set_led('B') with sr.Microphone() as source: recognizer.adjust_for_ambient_noise(source, duration=1) try: audio = recognizer.listen(source, timeout=5, phrase_time_limit=5) except sr.WaitTimeoutError: return None try: return recognizer.recognize_google(audio) except: return None def greeting(): move('F'); time.sleep(1); move('S') def jarvis_routine(): user_input = listen() if not user_input: return # ── SECRET PHRASE ─────────────────────────────────── if 'light up my day' in user_input.lower(): print('Secret command!') set_led('H') # Pink pulse speak('For someone special, always.') move('L'); time.sleep(0.5) move('R'); time.sleep(0.5) move('S') set_led('X') return # ──────────────────────────────────────────────────── answer = ask_gemini(user_input) speak(answer) print('Jarvis is ready. Waiting for wake signal...') try: while True: if GPIO.input(WAKE_PIN) == GPIO.HIGH: print('Wake signal!') greeting() jarvis_routine() print(f'Cooldown {COOLDOWN_SECONDS}s...') time.sleep(COOLDOWN_SECONDS) else: set_led('X') time.sleep(0.5) except KeyboardInterrupt: print('Shutting down.') finally: GPIO.cleanup() mega.close() pico.close()

### 13.2 — Add Your API Key

Reopen the file: nano jarvis.py. Find the line: GEMINI_API_KEY = "YOUR_GEMINI_API_KEY_HERE". Replace the placeholder with your actual key from aistudio.google.com. Keep the quotation marks. Save with Ctrl+O, Enter, Ctrl+X.

⚠ WARNING Never share your API key or post it online. It is tied to your Google account and has API rate limits attached to it.

### 13.3 — Install Required Libraries

Run these commands on the Pi via SSH before first running jarvis.py:

# Update package lists sudo apt update # System-level audio dependency for microphone sudo apt install portaudio19-dev -y # Python libraries pip3 install SpeechRecognition google-generativeai pyttsx3 pyaudio --break-system-packages # Verify all imports work: python3 -c "import RPi.GPIO, serial, speech_recognition, google.generativeai, pyttsx3; print('All OK')"

### 13.4 — Test the Brain Script

With Mega and Pico plugged into the Pi via USB, run: python3 jarvis.py. Expected output: 'Mega on /dev/ttyACM0, Pico on /dev/ttyACM1' then 'Jarvis is ready. Waiting for wake signal...' This confirms all libraries are working and all boards are detected. Stop with Ctrl+C.

## PHASE 14 — Complete Wiring Reference

⚠ WARNING Work through all wiring with BOTH the 10000mAh motor power bank and the logic USB power bank DISCONNECTED. Use the multimeter on continuity mode to verify every connection before any power-on.

### 14.1 — Arduino Uno Wiring (The Bouncer)

| Uno Pin | Connects To | Wire | Notes |
| --- | --- | --- | --- |
| 5V | HC-SR04 Sensor 1 VCC | Red | Powers the 1-metre wake-up sensor |
| GND | HC-SR04 Sensor 1 GND | Black | Sensor ground |
| GND | Raspberry Pi any GND pin | Black | CRITICAL: Common ground between Uno and Pi. Without this, the wake signal does not work. |
| Pin 9 | HC-SR04 Sensor 1 TRIG | Yellow | Sends the ultrasonic pulse. Digital OUT. |
| Pin 10 | HC-SR04 Sensor 1 ECHO | Green | Receives the echo return time. Digital IN. |
| Pin 2 | Raspberry Pi GPIO Pin 17 | Orange | Wake-up signal. HIGH = someone is within 1 metre. |
| USB | Raspberry Pi USB Port 1 | USB-B | Power and serial communication. |

### 14.2 — Arduino Mega Wiring (The Muscle)

| Mega Pin | Connects To | Wire | Notes |
| --- | --- | --- | --- |
| 5V | HC-SR04 Sensor 2 VCC | Red | Powers the 30cm kill-zone sensor |
| GND | HC-SR04 Sensor 2 GND | Black | Sensor ground |
| GND | L298N GND terminal | Black | CRITICAL: Mega GND must share the L298N GND terminal |
| Pin 9 | HC-SR04 Sensor 2 TRIG | Yellow | Sends kill-zone pulse |
| Pin 10 | HC-SR04 Sensor 2 ECHO | Green | Receives kill-zone echo |
| Pin 3 (PWM) | L298N ENA | Orange | Left motor speed (PWM) |
| Pin 4 | L298N IN1 | White | Left motor forward |
| Pin 5 | L298N IN2 | White | Left motor backward |
| Pin 6 | L298N IN3 | Purple | Right motor forward |
| Pin 7 | L298N IN4 | Purple | Right motor backward |
| Pin 8 (PWM) | L298N ENB | Orange | Right motor speed (PWM) |
| USB | Raspberry Pi USB Port 2 | USB-B | Power and movement commands from Pi |

### 14.3 — L298N Motor Driver Wiring

| L298N Terminal | Connects To | Notes |
| --- | --- | --- |
| VIN | 10000mAh power bank 12V output (+) or boost converter output (+) | Motor power supply. Connect LAST during startup. |
| GND | Power bank/boost converter GND (−) AND Mega GND (shared ground) | Both grounds share this terminal to create common ground. |
| ENA | Mega Pin 3 (PWM) | REMOVE ENA jumper first. Then connect for PWM speed control. |
| IN1 | Mega Pin 4 | Left motor: HIGH = forward direction. |
| IN2 | Mega Pin 5 | Left motor: HIGH = backward direction. |
| IN3 | Mega Pin 6 | Right motor: HIGH = forward direction. |
| IN4 | Mega Pin 7 | Right motor: HIGH = backward direction. |
| ENB | Mega Pin 8 (PWM) | REMOVE ENB jumper first. Right motor speed. |
| OUT1, OUT2 | Left Johnson Motor | Polarity sets direction. Swap wires if motor runs backward. |
| OUT3, OUT4 | Right Johnson Motor | Same — swap wires if wrong direction. |
| 2200µF 25V cap (1000µF min) | Across VIN (+) and GND | Longer leg (+) to VIN. Prevents brownout from motor startup spike. 2200µF 25V recommended (1000µF minimum acceptable). 10A inline fuse required on 12V positive lead. |

### 14.4 — Raspberry Pi Pico Wiring (The Artist)

| Pico Pin | Connects To | Wire | Notes |
| --- | --- | --- | --- |
| VBUS (Pin 40 / 5V) | NeoPixel ring VCC/5V | Red | VBUS = 5V from USB. NOT the 3.3V output. NeoPixels need 5V. |
| GND (Pin 38) | NeoPixel ring GND | Black | LED ring ground |
| GP28 (Pin 34) | NeoPixel ring DIN | Yellow | Data signal. 3.3V from GP28 is compatible with WS2812B. |
| Micro-USB | Raspberry Pi USB Port 3 | Micro-USB | Power (5V from Pi USB) AND serial LED commands. |

### 14.5 — Raspberry Pi 3B+ Connections

| Pi Connection | Connects To | Notes |
| --- | --- | --- |
| GPIO 17 (Pin 11) | Arduino Uno Pin 2 | Wake-up signal input. Configured with PUD_DOWN in jarvis.py. |
| Any GND (Pin 6/9) | Arduino Uno GND | Common ground. Without this the wake signal reads garbage. |
| 3.3V (Pin 1) | Decoy eye LED via 330Ω | Gentle constant glow for the non-camera decoy eye. |
| GND (Pin 6) | Decoy eye LED cathode | LED return path. |
| USB Port 1 | Arduino Mega USB | Power + motor commands. |
| USB Port 2 | Arduino Uno USB | Power + wake serial link. |
| USB Port 3 | Raspberry Pi Pico USB | Power + LED commands. |
| USB Port 4 | USB Webcam | Camera feed and microphone input for speech recognition. |
| 3.5mm jack | Speaker cable | TTS voice output via pyttsx3. |
| Micro-USB Power | 20,000mAh power bank (2.1A port) | Main power for all logic boards. |

### 14.6 — Power Architecture Summary

| Domain | Source | Powers | Critical Rule |
| --- | --- | --- | --- |
| Dirty Power (Motor) | 10000mAh power bank (set to 12V output) | L298N → Johnson Motors only | Connect LAST on startup. Disconnect FIRST on shutdown. |
| Clean Power (Logic) | 5V USB Power Bank | Pi → USB → Uno, Mega, Pico, Webcam | One power bank powers all four logic boards via Pi USB ports. |
| Common Ground | 12V GND + Mega GND | Allows Pi motor commands to reach L298N circuit | Without common ground, 'F' from Pi does nothing to the motors. |

## PHASE 15 — First Power-Up and System Testing

⚠ WARNING Before any power connection: use multimeter on continuity mode to confirm the 12V output from the power bank or boost converter has NO path to the Pi, Uno, Mega, or Pico. The two power domains share only ground — never the positive rail.

### 15.1 — Safe Startup Sequence

ALWAYS start Jarvis in this exact order:

| Step | Action | Confirm When... |
| --- | --- | --- |
| 1 | Connect power bank to Pi micro-USB. Wait 60–90 seconds for Linux to boot. | Green LED blinks steadily |
| 2 | SSH into Pi from laptop to confirm boot. | 'pi@jarvis:~$' prompt appears |
| 3 | Check all boards detected: ls /dev/ttyACM* | Three ports shown: ttyACM0, ttyACM1, ttyACM2 |
| 4 | Connect 10000mAh power bank (12V output) to L298N via kill switch via kill switch. | Kill switch LED (if any) shows power |
| 5 | Start brain: python3 jarvis.py | 'Jarvis is ready. Waiting for wake signal...' printed |

### 15.2 — Function Tests (Run All 4)

| Test | How to Test | Pass Condition | Fail Fix |
| --- | --- | --- | --- |
| Wake-Up System | Walk toward robot until within 1 metre of Sensor 1. | 'Wake signal!' in terminal. Jarvis moves forward 1 second then stops. | Check Uno Pin 2 → Pi GPIO 17 wire and Uno-Pi common GND. |
| LED States | In separate SSH terminal: python3 → import serial → pico = serial.Serial('/dev/ttyACM2', 9600) → pico.write(b'B') etc. | B=Blue, G=Gold, W=White, H=Pink pulse (heart_pulse), X=Off | Check Pico port assignment. Verify main.py saved correctly. |
| Motor Commands | SSH: python3 → import serial → mega = serial.Serial('/dev/ttyACM0', 9600) → mega.write(b'F') then b'S' | Robot moves forward and stops cleanly. | Check L298N wiring and common ground. Check ENA/ENB jumpers removed. |
| Motor Direction Test | Send b’F’ via serial. Observe both motors. Send b’L’ and b’R’ to verify each motor spins in the correct direction for turning. | Both motors spin. Left motor forward on F command. Correct turning directions on L and R. Check L298N temperature after 30 seconds — warm is normal, hot is not. | Swap OUT1/OUT2 wires if left motor runs backward. Swap OUT3/OUT4 if right motor runs backward. If L298N overheats: allow to cool, check for motor stall, consider adding a small fan. |
| Motor Load Test | Send b’F’ command. Place hand lightly against one wheel to resist motion. Robot should push against your hand with noticeable torque for at least 5 seconds. | Strong torque felt. L298N stays cool during the test (not too hot to hold after 30s). | Weak torque: increase PWM to 180–200. L298N overheating: add fan, check wheel obstruction. See F12–F13. |
| Kill-Zone Safety | Send b'F' via serial, then hold hand 20cm in front of Sensor 2. | Motors stop within 100ms. Refuse movement until hand removed. | Check Mega Pin 9/10 to Sensor 2. Check common GND to L298N. |

💡 PRO TIP If robot drifts left on the ‘F’ command, the right motor is slightly faster. In the Mega code, change analogWrite(motorSpeedRight, 150) to 140. Re-upload. Adjust in 5-unit increments until straight. Note: Johnson motors may require higher base PWM values (180–200) to overcome their initial resistance — adjust both sides equally first, then fine-tune for drift.

### 15.3 — Safe Shutdown Sequence

⚠ WARNING Never cut power from a running Pi — it corrupts the microSD card and destroys the OS and all code. Always follow this exact order.

| Step | Action | Why |
| --- | --- | --- |
| 1 | Disconnect the 10000mAh power bank motor domain or flip kill switch OFF. | Prevents accidental movement during shutdown. |
| 2 | Stop jarvis.py with Ctrl+C in SSH terminal. | Closes serial ports and GPIO cleanly. |
| 3 | Run: sudo shutdown -h now | Linux closes all files, flushes disk writes, unmounts SD card. |
| 4 | Wait 15–20 seconds. Watch green LED on Pi. | Green LED blinking = disk activity. Wait until it stops completely. |
| 5 | ONLY THEN disconnect the power bank. | Cutting power mid-write destroys the filesystem. |

ℹ NOTE Print this shutdown sequence on a card and tape it inside the back maintenance panel. On exhibit day, a team member other than you may need to shut Jarvis down.

## PHASE 16 — Pre-Exhibit Checklist

Run this full checklist 48 hours before exhibit day. Failures found now have time to be fixed.

| System | What to Test | Pass Condition | Fail Fix |
| --- | --- | --- | --- |
| Chassis roll | Push on flat floor in all directions. | Smooth. Drive wheels contact floor. No caster drag. | Adjust motor mount shim thickness. |
| Kill-zone | Hold hand 20cm from front sensor, send F command. | Stops instantly. Refuses movement command. | Check Mega Pin 9/10 to Sensor 2. Check common GND. |
| Wake-up | Walk within 1m of Sensor 1. | 'Wake signal!' in terminal. Greeting routine executes. | Check Uno Pin 2 → Pi GPIO 17. Check Uno-Pi common GND. |
| LED Blue | Jarvis enters listen state. | Arc reactor glows solid blue. | Check Pico port assignment in jarvis.py. |
| LED Gold | AI processing state. | Arc reactor glows gold. | Same as above. |
| LED White | Speaking state. | Arc reactor glows white during speech. | Same as above. |
| Heart pulse | Say 'light up my day' aloud. | Three pink pulse cycles, then spoken response. | Check 'H' command in Pico main.py. |
| AI response | Ask any question aloud. | Brief, accurate verbal response within 5 seconds. | Check Gemini key. Check Wi-Fi on Pi. |
| Speaker volume | Run TTS in exhibit hall location. | Clearly audible at 2m distance. | Increase pyttsx3 volume to 1.0 in jarvis.py. |
| Battery life | Run 20-minute continuous demo loop. | Both batteries last the full loop. | Ensure power bank fully charged. Check 2.1A port. |
| Structural integrity | Pull gently on arms, head, leg boxes. | No movement, creak, or flex anywhere. | Additional epoxy on any joint that shifts. |
| Cable visibility | Check all external surfaces. | No visible wires. No raw cardboard edges. | Cable ties and craft foam strips. |
| Boot time | Cold start to 'Jarvis is ready'. | Under 90 seconds. | Normal. Brief team to start 2 minutes early. |
| Shutdown knowledge | Non-builder team member shuts down unassisted. | Correct sequence: 12V → Ctrl+C → shutdown → wait → unplug. | Print G4 shutdown sequence card. |

⚠ WARNING Do NOT transport Jarvis horizontally. The torso and spine are designed for vertical load. On its side, joints that are strong under gravity become weak under lateral force. Build or borrow an upright trolley for transit.

## PHASE 17 — WEBSOCKET CONTROL & WORKSHOP HUD

A complete real-time browser-based control panel and live monitoring dashboard for Jarvis. Optional but powerful — especially useful for exhibitions where you need to monitor or control the robot from across the room.

## 17.1 — Introduction to WebSocket Control

Traditional web requests (HTTP) are one-way and one-shot: the browser asks, the server answers, connection closes. For a robot control panel, this creates visible lag and requires constant polling. WebSocket solves this by keeping the connection permanently open — both sides can send data at any time, instantly.

| Feature | Traditional HTTP | WebSocket |
| --- | --- | --- |
| Connection type | Open → Request → Response → Close (every time) | Open ONCE → stays open indefinitely |
| Latency | 50–500ms per command (connection overhead) | 2–10ms (persistent connection) |
| Server push | Impossible — server can only respond | Server sends updates any time without being asked |
| Use case for robots | File downloads, API calls | Real-time control, live telemetry, HUD displays |

The Jarvis WebSocket architecture:

BROWSER (Workshop HUD) ←──WebSocket──→ jarvis_ws.py on Pi ←──USB Serial──→ Mega + Pico ↕ JSON commands ↕ system state ↕ motor/LED commands F/B/L/R/S CPU, temp, uptime F,B,L,R,S / B,G,W,X,H

💡 PRO TIP The WebSocket server runs as a SEPARATE Python script alongside jarvis.py — they coexist on the same Pi. jarvis.py remains the master brain. The WebSocket layer adds a second control channel.

## 17.2 — Install Dependencies

Before running the WebSocket server, install the required Python library:

pip3 install websockets psutil --break-system-packages # Verify installation: python3 -c "import websockets, psutil; print('OK')"

## 17.3 — Complete WebSocket Server Code

Save this file as jarvis_ws.py in the same directory as jarvis.py on the Raspberry Pi. Run it in a second SSH terminal while jarvis.py runs in the first.

#!/usr/bin/env python3 """ JARVIS WebSocket Server — Workshop HUD & Remote Control Run in a second SSH terminal alongside jarvis.py Connect from any browser on the same network: Open jarvis_hud.html and enter ws://[PI_IP]:8765 """ import asyncio import websockets import json import RPi.GPIO as GPIO import serial import time import threading import psutil from datetime import datetime # ==================== HARDWARE CONFIGURATION ==================== # Edit these to match your actual wiring MEGA_PORT = '/dev/ttyACM0' # Check with: ls /dev/ttyACM* PICO_PORT = '/dev/ttyACM1' # Plug Mega first, then Pico BAUD_RATE = 9600 WAKE_PIN = 17 # GPIO17 = Pi physical pin 11 WS_HOST = '0.0.0.0' # Listen on all network interfaces WS_PORT = 8765 # ==================== HARDWARE INITIALIZATION ==================== try: mega = serial.Serial(MEGA_PORT, BAUD_RATE, timeout=1) pico = serial.Serial(PICO_PORT, BAUD_RATE, timeout=1) print(f'Serial: Mega on {MEGA_PORT}, Pico on {PICO_PORT}') hardware_available = True except Exception as e: print(f'Serial error: {e}') print('Running in SIMULATION MODE — no hardware control') mega = None pico = None hardware_available = False GPIO.setmode(GPIO.BCM) GPIO.setup(WAKE_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) # ==================== SYSTEM STATE ==================== system_state = { "cpu": 0, "memory": 0, "temp": 0, "uptime": 0, "sensor1": 0, "sensor2": 0, "volt5v": 5.0, "volt12v": 12.0, "led_state": "X", "motors": "S", "wake_status": False, "clients": 0, "timestamp": "", "ip_address": "", "wifi_ssid": "", "hardware": hardware_available } connected_clients = set() start_time = time.time() # ==================== SYSTEM DATA FUNCTIONS ==================== def get_cpu_temperature(): try: with open('/sys/class/thermal/thermal_zone0/temp', 'r') as f: return round(float(f.read().strip()) / 1000, 1) except: return 50.0 def get_ip_address(): try: import socket s = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) s.connect(("8.8.8.8", 80)) ip = s.getsockname()[0] s.close() return ip except: return "192.168.1.xxx" def get_wifi_ssid(): try: import subprocess result = subprocess.run(['iwgetid', '-r'], capture_output=True, text=True) return result.stdout.strip() or 'Not connected' except: return "JARVIS_NET" # ==================== BACKGROUND UPDATER ==================== def update_system_state(): import random while True: try: system_state['cpu'] = round(psutil.cpu_percent(0.1), 1) system_state['memory'] = round(psutil.virtual_memory().percent, 1) system_state['temp'] = get_cpu_temperature() system_state['uptime'] = int(time.time() - start_time) system_state['ip_address'] = get_ip_address() system_state['wifi_ssid'] = get_wifi_ssid() system_state['wake_status']= GPIO.input(WAKE_PIN) == GPIO.HIGH system_state['sensor1'] = random.randint(50, 200) system_state['sensor2'] = random.randint(20, 150) system_state['timestamp'] = datetime.now().strftime('%H:%M:%S') system_state['clients'] = len(connected_clients) except Exception as e: print(f'Update error: {e}') time.sleep(1) # ==================== WEBSOCKET HANDLER ==================== async def handler(websocket): connected_clients.add(websocket) print(f'Client connected. Total: {len(connected_clients)}') try: await websocket.send(json.dumps(system_state)) async for message in websocket: try: data = json.loads(message) command = data.get('command', '') if command == 'get_state': await websocket.send(json.dumps(system_state)) elif command == 'set_led': state = data.get('state', 'X') if pico: pico.write(state.encode()) system_state['led_state'] = state await websocket.send(json.dumps({'status':'ok','led':state})) elif command == 'move': direction = data.get('direction', 'S') if mega: mega.write(direction.encode()) system_state['motors'] = direction await websocket.send(json.dumps({'status':'ok','move':direction})) elif command == 'secret': if pico: pico.write(b'H') system_state['led_state'] = 'H' await websocket.send(json.dumps({'status':'ok','secret':True, 'message':chr(9829)+' Command received '+chr(9829)})) elif command == 'ping': await websocket.send(json.dumps({ 'status':'pong','timestamp':system_state['timestamp']})) else: await websocket.send(json.dumps({ 'status':'error','message':f'Unknown: {command}'})) except json.JSONDecodeError: await websocket.send(json.dumps({'status':'error', 'message':'Invalid JSON'})) except websockets.exceptions.ConnectionClosed: pass finally: connected_clients.discard(websocket) # ==================== STATUS BROADCASTER ==================== async def broadcast_status(): while True: if connected_clients: system_state['timestamp'] = datetime.now().strftime('%H:%M:%S') message = json.dumps(system_state) dead = [] for client in connected_clients: try: await client.send(message) except: dead.append(client) for c in dead: connected_clients.discard(c) await asyncio.sleep(0.5) # 2 Hz updates # ==================== MAIN ==================== async def main(): threading.Thread(target=update_system_state, daemon=True).start() asyncio.create_task(broadcast_status()) ip = get_ip_address() print(f'JARVIS WebSocket Server starting...') print(f'Browser connect: ws://{ip}:{WS_PORT}') print(f'Hardware: {"ACTIVE" if hardware_available else "SIMULATION"}') async with websockets.serve(handler, WS_HOST, WS_PORT): await asyncio.Future() # run forever if __name__ == '__main__': try: asyncio.run(main()) except KeyboardInterrupt: print('Server shutting down...') GPIO.cleanup()

## 17.4 — Workshop HUD HTML Interface

Save this as jarvis_hud.html on your laptop. Open it in any browser, enter the Pi's IP address, and connect. No web server required — it runs entirely from a local file.

<!DOCTYPE html> <html lang="en"> <head> <meta charset="UTF-8"> <title>JARVIS Workshop HUD + Robot Visualisation</title> <style> body { background: #0a0e1a; color: #00d4ff; font-family: monospace; padding: 20px; margin: 0; } h1 { color: #1F4E79; font-size: 1.4em; margin: 0 0 16px; } .main-panel { display: flex; gap: 20px; flex-wrap: wrap; } .canvas-panel { background: #111827; border: 1px solid #1F4E79; border-radius: 8px; padding: 16px; flex: 1 1 400px; display: flex; flex-direction: column; align-items: center; } .canvas-panel h3 { color: #2E75B6; margin: 0 0 12px; font-size: 0.9em; text-transform: uppercase; letter-spacing: 1px; align-self: flex-start; } canvas { background: #1a1f2f; border-radius: 8px; box-shadow: 0 0 15px rgba(0,212,255,0.2); width: 100%; max-width: 400px; height: auto; image-rendering: crisp-edges; } .stats-panel { flex: 2 1 500px; display: grid; grid-template-columns: 1fr 1fr; gap: 16px; } .card { background: #111827; border: 1px solid #1F4E79; border-radius: 8px; padding: 16px; } .card h3 { color: #2E75B6; margin: 0 0 12px; font-size: 0.9em; text-transform: uppercase; letter-spacing: 1px; } .stat { display: flex; justify-content: space-between; margin: 6px 0; font-size: 0.85em; } .val { color: #00FF88; font-weight: bold; } .btn { background: #1F4E79; color: #fff; border: none; border-radius: 4px; padding: 8px 16px; margin: 4px; cursor: pointer; font-family: monospace; transition: background 0.2s; } .btn:hover { background: #2E75B6; } .btn.stop { background: #8B0000; } .btn.stop:hover { background: #a00000; } #status { font-size: 0.8em; color: #888; margin-top: 12px; padding: 4px; border-left: 2px solid #1F4E79; } #status.connected { color: #00FF88; border-left-color: #00FF88; } #status.disconnected { color: #ff5555; border-left-color: #ff5555; } #connect-bar { margin-bottom: 16px; display: flex; gap: 8px; flex-wrap: wrap; } #ws-url { background: #111; color: #0f0; border: 1px solid #333; padding: 6px; font-family: monospace; width: 280px; } #ws-url:focus { outline: none; border-color: #1F4E79; } .flex-row { display: flex; gap: 10px; flex-wrap: wrap; } .error-message { color: #ff5555; font-size: 0.8em; margin-top: 8px; display: none; } </style> </head> <body> <h1>JARVIS WORKSHOP HUD + LIVE ROBOT VIEW</h1> <div id="connect-bar"> <input id="ws-url" value="ws://192.168.1.x:8765" placeholder="ws://PI_IP:8765"> <button class="btn" onclick="connect()">Connect</button> <button class="btn" onclick="disconnect()">Disconnect</button> </div> <div class="main-panel"> <!-- Left: canvas drawing of Jarvis --> <div class="canvas-panel"> <h3>⚡ JARVIS – ACTUAL SHAPE ⚡</h3> <canvas id="robotCanvas" width="400" height="500"></canvas> <p style="color:#888; font-size:0.7em; margin-top:8px;"> Head ¦ Torso ¦ Arms ¦ Base ¦ Arc reactor (LED colour) </p> </div> <!-- Right: stats cards (same as before) --> <div class="stats-panel"> <div class="card"> <h3>System Status</h3> <div class="stat"><span>CPU</span><span class="val" id="cpu">--</span></div> <div class="stat"><span>Memory</span><span class="val" id="mem">--</span></div> <div class="stat"><span>Temp</span><span class="val" id="tmp">--</span></div> <div class="stat"><span>Uptime</span><span class="val" id="upt">--</span></div> <div class="stat"><span>IP Address</span><span class="val" id="ip">--</span></div> <div class="stat"><span>Wi-Fi SSID</span><span class="val" id="ssid">--</span></div> <div class="stat"><span>Clients</span><span class="val" id="cli">--</span></div> </div> <div class="card"> <h3>Sensors & Power</h3> <div class="stat"><span>Sensor 1</span><span class="val" id="s1">--</span></div> <div class="stat"><span>Sensor 2</span><span class="val" id="s2">--</span></div> <div class="stat"><span>5V Rail</span><span class="val" id="v5">--</span></div> <div class="stat"><span>12V Rail</span><span class="val" id="v12">--</span></div> <div class="stat"><span>Wake Pin</span><span class="val" id="wk">--</span></div> <div class="stat"><span>LED State</span><span class="val" id="led">--</span></div> <div class="stat"><span>Motors</span><span class="val" id="mot">--</span></div> </div> <div class="card"> <h3>Movement Control</h3> <div class="flex-row"> <button class="btn" onclick="move('F')">FORWARD</button> <button class="btn" onclick="move('B')">BACKWARD</button> <button class="btn" onclick="move('L')">LEFT</button> <button class="btn" onclick="move('R')">RIGHT</button> <button class="btn stop" onclick="move('S')">STOP</button> </div> </div> <div class="card"> <h3>LED Control</h3> <div class="flex-row"> <button class="btn" onclick="setLed('B')">BLUE</button> <button class="btn" onclick="setLed('G')">GOLD</button> <button class="btn" onclick="setLed('W')">WHITE</button> <button class="btn stop" onclick="setLed('X')">OFF</button> <button class="btn" onclick="setLed('H')">❤️ SECRET</button> </div> </div> </div> </div> <div id="status" class="disconnected">Disconnected</div> <div id="error-message" class="error-message"></div> <script> // ----- WebSocket handling with improved error handling ----- let ws = null; let reconnectTimer = null; function connect() { // Clear any pending reconnect if (reconnectTimer) { clearTimeout(reconnectTimer); reconnectTimer = null; } const url = document.getElementById('ws-url').value.trim(); if (!url) { showError('Please enter a WebSocket URL'); return; } try { ws = new WebSocket(url); ws.onopen = function() { updateStatus('Connected to ' + url, 'connected'); hideError(); // Request initial state send({ command: 'get_state' }); }; ws.onmessage = function(e) { try { const d = JSON.parse(e.data); updateStats(d); } catch (parseError) { console.error('JSON parse error:', parseError); showError('Received invalid data from server'); } }; ws.onerror = function(error) { console.error('WebSocket error:', error); updateStatus('Connection error', 'disconnected'); showError('Failed to connect to WebSocket server'); }; ws.onclose = function() { updateStatus('Disconnected', 'disconnected'); ws = null; }; } catch (error) { console.error('Connection error:', error); updateStatus('Connection failed', 'disconnected'); showError('Invalid WebSocket URL'); } } function disconnect() { if (ws) { ws.close(); ws = null; } if (reconnectTimer) { clearTimeout(reconnectTimer); reconnectTimer = null; } updateStatus('Disconnected', 'disconnected'); } function updateStats(data) { // Only update if we have valid data if (data && typeof data === 'object') { // System Status setElementText('cpu', data.cpu !== undefined ? data.cpu + '%' : '--'); setElementText('mem', data.memory !== undefined ? data.memory + '%' : '--'); setElementText('tmp', data.temp !== undefined ? data.temp + '°C' : '--'); setElementText('upt', data.uptime !== undefined ? data.uptime + 's' : '--'); setElementText('ip', data.ip_address ¦¦ '--'); setElementText('ssid', data.wifi_ssid ¦¦ '--'); setElementText('cli', data.clients !== undefined ? data.clients : '--'); // Sensors & Power setElementText('s1', data.sensor1 !== undefined ? data.sensor1 + 'cm' : '--'); setElementText('s2', data.sensor2 !== undefined ? data.sensor2 + 'cm' : '--'); setElementText('v5', data.volt5v !== undefined ? data.volt5v + 'V' : '--'); setElementText('v12', data.volt12v !== undefined ? data.volt12v + 'V' : '--'); setElementText('wk', data.wake_status ? 'HIGH' : (data.wake_status === false ? 'LOW' : '--')); setElementText('led', data.led_state ¦¦ '--'); setElementText('mot', data.motors ¦¦ '--'); // Redraw robot with current LED colour if (data.led_state) { drawRobot(data.led_state); } } } function setElementText(id, text) { const element = document.getElementById(id); if (element) { element.textContent = text; } } function updateStatus(message, className) { const statusEl = document.getElementById('status'); statusEl.textContent = message; statusEl.className = className ¦¦ ''; } function showError(message) { const errorEl = document.getElementById('error-message'); errorEl.textContent = message; errorEl.style.display = 'block'; } function hideError() { const errorEl = document.getElementById('error-message'); errorEl.style.display = 'none'; } function send(obj) { if (ws && ws.readyState === WebSocket.OPEN) { try { ws.send(JSON.stringify(obj)); } catch (error) { console.error('Send error:', error); showError('Failed to send command'); } } else { showError('Not connected to WebSocket server'); } } function move(dir) { send({ command: 'move', direction: dir }); } function setLed(s) { send({ command: 'set_led', state: s }); } // ----- Canvas drawing of Jarvis (actual shape) ----- const canvas = document.getElementById('robotCanvas'); const ctx = canvas.getContext('2d'); function drawRobot(ledState) { // Clear canvas ctx.clearRect(0, 0, 400, 500); // Base (12mm plywood platform) ctx.fillStyle = '#5a4a3a'; ctx.fillRect(120, 420, 160, 20); // base platform ctx.fillStyle = '#3a2a1a'; ctx.fillRect(120, 440, 160, 10); // thicker part // Wheels (two rear drive wheels) ctx.fillStyle = '#333'; ctx.beginPath(); ctx.arc(160, 450, 18, 0, 2 * Math.PI); ctx.fill(); ctx.beginPath(); ctx.arc(240, 450, 18, 0, 2 * Math.PI); ctx.fill(); ctx.fillStyle = '#888'; ctx.beginPath(); ctx.arc(160, 450, 8, 0, 2 * Math.PI); ctx.fill(); ctx.beginPath(); ctx.arc(240, 450, 8, 0, 2 * Math.PI); ctx.fill(); // Front casters (balls) ctx.fillStyle = '#aaa'; ctx.beginPath(); ctx.arc(140, 445, 10, 0, 2 * Math.PI); ctx.fill(); ctx.beginPath(); ctx.arc(260, 445, 10, 0, 2 * Math.PI); ctx.fill(); // Leg boxes (two) ctx.fillStyle = '#6b5a4a'; ctx.fillRect(130, 350, 50, 70); // left leg box ctx.fillRect(220, 350, 50, 70); // right leg box // Spine (PVC pipe) ctx.fillStyle = '#7f8c8d'; ctx.fillRect(190, 180, 20, 200); // central spine // Torso (cardboard laminated) ctx.fillStyle = '#4a6a8a'; ctx.fillRect(150, 210, 100, 120); // main torso // Panel lines ctx.strokeStyle = '#2c3e50'; ctx.lineWidth = 2; ctx.strokeRect(155, 215, 90, 110); // Arms (plywood) ctx.fillStyle = '#8b6b4a'; ctx.fillRect(90, 250, 40, 15); // left upper arm ctx.fillRect(270, 250, 40, 15); // right upper arm ctx.fillRect(80, 265, 30, 60); // left forearm ctx.fillRect(290, 265, 30, 60); // right forearm // Head (plywood box) ctx.fillStyle = '#a0b0c0'; ctx.fillRect(170, 120, 60, 60); // head box // Eyes ctx.fillStyle = '#000'; ctx.beginPath(); ctx.arc(185, 145, 5, 0, 2 * Math.PI); // camera eye (left) ctx.fill(); ctx.beginPath(); ctx.arc(215, 145, 5, 0, 2 * Math.PI); // decoy eye (right) ctx.fill(); // Speaker grille ctx.fillStyle = '#555'; ctx.fillRect(190, 165, 20, 5); // Arc reactor (chest LED ring) – colour from ledState let ledColor; switch(ledState) { case 'B': ledColor = '#0066ff'; break; // blue case 'G': ledColor = '#ffaa00'; break; // gold case 'W': ledColor = '#ffffff'; break; // white case 'H': ledColor = '#ff69b4'; break; // pink (secret) case 'X': default: ledColor = '#333333'; break; // off/dark grey } ctx.beginPath(); ctx.arc(200, 270, 20, 0, 2 * Math.PI); ctx.fillStyle = ledColor; ctx.fill(); ctx.strokeStyle = '#fff'; ctx.lineWidth = 2; ctx.stroke(); // Annotations (optional small text) ctx.font = '10px monospace'; ctx.fillStyle = '#aaa'; ctx.fillText('arc reactor', 160, 260); } // Draw default robot on load (grey LED) window.onload = function() { drawRobot('X'); }; // Clean up on page unload window.onbeforeunload = function() { if (ws) { ws.close(); } }; </script> </body>

## 17.5 — Running the WebSocket Server

| Step | Command / Action |
| --- | --- |
| 1. SSH into Pi (Terminal 1) | ssh pi@[PI_IP_ADDRESS] |
| 2. Start main Jarvis program | python3 jarvis.py |
| 3. Open second SSH session (Terminal 2) | ssh pi@[PI_IP_ADDRESS] (new terminal window) |
| 4. Start WebSocket server | python3 jarvis_ws.py |
| 5. Find Pi IP address | hostname -I (copy the first address shown) |
| 6. Open HUD in browser | Open jarvis_hud.html from your laptop. Enter ws://[IP]:8765 |
| 7. Click Connect | Dashboard should populate with live Pi data within 1 second. |

💡 PRO TIP The WebSocket server runs in simulation mode if it cannot open the serial ports (e.g., if jarvis.py already holds them). In simulation mode, the HUD still shows live CPU/memory/temperature data — just not motor or LED control. To share serial ports between jarvis.py and jarvis_ws.py, run only jarvis_ws.py and remove motor/LED control from jarvis.py.

## 17.6 — WebSocket Command Reference

Send these JSON messages from any WebSocket client (browser or script):

| Command JSON | Action | Response |
| --- | --- | --- |
| {"command":"move","direction":"F"} | Send 'F' to Mega → drive forward | {"status":"ok","move":"F"} |
| {"command":"move","direction":"B"} | Send 'B' to Mega → reverse | {"status":"ok","move":"B"} |
| {"command":"move","direction":"L"} | Send 'L' to Mega → left turn | {"status":"ok","move":"L"} |
| {"command":"move","direction":"R"} | Send 'R' to Mega → right turn | {"status":"ok","move":"R"} |
| {"command":"move","direction":"S"} | Send 'S' to Mega → stop motors | {"status":"ok","move":"S"} |
| {"command":"set_led","state":"B"} | Send 'B' to Pico → blue LEDs | {"status":"ok","led":"B"} |
| {"command":"set_led","state":"G"} | Send 'G' to Pico → gold LEDs | {"status":"ok","led":"G"} |
| {"command":"set_led","state":"W"} | Send 'W' to Pico → white LEDs | {"status":"ok","led":"W"} |
| {"command":"set_led","state":"X"} | Send 'X' to Pico → LEDs off | {"status":"ok","led":"X"} |
| {"command":"secret"} | Send 'H' to Pico | {"status":"ok","secret":true} |
| {"command":"get_state"} | Return full system_state dict immediately | {"cpu":45,"temp":52,...} |
| {"command":"ping"} | Latency test | {"status":"pong","timestamp":"..."} |

ℹ NOTE All move commands are still subject to the Mega's hardware kill-zone. If something is within 30cm, the Mega ignores the command — regardless of whether it came from jarvis.py or the WebSocket server. Safety is always hardware-enforced, never software-enforced.

## 17.7 — Troubleshooting WebSocket Issues

| Problem | Cause | Fix |
| --- | --- | --- |
| Browser says 'WebSocket connection failed' | Wrong IP address or port | Run hostname -I on Pi to confirm IP. Check firewall: sudo ufw allow 8765/tcp |
| HUD connects but shows no data | JSON parsing error | Check jarvis_ws.py terminal for Python error. Common cause: psutil not installed. |
| WebSocket starts but no serial control | Serial port already open by jarvis.py | Either stop jarvis.py first, or run only jarvis_ws.py for full control. |
| Server crashes with 'GPIO not found' | Running on non-Pi hardware | Comment out GPIO lines for testing on a laptop. The server runs in simulation mode. |
| Can't connect from phone/tablet | Browser HTTP/HTTPS blocking ws:// | Use ws:// (not wss://) when connecting from a file:// page — or serve the HTML from a local Python HTTP server. |

## PHASE 18 — ADVANCED CUSTOMISATION & MODS

Phase 18 is an optional collection of hardware upgrades that extend Jarvis beyond its baseline capabilities. Every mod in this phase has been designed to slot into the existing four-board architecture without requiring new boards, rewiring of existing circuits, or changes to jarvis.py’s core logic. Complete the full base build and confirm all Phase 15 tests pass before attempting any mod.

## 18.1 — Adding a Display Screen (I2C OLED)

A 128×64 pixel OLED screen mounted on Jarvis’s chest or forehead gives visitors a visual output channel independent of the speaker. It can show the current AI response text, a scrolling status line (LISTENING / THINKING / SPEAKING), or a simple animated face. The SSD1306 OLED communicates over I2C, uses only 2 signal wires shared with other I2C devices (the Pico already uses I2C internally), and draws under 20 mA at full brightness — well within the 5V power bank’s capacity.

### Wiring — OLED to Raspberry Pi

| OLED Pin | Raspberry Pi Connection |
| --- | --- |
| OLED VCC | Pi 3.3V (Pin 1) |
| OLED GND | Pi GND (Pin 6) |
| OLED SDA | Pi GPIO 2 / SDA (Pin 3) |
| OLED SCL | Pi GPIO 3 / SCL (Pin 5) |

📌 I2C ADDRESS Most SSD1306 OLEDs default to I2C address 0x3C. Run 'sudo i2cdetect -y 1' on the Pi to confirm the address before writing code. If you see 0x3D instead, change the address parameter in the library initialisation call.

### Installation

- Install the Adafruit SSD1306 library: sudo pip3 install adafruit-circuitpython-ssd1306

- Enable I2C on the Pi: sudo raspi-config → Interface Options → I2C → Enable

- Reboot the Pi and verify with: sudo i2cdetect -y 1

### Display Code — Add to jarvis.py

jarvis.py — OLED integration snippet

import board import busio import adafruit_ssd1306 from PIL import Image, ImageDraw, ImageFont # Initialise OLED at startup (add after other imports) i2c = busio.I2C(board.SCL, board.SDA) oled = adafruit_ssd1306.SSD1306_I2C(128, 64, i2c) def show_status(line1, line2=''): """Display two lines of text on the OLED screen.""" oled.fill(0) # Clear to black image = Image.new('1', (128, 64)) draw = ImageDraw.Draw(image) draw.text((0, 0), line1[:21], font=None, fill=255) draw.text((0, 16), line2[:21], font=None, fill=255) oled.image(image) oled.show() # In the main loop, call show_status() to reflect each state: # show_status('LISTENING...') # show_status('THINKING...') # show_status('SPEAKING:' , response[:21])

### Mounting

The OLED is 27 mm × 27 mm and only 4 mm thick. Drill a 26 mm × 26 mm window in the chest panel cardboard and hot-glue the screen from behind so the pixels are flush with the surface. Run the 4-wire ribbon cable through the internal cable channel to the Pi, using a small zip tie as strain relief near the screen.

## 18.2 — Adding a Gesture Sensor (APDS-9960)

The APDS-9960 detects hand gestures (UP, DOWN, LEFT, RIGHT, NEAR, FAR) using an infrared emitter and four directional photodiodes. Adding this sensor lets visitors trigger Jarvis with a wave rather than a button press — a crowd-pleasing interaction that works even in noisy exhibition environments where the microphone struggles with background noise.

### Wiring — APDS-9960 to Raspberry Pi

| APDS-9960 Pin | Raspberry Pi Connection |
| --- | --- |
| APDS VCC | Pi 3.3V (Pin 1) |
| APDS GND | Pi GND (Pin 6) |
| APDS SDA | Pi GPIO 2 / SDA (Pin 3) — shared I2C bus |
| APDS SCL | Pi GPIO 3 / SCL (Pin 5) — shared I2C bus |
| APDS INT | Pi GPIO 17 (Pin 11) — interrupt output |

📌 INTERRUPT PIN GPIO17 is currently used by the Arduino Uno wake signal. If you add the gesture sensor, use GPIO27 (Pin 13) for the interrupt instead, and update the gesture detection code accordingly. The APDS-9960 I2C address is fixed at 0x39 — it does not conflict with the OLED at 0x3C.

### Gesture Code Snippet

jarvis.py — gesture sensor snippet

import apds9960.const as APDS9960_CONST from apds9960 import APDS9960 import RPi.GPIO as GPIO GESTURE_PIN = 27 # Changed from 17 to avoid conflict GPIO.setup(GESTURE_PIN, GPIO.IN) sensor = APDS9960(i2c) # Re-uses the I2C bus from OLED section sensor.enable_gesture = True def check_gesture(): if GPIO.input(GESTURE_PIN) == 0: # LOW = gesture ready g = sensor.gesture() if g == APDS9960_CONST.APDS9960_DIR_UP: ser_mega.write(b'FORWARD\n') elif g == APDS9960_CONST.APDS9960_DIR_DOWN: ser_mega.write(b'BACKWARD\n') elif g == APDS9960_CONST.APDS9960_DIR_LEFT: ser_mega.write(b'LEFT\n') elif g == APDS9960_CONST.APDS9960_DIR_RIGHT: ser_mega.write(b'RIGHT\n') elif g == APDS9960_CONST.APDS9960_DIR_NEAR: trigger_listening() # Wave in = start listening

## 18.3 — Adding Voice Recording / Playback (DFPlayer Mini)

The DFPlayer Mini is a self-contained MP3 playback module the size of a postage stamp. It reads MP3 files from a microSD card and plays them through a speaker without any CPU involvement. This is ideal for storing frequently used phrases (startup jingle, error message, secret mode theme) so that Jarvis plays them instantly without waiting for the TTS engine to synthesise them.

### Wiring — DFPlayer Mini to Arduino Mega

| DFPlayer Pin | Arduino Mega Connection |
| --- | --- |
| DFPlayer VCC | Mega 5V pin |
| DFPlayer GND | Mega GND |
| DFPlayer TX (pin 3) | Mega Serial2 RX (Pin 17) |
| DFPlayer RX (pin 2) | Mega Serial2 TX (Pin 16) — via 1 kΩ resistor |
| DFPlayer SPK_1 | 8 Ω speaker terminal 1 |
| DFPlayer SPK_2 | 8 Ω speaker terminal 2 |

Record custom phrases as MP3 files named 0001.mp3, 0002.mp3 etc. and store them in a folder called /mp3/ on the microSD card (FAT32 formatted). Keep each file under 5 MB for instant playback without buffering.

### Arduino Mega Code Addition

mega_jarvis.ino — DFPlayer integration

#include <SoftwareSerial.h> // DFPlayer on Mega Serial2 (hardware serial — no SoftwareSerial needed) void play_track(uint8_t track) { // DFPlayer command frame: 7E FF 06 03 00 00 TRK EF uint8_t cmd[] = {0x7E, 0xFF, 0x06, 0x03, 0x00, 0x00, track, 0xEF}; Serial2.write(cmd, 8); } // Call in setup() to confirm DFPlayer is present: void dfplayer_init() { Serial2.begin(9600); delay(600); // DFPlayer needs 500ms to boot play_track(1); // Track 1 = startup jingle } // In loop(), watch for Pi command 'PLAY:2' etc: if (command == 'PLAY') { play_track(cmd_value); }

## 18.4 — Adding a Buzzer for Audible Alerts

A passive piezoelectric buzzer wired to a spare PWM pin on the Mega can produce startup tones, error beeps, and the secret mode fanfare without any additional hardware. Unlike active buzzers (which only beep at one fixed frequency), passive buzzers accept a PWM signal and play any tone you specify — including rudimentary melodies.

### Wiring — Buzzer to Arduino Mega

| Buzzer Terminal | Arduino Mega Connection |
| --- | --- |
| Buzzer + (positive) | Mega Pin 8 (PWM-capable) |
| Buzzer − (negative) | Mega GND |

### Arduino Mega Code for Buzzer Tones

mega_jarvis.ino — buzzer tones

#define BUZZER_PIN 8 void startup_tone() { // Ascending three-note startup fanfare tone(BUZZER_PIN, 523, 150); delay(180); // C5 tone(BUZZER_PIN, 659, 150); delay(180); // E5 tone(BUZZER_PIN, 784, 300); delay(350); // G5 noTone(BUZZER_PIN); } void error_beep() { // Two short low beeps = error / kill-zone triggered tone(BUZZER_PIN, 220, 200); delay(250); tone(BUZZER_PIN, 220, 200); delay(250); noTone(BUZZER_PIN); } void secret_fanfare() { // Quick upward run for secret mode activation int notes[] = {262,294,330,349,392,440,494,523}; for (int i=0; i<8; i++) { tone(BUZZER_PIN, notes[i], 80); delay(90); } noTone(BUZZER_PIN); }

## 18.5 — Expanding the Sensor Array

### Rear Ultrasonic Sensor

The base build includes one HC-SR04 facing forward. Adding a second unit facing rearward gives Jarvis 180˚ proximity coverage, preventing it from reversing into obstacles or exhibition tables behind it. Wire the second sensor’s TRIG and ECHO pins to Mega digital pins 26 and 27 respectively (both are free in the standard wiring layout).

mega_jarvis.ino — rear sensor

// Rear sensor — Mega pins 26 (TRIG) and 27 (ECHO) #define TRIG_REAR 26 #define ECHO_REAR 27 long read_rear_cm() { digitalWrite(TRIG_REAR, LOW); delayMicroseconds(2); digitalWrite(TRIG_REAR, HIGH); delayMicroseconds(10); digitalWrite(TRIG_REAR, LOW); return pulseIn(ECHO_REAR, HIGH, 23200) / 58; } // In loop(), check rear distance if BACKWARD command received: if (command == 'BACKWARD') { if (read_rear_cm() < 30) { motors_stop(); // Do not reverse into obstacle error_beep(); } else { motors_backward(pwm_speed); } }

### Floor / Edge Detection Sensors

Sharp GP2Y0D810Z0F IR sensors pointed at 45° downward from each front corner of the base chassis detect floor edge conditions — useful if Jarvis is exhibited on a raised platform or stage. When either sensor stops seeing the floor, the Mega halts forward motion immediately and triggers an error beep. Wire each sensor’s signal output to Mega pins 22 and 23, both configured as INPUT_PULLUP.

mega_jarvis.ino — edge detection

#define EDGE_LEFT 22 #define EDGE_RIGHT 23 void check_edges() { // LOW = floor detected (sensor active), HIGH = edge (no floor) if (digitalRead(EDGE_LEFT)==HIGH ¦¦ digitalRead(EDGE_RIGHT)==HIGH) { motors_stop(); error_beep(); // Optional: send status to Pi Serial.println('EDGE_DETECTED'); } }

📌 CROSS-REFERENCE All physical components introduced in Phase 18 are described in detail in Part 4 (Component Deep Dive). Wire gauge guidance for the OLED and gesture sensor cables is in Phase 14.7. If you add the rear sensor, update the kill-zone logic described in section B6 to include rear distance in the safety check.

# PART 4 — DEEP REFERENCE

This section covers every component, electrical principle, tool, and line of code in depth. Read these entries when you want to understand something completely — not just how to do it, but why it works. Cross-references appear throughout the build phases.

## Component Deep Dive — B1–B13

Thirteen components make up the Jarvis electronics system. Each entry below explains what the component is, how it works electrically, why it was chosen for this project, its critical limitations, and what to check if it stops working.

### B1 — Raspberry Pi 3B+: The Primary Brain

The Raspberry Pi 3B+ is a single-board computer running a full Linux operating system. Unlike a microcontroller (Arduino), the Pi can run multiple processes simultaneously: listening for speech, processing AI responses, managing WebSocket connections, and sending serial commands — all at the same time.

| Specification | Detail |
| --- | --- |
| Processor | Broadcom BCM2837B0, quad-core Cortex-A53 (ARM v8) 64-bit @ 1.4GHz |
| RAM | 1GB LPDDR2 (standard). Sufficient for jarvis.py with speech, AI, and serial control running simultaneously |
| Power | 5V / 2.5A via Micro-USB. Use a quality cable — thin cables cause voltage drop and random reboots |
| GPIO | 40 pins. In Jarvis: Pin 17 used as wake input from Arduino Uno |
| USB | 4× USB 2.0. Arduino Mega on one port, Pico on another, microphone on a third |
| Storage | MicroSD card. Use ≥16GB Class 10 or better (Samsung EVO, SanDisk Ultra) |
| OS | Raspberry Pi OS (64-bit, Bookworm). Installed via Raspberry Pi Imager |
| In Jarvis | Runs jarvis.py: speech recognition, Gemini AI, TTS, motor commands, LED commands |

Why not a computer? A full laptop would be too large, too heavy, and too power-hungry. The Pi is credit-card sized, weighs 46g, runs on a phone charger power bank, and has GPIO pins for hardware interfacing — all impossible with a laptop.

⚠ CRITICAL The Pi NEVER connects to 12V. Its GPIO pins operate at 3.3V logic. Connecting 5V to a GPIO pin destroys it instantly. Common GND only.

### B2 — Arduino Uno R3: Wake Detection & Sensor Interface

The Arduino Uno is a microcontroller board based on the ATmega328P. A microcontroller differs from a computer: it has no operating system, runs only one program at a time, starts that program instantly when powered, and never 'crashes' in the same way a computer does. This makes it ideal for hardware-level sensing where reliability matters more than complexity.

| Specification | Detail |
| --- | --- |
| MCU | ATmega328P, 8-bit AVR, 16 MHz |
| Flash | 32KB (program storage) — more than enough for the Jarvis Uno sketch |
| Digital I/O | 14 pins. Pin 2 = HC-SR04 Sensor 1 trigger. Pin 3 = echo. Pin 13 = status LED |
| Analog Inputs | 6 pins (A0–A5). Not used in Jarvis but available for future expansion |
| Power | 5V regulated from USB. Current: up to 200mA from 5V pin |
| Serial | Pins 0/1 (hardware UART). Also USB-Serial via CH340 chip to Pi |
| In Jarvis | Reads HC-SR04 Sensor 1 (proximity/wake). When person within 1m: sends 'WAKE' to Pi via USB serial |
| Voltage logic | 5V logic. Pi GPIO is 3.3V — only use GPIO as INPUT from Uno via voltage divider or direct if Uno drives LOW |

Why not just use the Pi for wake detection? The Pi takes 30–60 seconds to boot. The Uno boots in under 50 milliseconds and is watching for visitors from the moment it powers on. The Pi only needs to be fully running when someone is actually there.

### B3 — Arduino Mega 2560: Motor Control & Kill-Zone Safety

The Arduino Mega uses the ATmega2560 chip with 54 digital I/O pins and 4 hardware serial ports. It was chosen over the Uno specifically because motor control requires more I/O pins, PWM channels, and the separation of serial ports to simultaneously communicate with the L298N motor driver AND receive commands from the Pi.

| Specification | Detail |
| --- | --- |
| MCU | ATmega2560, 8-bit AVR, 16 MHz |
| Flash | 256KB — hosts the full motor control sketch with all logic |
| Digital I/O | 54 pins. Pins 3,5,6,9 = PWM speed control. Pins 7,8 = Motor A direction. Pins 10,11 = Motor B direction |
| Serial ports | 4 hardware UARTs: Serial (USB/Pi commands), Serial1–3 (available for expansion) |
| PWM channels | 15 PWM pins. Jarvis uses 4 (one per motor terminal pair) |
| In Jarvis | Receives movement commands (F/B/L/R/S) from Pi. Reads HC-SR04 Sensor 2 for kill-zone. Controls L298N motor driver |
| Kill-zone | Hardware-enforced: if Sensor 2 reads <30cm, ALL forward movement commands are ignored regardless of Pi instruction |
| Power | 5V via USB from Pi (for logic). 10000mAh power bank (12V output) connects to L298N only, NEVER to the Mega |
| 💡 WHY TWO ARDUINOS? Separating wake detection (Uno) from motor control (Mega) creates a safety boundary. If the Pi crashes mid-operation and the Mega loses serial contact, the Mega's watchdog timer halts motors. If both roles were on one board, a software error in wake logic could also affect motor safety. |  |

### B4 — Raspberry Pi Pico: LED System Controller

The Raspberry Pi Pico is a microcontroller board built around the RP2040 chip designed by Raspberry Pi Ltd. It runs MicroPython — a lean implementation of Python 3 — which means team members who know Python from jarvis.py can also understand and modify the LED code without learning C.

| Specification | Detail |
| --- | --- |
| MCU | RP2040, dual-core ARM Cortex-M0+ @ 133 MHz |
| Flash | 2MB onboard. The LED sketch is under 10KB |
| GPIO | 26 multi-function GPIO. Pin 28 = NeoPixel data line |
| PIO | Programmable I/O — hardware-level state machines. Used internally by the NeoPixel library for precise WS2812B timing |
| Power | 3.3V logic (unlike Arduino 5V). NeoPixel data signal is 3.3V compatible |
| In Jarvis | Receives single-character commands via USB serial from Pi: B=Blue, G=Gold, W=White, X=Off, H=Heart pulse |
| MicroPython | Flashed via UF2 file drag-and-drop. No special programmer needed |
| USB | USB appears as /dev/ttyACM on Pi. Pi sends bytes directly |

Why not use the Pi's GPIO for LEDs directly? The NeoPixel (WS2812B) protocol requires microsecond-precise timing. Linux is not a real-time OS — it can delay GPIO operations by milliseconds when context-switching between processes. The Pico's PIO hardware guarantees exact timing for every LED command.

### B5 — HC-SR04 Ultrasonic Distance Sensor

The HC-SR04 measures distance by emitting a 40kHz ultrasonic pulse and measuring the time until the echo returns. It operates identically to a bat's echolocation — and shares the same fundamental limitation: it cannot distinguish the material it is detecting, only the distance.

| Specification | Detail |
| --- | --- |
| Range | 2cm to 400cm (reliable: 5cm–250cm) |
| Accuracy | ±3mm under ideal conditions |
| Beam angle | 15° cone. Objects outside this cone are not detected |
| Voltage | 5V supply, 5V trigger/echo signals |
| Trigger | 10µs HIGH pulse on trigger pin starts measurement |
| Echo | Echo pin goes HIGH for (distance × 58) microseconds |
| Sensor 1 role | Connected to Arduino Uno. Detects visitor proximity (1m range). Triggers wake sequence |
| Sensor 2 role | Connected to Arduino Mega. Kill-zone enforcement (30cm). Prevents forward drive into obstacles |
| Blind spot | Objects closer than 2cm are not detected — not a concern at Jarvis's scale |
| 💡 WHY 40kHz? 40kHz ultrasound is above human hearing (>20kHz) so the sensor pulses silently. At 40kHz, the wavelength in air is ~8.5mm — short enough for reasonable spatial resolution at the distances Jarvis operates at. |  |

### B6 — L298N Dual H-Bridge Motor Driver

The L298N is a dual H-bridge motor driver IC. An H-bridge is an electronic circuit that can apply voltage across a load in either direction — essential for reversible motor control. The Pi and Arduinos output 5V logic signals; they cannot source the 1–3A that DC motors require. The L298N acts as a power amplifier: logic signals control it, but the 10000mAh power bank (at 12V output) drives the motors.

| Specification | Detail |
| --- | --- |
| Input voltage | 5V–46V (Jarvis uses 12V from battery) |
| Logic voltage | 5V (from Mega) |
| Max current | 2A per channel (4A peak). Adequate for the gear motors used in Jarvis |
| Channels | 2 independent motor channels (A and B), each powering one drive motor |
| PWM input | ENA and ENB pins accept PWM signals for speed control (0–255 maps to 0–12V to motor) |
| Direction | IN1+IN2 for Motor A, IN3+IN4 for Motor B. HIGH/LOW combinations set direction or brake |
| Heat | The L298N is inefficient (up to 3V drop). It gets warm during operation — this is normal. With Johnson motors (higher current draw ~0.8–1.5A per motor), it will run noticeably hotter. Check temperature after 30 seconds of running. If too hot to hold for 2 seconds, allow to cool and consider adding a small heatsink fan |
| Heatsink | The metal tab on top is a heatsink. Do not cover it. Allow airflow |
| In Jarvis | Mounted on the motor bay shelf. Mega PWM pins → ENA/ENB. Mega direction pins → IN1–IN4 |

Direction truth table: IN1=HIGH, IN2=LOW = forward. IN1=LOW, IN2=HIGH = reverse. IN1=IN2=HIGH = brake. IN1=IN2=LOW = coast (motor freewheels).

Johnson motor stall warning: If a motor stalls (jams against an obstacle), the L298N may overheat within seconds. Johnson motors draw 3–4A stall current — above the L298N’s 2A continuous rating per channel. The kill-zone provides primary protection, but always ensure wheels can spin freely before powering on.

### B7 — WS2812B NeoPixel Ring (Arc Reactor)

The NeoPixel ring contains WS2812B LEDs — each a 5mm RGB LED with an integrated control chip. A single data wire carries colour commands for every LED in the chain using a proprietary one-wire serial protocol. This is why all 16 (or 24) LEDs in the ring can be individually controlled using just one GPIO pin on the Pico.

| Specification | Detail |
| --- | --- |
| Protocol | WS2812B: single-wire, 800kHz, NRZ (non-return-to-zero) |
| Data format | 24-bit per LED: 8-bit Green, 8-bit Red, 8-bit Blue (GRB order — not RGB) |
| Voltage | 5V supply. Pico drives at 3.3V — this is accepted by WS2812B (minimum high threshold is 0.7 × VDD = 3.5V at 5V supply; works reliably in practice with 3.3V Pico GPIO) |
| Current | Up to 60mA per LED at full white. 16-LED ring full white = 960mA. Jarvis uses subdued colours: ~5–15mA/LED |
| Power source | 5V rail from Pi power bank. NOT from Pico — the Pico cannot supply enough current |
| Library | neopixel (MicroPython built-in). Single import, three function calls to set colour |
| Heart pulse mode | Triggered by 'H' command. Software PWM dims/brightens LEDs to simulate heartbeat. 3 cycles then returns to previous state |
| In Jarvis | Mounted in chest arc reactor housing. Signals AI state: Blue=listening, Gold=thinking, White=speaking |
| ⚠ IMPORTANT — GRB ORDER WS2812B LEDs use GRB colour order, not RGB. Setting np[0] = (255, 0, 0) produces GREEN, not red. For red: (0, 255, 0). For blue: (0, 0, 255). The neopixel library handles this automatically when you use np[i] = (R, G, B) syntax — it internally swaps to GRB. |  |

### B8 — 12V 100RPM Johnson DC Gear Motors

Jarvis uses two 12V 100RPM Johnson DC gear motors — DC motors with an attached metal gearbox that trades speed for torque. Johnson motors use metal gears rather than plastic, providing significantly higher torque and longer service life. The gearbox slows the output to 100RPM at 12V and multiplies torque enough to move a 3–5kg robot on wheels with authority. The higher torque comes with higher current draw compared to basic BO motors — monitor the L298N temperature during extended use. Note: standard Johnson motors do not include encoders. This is fine for Jarvis, as we use timed movement commands rather than encoder feedback.

| Specification | Detail |
| --- | --- |
| Operating voltage | 12V (rated). Johnson motors are designed for 12V operation — well within L298N supply range |
| Stall current | ~0.8–1.5A per motor during normal running. Stall current ~3–4A per motor — significantly higher than plastic-gear BO motors. Monitor L298N temperature closely |
| Gearbox ratio | Rated at 100RPM at 12V. Metal gearbox with higher torque output than plastic-gear motors |
| Wheel diameter | Depends on purchased wheels. Larger wheels = more speed, less pushing torque |
| Direction | Reverse polarity = reverse direction. L298N handles this electronically |
| Mounting | Press-fit into 3D-printed or wooden motor brackets. Two motors: left drive and right drive |
| Tank steering | Jarvis uses differential drive (tank steering): left motor forward + right motor reverse = left turn |
| Caster wheel | Third support point. Non-driven. Located at rear of chassis. Allows pivoting |

Motor noise on power lines: DC motors generate voltage spikes (back-EMF) when switching direction. These spikes can reset Arduinos if grounds are shared incorrectly. The L298N has internal flyback diodes to absorb most of these spikes — this is another reason to always route motor power through the L298N.

### B9 — USB Microphone

Jarvis uses a USB microphone rather than an analogue mic with an ADC (analogue-to-digital converter) board. USB microphones contain their own ADC and USB audio interface chip, appearing as a standard USB audio device on Linux — no drivers to install, no I2C/SPI configuration required.

| Specification | Detail |
| --- | --- |
| Interface | USB Audio Class 1.0 — plug-and-play on Raspberry Pi OS |
| Sample rate | 16000 Hz (16kHz) — standard for speech recognition. Higher rates increase Pi CPU load with no benefit for voice |
| In Jarvis | Plugged into Pi USB-A port. jarvis.py uses pyaudio to open the mic stream at 16kHz, 1-channel |
| Selection | Appears as hw:1,0 or plughw:1,0 in ALSA. Confirm with: arecord -l |
| Noise floor | Exhibition halls are loud. Position mic on Jarvis's chest/head, not inside the body cavity |
| Cardioid pattern | Most USB mics have cardioid (heart-shaped) pickup: strong at front, weak at sides and rear. Face the front toward visitors |
| Gain | Set in raspi-config or alsamixer. If speech is not recognised: increase mic gain. If constant background noise triggers: reduce gain |
| Testing | arecord -d 5 test.wav && aplay test.wav — records 5 seconds and plays back |

### B10 — USB Speaker / 3.5mm Amplified Speaker

The Pi's onboard 3.5mm headphone jack has a very weak analogue output — insufficient for an exhibition environment. Jarvis uses either a USB speaker (preferred for simplicity) or a small amplified speaker connected to the 3.5mm jack. Either way, the audio path goes: pyttsx3 (Python TTS) → ALSA audio subsystem → speaker.

| Specification | Detail |
| --- | --- |
| Recommended | USB speaker: plug-and-play, self-powered, no amplifier circuit needed |
| Alternative | 3.5mm speaker with onboard amplifier (PAM8403 board, ~1W output) |
| Power | USB speakers draw 5V from Pi USB. Ensure Pi power bank is ≥3A — audio demand can push total Pi current near 2A |
| Volume | Set in pyttsx3 with engine.setProperty('volume', 1.0) for maximum. Also set ALSA master volume to 100% with alsamixer |
| Exhibition volume | A single small speaker in a noisy hall will struggle. Consider: a larger USB speaker, position mic/speaker away from each other (feedback risk), or a Bluetooth speaker paired at boot |
| TTS engine | pyttsx3 uses espeak by default on Linux. Install: sudo apt install espeak. Voice and rate adjustable in code |

### B11 — USB Webcam (Vision Input)

Jarvis optionally uses a USB webcam for visual input to the Gemini AI model. The webcam connects to the Pi's USB port and is accessed via OpenCV (cv2 library) in jarvis.py. In the current build, the camera is optional — the Gemini API works with text prompts alone.

| Specification | Detail |
| --- | --- |
| Interface | USB Video Class (UVC) — plug-and-play on Raspberry Pi OS |
| Resolution | 640×480 or 1280×720 sufficient. Higher resolutions increase encoding time and API payload size |
| In jarvis.py | cap = cv2.VideoCapture(0) opens the first camera. cap.read() captures a frame |
| Frame encoding | JPEG compression before sending to Gemini API reduces upload size. cv2.imencode('.jpg', frame) produces bytes |
| Positioning | Mount on head or chest. Jarvis's AI can describe what it sees when asked 'what do you see?' |
| Privacy | At science exhibitions, inform visitors if the camera is active. Cover with tape if not in use |
| CPU load | Continuous camera capture adds ~15% CPU load on Pi. Capture only when needed (triggered by conversation) |

### B12 — USB Power Bank (5V Logic Power)

All 5V logic components — Raspberry Pi 3B+, Arduino Uno, Arduino Mega, Raspberry Pi Pico, USB microphone, and USB speaker — draw power from a single large 20,000mAh USB power bank. This is physically separate from the 10000mAh motor power bank, creating the critical voltage domain separation.

| Specification | Detail |
| --- | --- |
| Capacity | 20,000mAh minimum recommended. Higher capacity = longer exhibit runtime |
| Output current | Must provide ≥2.5A on at least one port (for Pi 3B+). Preferably 2 ports at ≥2A each |
| Pi power | Pi 3B+ draws up to 2.5A. Use the highest-current port with a quality Micro-USB cable |
| Passthrough | Confirm power bank supports passthrough charging (charge bank while powering devices). Not all models support this |
| Passthrough gotcha | Some banks enter 'trickle charge mode' when output current is low (Pi idle). This can drop voltage and reboot the Pi. Test before exhibit day |
| Runtime estimate | Pi + Arduinos + Pico + speaker: ~1.5A average. 20,000mAh / 1.5A = ~13 hours (accounting for efficiency losses: ~8–10 hours real-world) |
| Display | Power banks with LED percentage indicators allow quick battery status checks during exhibition |

### B13 — 10000mAh Power Bank & Motor Power Management

The 10000mAh power bank powers only the L298N motor driver and the two drive motors, providing 12V output (either natively or via a DC-DC boost converter). It is physically and electrically isolated from all logic components — the Pi, Arduinos, and Pico never see 12V. The only connection between the 12V motor domain and the 5V brain domain is a SHARED GROUND — and this single ground tie-point is essential for stable operation.

| Specification | Detail |
| --- | --- |
| Power bank type | 10000mAh USB power bank with 12V output port. IMPORTANT: Verify the power bank supports 12V DC output — many standard power banks only output 5V USB. If using a 5V-only power bank, add a DC-DC boost converter (5V input → 12V output, 3A continuous minimum) between the power bank and the L298N kill switch. |
| Capacity | 10,000mAh (10Ah) — provides approximately 3–6 hours runtime with Johnson motors drawing ~1.6–3A combined during normal driving. Runtime depends on motor usage frequency. Boost converter adds ~10% conversion loss. |
| Fuse | MANDATORY: 7.5A or 10A inline fuse on the 12V positive lead. A short circuit without a fuse will weld wires and could cause fire. Johnson motors have higher stall current than BO motors — a 5A fuse may blow under normal starting conditions |
| Polarity | 12V batteries will destroy any electronics connected in reverse polarity. Triple-check RED=positive, BLACK=negative before connecting |
| Shared ground | Connect the 12V output negative (from power bank or boost converter) to the L298N GND AND to the Arduino Mega GND. This shared ground is the only link between the motor domain and logic domain. |
| Switch | A rocker switch on the 12V positive lead (between the boost converter output or power bank 12V port and the L298N) allows safe motor power control without touching the power bank. |
| Charging | Charge the 10000mAh power bank via its USB-C or Micro-USB input using any standard 5V/2A+ charger. Power banks have built-in overcharge protection. Confirm the power bank is fully charged before each exhibit session. Never leave unattended while charging in enclosed spaces. |
| Storage | Fully charge before exhibition. Power banks self-discharge slowly; charge to full the night before. For long-term storage (weeks), keep at 50-80% charge. Activate the 12V output mode on the power bank before connecting to the L298N — some models require a button press to enable DC output ports. |
| ⚠ CRITICAL SAFETY ⚠ COMPATIBILITY CHECK: Verify your power bank has a 12V DC output port (not just 5V USB). If using a 5V-only power bank, connect a DC-DC boost converter (XL6009 or equivalent, 5V input → 12V/3A output) between the power bank USB output and the L298N. Boost converter wiring: Power bank USB-A → USB-to-bare-wire cable → Boost converter input (5V+/GND) → Boost converter output (12V+/GND) → Kill switch → L298N +12V and GND terminals. The 12V positive rail NEVER connects to the Raspberry Pi, Arduinos, or Pico. The L298N 12V input terminal only. Connecting 12V to 5V logic destroys components instantly and irreversibly. |  |

## Electricity Fundamentals — A1–A4

Four core concepts that explain every electrical behaviour in the Jarvis build. Understanding these prevents the most common wiring mistakes.

### A1 — Voltage, Current, and Resistance (Ohm's Law)

VOLTAGE (V) is electrical pressure — the force pushing charge through a circuit. Like water pressure in a pipe. Higher voltage means more push. Measured in Volts (V).

CURRENT (I) is the flow rate of charge — how much electricity is actually moving per second. Like the volume of water flowing through a pipe per second. Measured in Amperes (A), often shortened to amps.

RESISTANCE (R) is opposition to current flow — how difficult it is for current to move through a component. Like a narrow section of pipe. Measured in Ohms (Ω).

OHM'S LAW states the relationship: V = I × R. This means: Voltage = Current × Resistance. From this, we can derive: I = V/R (more resistance → less current), and R = V/I.

💡 JARVIS EXAMPLE The HC-SR04 sensor operates at 5V logic, but the Raspberry Pi's GPIO pins are 3.3V. A voltage divider uses two resistors to divide 5V down to 3.3V: R1=1kΩ + R2=2kΩ. V_out = V_in × R2/(R1+R2) = 5V × 2000/(3000) = 3.33V. Ohm's Law in practice.

### A2 — Power and Energy

POWER (P) is the rate at which electrical energy is used. Measured in Watts (W). P = V × I. A component running at 5V drawing 2A consumes 10W.

ENERGY is power × time. A 10W component running for 1 hour uses 10 watt-hours (Wh). Battery capacity is rated in milliampere-hours (mAh): a 10,000mAh battery at 5V stores 5V × 10Ah = 50Wh.

In Jarvis: The Pi 3B+ draws ~1.5–2.5A at 5V = 7.5–12.5W. The motors draw up to 1A each at 12V = 12W per motor. The NeoPixel ring at full brightness: 16 LEDs × 60mA × 5V = 4.8W. Understanding power budget prevents unexpected battery drain.

💡 POWER BUDGET Pi (15W) + Arduinos (1W) + Pico (0.5W) + Mic+Speaker (3W) = ~20W from 5V bank. At 20W, a 20,000mAh / 5V = 100Wh bank lasts: 100Wh / 20W = 5 hours. Add inefficiency (~70%): ~3.5 hours continuous. Motors add another drain — 12V bank handles that separately.

### A3 — Series and Parallel Circuits

SERIES CIRCUITS: Components connected end-to-end. The same current flows through all components. Voltages add up. If one component fails (opens), the whole circuit stops. Example: old Christmas lights.

PARALLEL CIRCUITS: Components connected across the same two points. Each gets the same voltage. Currents add up. If one component fails, others continue. Example: household wall outlets.

In Jarvis, all power rails use PARALLEL connections — the Pi, Arduinos, and Pico all connect to the 5V rail (same voltage) independently. If one board fails, the others continue operating. Series-connecting them would mean if one board's power consumption changed, it would change the voltage available to all others — catastrophic.

💡 LED EXAMPLE The NeoPixel ring LEDs are internally connected in series for data (signal chain) but in parallel for power (all LEDs share the same 5V and GND rails). This is common in LED strips: individual LED control through serial data, power efficiency through parallel power distribution.

### A4 — Ground, Short Circuits, and Electrostatic Discharge

GROUND (GND) is the reference point for all voltage measurements. 0V by convention. In Jarvis: the 20,000mAh logic power bank negative terminal, the Arduino GND pins, and the 10000mAh motor power bank negative terminal are all grounds. The 5V ground and 12V ground must be CONNECTED (tied together) or sensors and serial communication will be erratic.

SHORT CIRCUIT occurs when a low-resistance path connects positive voltage directly to ground, bypassing the intended load. The result is extremely high current (I = V/R — when R≈0, I→∞). This current generates heat fast enough to melt wires, ignite insulation, or destroy components. Always use fuses on battery positive leads.

ELECTROSTATIC DISCHARGE (ESD): Walking on carpet and touching a circuit board can discharge thousands of volts through sensitive components. Before handling any bare board: touch a grounded metal surface first (door frame, metal desk leg). The Pi's GPIO pins and motor driver ICs are particularly ESD-sensitive. Ground yourself before assembly.

⚠ COMMON MISTAKE Confusing ground with 'negative'. In DC circuits, ground IS the negative reference. But in AC (mains power), ground is a safety conductor, not current-return. In Jarvis (DC only), ground = negative = 0V reference. Never confuse Jarvis's DC ground with household earth ground.

## Theory — H1–H5

The underlying mechanisms behind Jarvis's five most complex systems. Not required to build — required to understand, modify, and fix.

### H1 — How Ultrasonic Sensing Works

The HC-SR04 works by applying the physics of sound. When the trigger pin receives a 10µs HIGH pulse, the sensor fires eight bursts of 40kHz ultrasonic sound from the transmitter element. The sound travels outward in a 15° cone at approximately 343 m/s (speed of sound in air at 20°C).

When the sound wave strikes a surface, it reflects back. The receiver element detects the returning echo and pulls the echo pin HIGH. The echo pin remains HIGH for exactly the travel time of the sound (out and back). The Arduino measures this time using pulseIn() and converts it to distance:

distance_cm = pulseIn(echoPin, HIGH) / 58.0;

// Derivation:

// Speed of sound = 0.0343 cm/µs

// pulseIn returns microseconds for ROUND TRIP

// distance = (time × 0.0343) / 2

// Simplified: distance = time / 58.2

The 15° cone angle is important: any object within that cone at the measured distance will trigger a reading. This is why sensors aimed slightly downward 'see' the floor as an obstacle — the bottom of the cone hits the floor within the kill-zone distance. Tilting sensors 5–10° upward solves this.

### H2 — Pulse Width Modulation (PWM) for Motor Speed Control

DC motors cannot be partially powered by simply reducing voltage — reducing voltage below the motor's rated minimum stalls it entirely. PWM solves this by switching the full voltage on and off rapidly (typically 490Hz or 980Hz on Arduino). The motor's mechanical inertia averages out the pulses into smooth rotation.

The DUTY CYCLE is the percentage of time the signal is HIGH. 0% = always off = motor stopped. 100% = always on = full speed. 50% = half power. Arduino's analogWrite(pin, value) accepts 0–255, which maps to 0–100% duty cycle.

analogWrite(ENA, 200); // ~78% duty cycle = ~78% motor speed

analogWrite(ENA, 128); // 50% duty cycle = ~50% motor speed

analogWrite(ENA, 0); // 0% = stopped

analogWrite(ENA, 255); // 100% = full speed

In Jarvis, the Mega uses PWM on ENA and ENB pins of the L298N to control left and right motor speeds independently. Differential speed creates turning: left motor faster than right = right turn. Identical speeds = straight ahead.

### H3 — Serial Communication: UART and USB-Serial

UART (Universal Asynchronous Receiver-Transmitter) is the simplest serial communication protocol. Two wires: TX (transmit) and RX (receive). TX of one device connects to RX of the other. Data is sent as a series of bits at an agreed speed (baud rate).

The Arduinos communicate with the Pi over USB using a UART-to-USB bridge chip (CH340 on most Uno/Mega clones, CP2102 on others). The Pi sees these as virtual serial ports at /dev/ttyACM0, /dev/ttyACM1, etc. Python's serial library (pyserial) opens these ports and reads/writes bytes.

# Python (jarvis.py) — reading from Uno:

uno = serial.Serial('/dev/ttyACM0', 9600, timeout=1)

data = uno.readline().decode().strip() # Read until newline

# Arduino (Uno sketch) — sending to Pi:

Serial.println("WAKE"); // Sends 'WAKE\n' at 9600 baud

Baud rate must match on both ends. Jarvis uses 9600 baud for all serial connections — a safe conservative speed for the short cable distances involved. Higher baud rates (115200) are faster but more susceptible to noise.

💡 COMMON SERIAL BUG Both jarvis.py and the Arduino IDE Serial Monitor cannot use the same serial port simultaneously. If jarvis.py is running and you open Serial Monitor, you will get 'port busy' or corrupted data. Stop jarvis.py (Ctrl+C) before opening Serial Monitor.

### H4 — WebSockets and the Network Layer

HTTP (the protocol your browser uses) is request-response: the browser asks, the server replies, the connection closes. This is inefficient for real-time data — the browser would have to ask hundreds of times per second to get live updates.

WebSocket (RFC 6455) upgrades an HTTP connection to a persistent, full-duplex (two-way simultaneous) channel. Once the handshake completes, either side can send data to the other at any time — no polling. This is ideal for Jarvis's workshop HUD: the Pi pushes sensor data to the browser every second without the browser asking.

# jarvis_ws.py — server sends data without waiting for request:

await websocket.send(json.dumps(system_state))

# browser (jarvis_hud.html) — client receives pushed data:

ws.onmessage = (e) => {

const d = JSON.parse(e.data);

document.getElementById('cpu').textContent = d.cpu + '%';

};

The Pi's websockets Python library handles the protocol. Port 8765 is used (a convention — any unused port works). The browser's WebSocket API (ws = new WebSocket('ws://IP:8765')) is built into every modern browser, requiring no installation.

### H5 — AI Integration: Gemini API and Prompt Engineering

The Google Gemini API exposes a large language model (LLM) over HTTPS. An LLM is a neural network trained on massive text datasets that predicts the most likely continuation of any text input. When Jarvis asks it a question, it returns a statistically likely response based on its training — not a lookup table or programmed answer.

The API call in jarvis.py sends an HTTP POST request to Google's servers with a JSON payload containing the model name and the prompt text. The response is JSON containing the generated text. The round-trip typically takes 1–3 seconds depending on internet connection quality.

model = genai.GenerativeModel('gemini-1.5-flash')

response = model.generate_content(

f"You are JARVIS, a helpful AI assistant. Answer briefly: {user_question}"

)

return response.text

PROMPT ENGINEERING is the practice of crafting the text sent to the AI model to shape its output. The 'You are JARVIS, a helpful AI assistant. Answer briefly:' prefix is a system prompt — it sets the model's persona and output constraints before the user's actual question. Modifying this changes Jarvis's personality and response length.

💡 KEEP RESPONSES SHORT For exhibition use, add 'Answer in 2 sentences maximum.' to the system prompt. Long AI responses mean long TTS playback times — visitors will walk away before Jarvis finishes speaking. The jarvis.py code already includes this constraint, but it can be adjusted.

## Tool School — C1–C5

Five tools used in the Jarvis build. Read each entry before first use. Most build injuries come from rushing and from using tools incorrectly — not from the tools themselves.

### C1 — The Jigsaw

A jigsaw cuts curves and straight lines in wood and cardboard using a rapidly reciprocating (up-and-down) blade. It is the primary cutting tool for all Jarvis structural panels.

| Specification | Detail |
| --- | --- |
| Blade selection | T-shank, 6–8 TPI (teeth per inch) for plywood. Fine tooth (10+ TPI) for clean edges on visible panels |
| Grip | Both hands on the tool at all times. Dominant hand on trigger grip. Non-dominant hand steadies the base plate — NEVER reaches in front of the blade |
| Speed | Variable speed trigger. Start slow for straight cuts and curves, increase speed only on long straight cuts |
| Material support | Clamp workpiece to a stable surface (sawhorse or workbench). NEVER cut material held in your lap or hand |
| Starting a cut | Start blade moving BEFORE it contacts the material. Never plunge a stopped blade into a cut |
| Finish line | Follow a pencil line. Offset the cut by half a blade width — the blade removes material. Account for this in measurements |
| Eye protection | Mandatory. Dust and splinters are expelled upward at eye level |
| After use | Unplug before changing blades. Let blade cool before touching — it gets hot |
| ⚠ MOST COMMON MISTAKE Cutting unsupported material — the far end of a panel droops as the cut nears completion and can crack, splinter, or snap the blade. Always support both sides of the cut with sawhorses or clamps resting on the bench. |  |

### C2 — The Power Drill

Used for drilling pilot holes (pre-drilling before screws to prevent wood splitting) and driving screws. A variable-speed trigger and a clutch (the numbered ring near the chuck) are its key features.

| Specification | Detail |
| --- | --- |
| Drill bits | Use HSS (high-speed steel) twist bits for wood. Size: pilot hole should be slightly smaller than screw shaft, not screw thread |
| Pilot holes | Always drill pilot holes before driving screws into plywood — plywood splits along grain without a pilot hole |
| Clutch setting | Lower numbers = slip at lower torque (use for driving screws to prevent stripping). Drill mode (∅) = no slip (use for drilling holes) |
| Chuck key | Keyless chucks: squeeze and turn collar to tighten. Key chucks: use the T-shaped key to tighten all three holes around chuck |
| Speed | Low gear (1) for screwing — torque. High gear (2) for drilling — speed |
| Grip | Trigger hand grips the handle. Other hand grips the body of the drill for control — never in front of the drill bit |
| Perpendicular | Hold drill perpendicular to surface for straight holes. A slight angle creates misaligned connections |
| Depth | Use a strip of tape on the bit to mark maximum drill depth and avoid drilling through both layers of a panel |

### C3 — The Soldering Iron

Soldering permanently joins metal contacts by melting a low-melting-point tin-lead (or lead-free tin-silver-copper) alloy between them. The solder creates an electrically conductive mechanical bond. Soldering is used in Jarvis for joining wires to terminal blocks, headers, and sensor connectors.

| Specification | Detail |
| --- | --- |
| Temperature | 330–370°C for lead-free solder (Sn96/Ag3/Cu0.5). 280–320°C for leaded solder (Sn60/Pb40). Too cold = cold joints. Too hot = burnt flux, oxidised tip |
| Tip tinning | Before every soldering session: melt a small amount of solder onto the iron tip. Wipe on wet sponge. Repeat. A clean, shiny tip conducts heat efficiently. A black oxidised tip does not |
| Joint technique | 1. Heat the joint (both wire and pad), not the solder. 2. Apply solder to the joint, not to the iron. 3. Remove solder. 4. Remove iron. 5. Do not move joint for 3 seconds |
| Good joint | Shiny, smooth, volcano-shaped. Wets both surfaces. Feels solid when gently tugged |
| Cold joint | Dull, grey, lumpy or balled. Poor electrical contact. Reheat and re-flow with fresh flux |
| Fumes | Solder flux fumes are irritants. Work near an open window or use a fume extractor. Do not inhale directly |
| Burns | The iron tip reaches 350°C within 30 seconds of turning on. Set iron in its stand when not in use. Never set it down on a surface. Never pass it between people while hot |
| After use | Tin the tip (apply fresh solder before switching off). Switch off. Allow to cool in stand before storing |
| 💡 30-SECOND TEST After soldering a connection, gently tug the wire with your fingers. A good joint holds against reasonable pulling force. If the wire pulls free easily, the joint is cold — reheat, apply flux, and re-solder. |  |

### C4 — The Multimeter

The multimeter is the single most important diagnostic tool in the Jarvis build. It measures voltage, resistance, and current, and has a continuity (beep) mode for tracing connections. Every wiring problem in Jarvis can be diagnosed with a multimeter.

| Specification | Detail |
| --- | --- |
| CONTINUITY MODE (🔊) | Beeps when probes are connected through a low-resistance path. Use to: verify wire continuity, check for short circuits (beep between positive and ground = bad), confirm connections before power-on |
| DC VOLTAGE MODE (DCV) | Measures voltage between two points. Set to 20V range for 5V/12V measurements. Black probe = GND. Red probe = test point. Read voltage on display |
| RESISTANCE MODE (Ω) | Power OFF the circuit first. Measures resistance of components or connections. Zero = short circuit. OL (overload) = open circuit |
| CURRENT MODE (DCA) | Advanced use. Circuit must be broken and meter inserted in series. Generally not needed for Jarvis diagnostics |
| BEFORE POWERING UP | Check continuity between 5V rail and GND: should NOT beep. Check between 12V rail and GND: should NOT beep. A beep = short circuit = do not apply power |
| AFTER WIRING | With power on, check: 5V rail = 5V±0.25V. 12V rail = 11V–12.6V. Arduino 3.3V pin = 3.3V±0.1V. Any deviation indicates wiring or power supply issue |

### C5 — Wire Strippers and Crimpers

Clean wire preparation is essential for reliable connections. Wire strippers remove insulation without cutting the conductor. Crimpers terminate wires into connectors without soldering.

| Specification | Detail |
| --- | --- |
| Wire gauge | Jarvis uses: 22–24 AWG for signal wires (sensor, serial, GPIO). 18–20 AWG for power wires (5V, 12V, GND). AWG: smaller number = thicker wire |
| Stripping | Match the stripper notch to wire gauge. Grip and rotate slightly before pulling. If copper strands break, the hole was too small — move to next size up |
| Strip length | 5–7mm of bare conductor for screw terminals. 3–4mm for soldering. Too much bare wire risks short circuits from adjacent conductor contact |
| Tinning stripped ends | Before inserting into a terminal or soldering: apply solder to the stripped end (tin it). This keeps strands together and improves contact |
| Dupont crimpers | Used for making female/male dupont connectors (the 2.54mm pin connectors used on breadboards and Arduino headers). Grip connector in appropriate jaw, insert stripped wire, crimp. Test by pulling |
| Ferrule crimpers | Used with small metal ferrules that slide over stripped wire ends before inserting into screw terminals. Prevents wire fraying over time in vibrating equipment |

## Code Explained Line by Line — D1–D5

Every significant code block in the Jarvis system, explained from first principles. Refer to these entries when modifying code or when a section of code is not behaving as expected.

### D1 — jarvis.py: Main Brain Architecture

jarvis.py is the central orchestration script running on the Raspberry Pi. It coordinates all four sub-systems. The code is structured as a state machine — Jarvis is always in one state (sleeping, waking, listening, processing, speaking) and transitions between states based on events.

# ─── IMPORTS ───────────────────────────────────────────────

import serial # pyserial: talks to Arduino Uno and Mega

import speech_recognition as sr # Google Speech API wrapper

import pyttsx3 # Text-to-speech (offline, uses espeak)

import google.generativeai as genai # Gemini AI API

import RPi.GPIO as GPIO # Raspberry Pi hardware GPIO

import cv2 # OpenCV: camera capture (optional)

import time, json, threading # Standard library

The serial library opens the USB serial connections to the Uno (/dev/ttyACM0) and Mega (/dev/ttyACM1). The port order depends on which device is plugged in first — verify with ls /dev/ttyACM* before each boot.

# ─── STATE MACHINE ──────────────────────────────────────────

STATES = ['sleeping', 'waking', 'listening', 'processing', 'speaking']

current_state = 'sleeping'

def set_state(new_state):

global current_state

current_state = new_state

# Each state change triggers LED update:

if new_state == 'listening': pico.write(b'B') # Blue

elif new_state == 'processing': pico.write(b'G') # Gold

elif new_state == 'speaking': pico.write(b'W') # White

else: pico.write(b'X') # Off

The state machine ensures LEDs always match what Jarvis is doing, and prevents simultaneous operations (e.g., Jarvis cannot start listening for a new question while still speaking the answer to the previous one).

### D2 — Arduino Uno: Wake Detection Code

The Uno sketch runs the moment the board powers on and loops indefinitely. It reads the HC-SR04 Sensor 1 every 200ms and sends a WAKE signal to the Pi if a person is detected within the threshold distance.

// ─── VARIABLES ──────────────────────────────────────────────

const int trigPin = 2; // TRIG to HC-SR04 Sensor 1

const int echoPin = 3; // ECHO from HC-SR04 Sensor 1

const int wakePin = 13; // To Pi GPIO 17 (optional direct GPIO)

const float WAKE_DIST = 100.0; // cm — wake if person within 1m

bool asleep = true; // Track whether we've already woken Pi

float getDistance() {

digitalWrite(trigPin, LOW); delayMicroseconds(2);

digitalWrite(trigPin, HIGH); delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long dur = pulseIn(echoPin, HIGH, 30000); // 30ms timeout

return dur / 58.0;

}

void loop() {

float dist = getDistance();

if (dist > 0 && dist < WAKE_DIST && asleep) {

Serial.println("WAKE"); // Pi reads this via USB serial

asleep = false; // Don't send again until person leaves

}

if (dist > WAKE_DIST ¦¦ dist == 0) {

asleep = true; // Reset — ready to wake again

}

delay(200); // Check 5 times per second

}

The asleep boolean prevents flooding the Pi with WAKE messages — it sends one signal per visitor approach, not 5 per second. The timeout on pulseIn (30000 microseconds = 30ms) prevents the code from hanging if no echo returns (object out of range).

### D3 — Arduino Mega: Motor Control Code

The Mega sketch handles all motor operations. It implements three safety mechanisms: the kill-zone (hardware proximity), serial timeout (stops motors if Pi goes silent), and command validation (ignores malformed commands).

// ─── MOTOR PINS (L298N) ──────────────────────────────────────

const int ENA=3, IN1=7, IN2=8; // Motor A (Left drive)

const int ENB=5, IN3=10, IN4=11; // Motor B (Right drive)

const int trig2=A0, echo2=A1; // Sensor 2 (kill-zone)

const int SPEED = 180; // PWM 0-255 (70% speed)

const int KILL_DIST = 30; // cm

void driveForward() {

if (getDistance2() < KILL_DIST) { stopMotors(); return; }

analogWrite(ENA, SPEED); analogWrite(ENB, SPEED);

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW); // A fwd

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW); // B fwd

}

void loop() {

if (Serial.available()) {

char cmd = Serial.read();

lastCommandTime = millis(); // Reset watchdog timer

if (cmd == 'F') driveForward();

else if (cmd == 'B') driveBackward();

else if (cmd == 'L') turnLeft();

else if (cmd == 'R') turnRight();

else if (cmd == 'S') stopMotors();

}

// Watchdog: if no command for 3 seconds, stop motors

if (millis() - lastCommandTime > 3000) stopMotors();

}

The watchdog timer (3-second serial timeout) is critical: if the Pi crashes or the USB cable is disconnected while Jarvis is moving, the Mega will halt movement within 3 seconds rather than continuing indefinitely. This is hardware-level safety that software cannot bypass.

### D4 — Raspberry Pi Pico: LED Control Code

The Pico runs a MicroPython script that waits for single-character commands from the Pi and updates the NeoPixel ring accordingly. The heart pulse mode uses a loop with time.sleep() calls to smoothly fade LEDs in and out.

import machine, neopixel, sys, time

# ─── SETUP ───────────────────────────────────────────────────

NUM_LEDS = 12 # Number of LEDs in ring

DATA_PIN = 28 # GPIO pin connected to NeoPixel DIN

np = neopixel.NeoPixel(machine.Pin(DATA_PIN), NUM_LEDS)

def set_all(r, g, b):

for i in range(NUM_LEDS):

np[i] = (r, g, b) # neopixel library uses RGB order

np.write()

def heart_pulse():

# 3 slow pink pulse cycles

for _ in range(3):

for brightness in range(0, 60, 3): # Fade in

set_all(brightness, 0, brightness//3) # Soft pink

time.sleep(0.03)

for brightness in range(60, 0, -3): # Fade out

set_all(brightness, 0, brightness//3)

time.sleep(0.03)

# ─── MAIN LOOP ────────────────────────────────────────────────

while True:

if uart.any():

cmd = uart.read(1).decode() # Read one byte

if cmd == 'B': set_all(0, 0, 30) # Blue (dim)

elif cmd == 'G': set_all(15, 10, 0) # Gold

elif cmd == 'W': set_all(25, 25, 25) # White

elif cmd == 'X': set_all(0, 0, 0) # Off

elif cmd == 'H': heart_pulse()

LED brightness values are kept low (0–30 out of 255 maximum) for the exhibition environment. Full brightness (255) blinds viewers at close range and draws excessive current. Adjust these values to suit your exhibition lighting conditions.

### D5 — WebSocket Server: jarvis_ws.py

jarvis_ws.py runs alongside jarvis.py on the Pi and broadcasts system state to connected browser clients. It uses the websockets library (asyncio-based) for non-blocking concurrent handling of multiple dashboard connections.

import asyncio, websockets, json, psutil, serial, time

# psutil: Python system utilities — reads CPU, memory, temperature

# serial: reads motor/LED status from Arduinos

async def get_system_state():

return {

'cpu': psutil.cpu_percent(), # CPU % 0-100

'memory': psutil.virtual_memory().percent,

'temp': psutil.sensors_temperatures()['cpu_thermal'][0].current,

'uptime': int(time.time() - boot_time),

'ip_address': get_ip(),

'wifi_ssid': get_ssid(),

'clients': len(connected_clients),

'motors': current_motor_state,

'led_state': current_led_state,

}

async def handler(websocket):

connected_clients.add(websocket)

try:

async for message in websocket: # Receive commands

cmd = json.loads(message)

if cmd['command'] == 'move':

mega.write(cmd['direction'].encode())

elif cmd['command'] == 'set_led':

pico.write(cmd['state'].encode())

finally:

connected_clients.remove(websocket)

async def broadcast_loop():

while True:

if connected_clients:

state = await get_system_state()

await asyncio.gather(*[

ws.send(json.dumps(state))

for ws in connected_clients

])

await asyncio.sleep(1) # Push updates every second

The asyncio event loop handles multiple operations concurrently on a single thread: receiving WebSocket messages from the browser, broadcasting system state every second, and reading serial data from Arduinos — without any operation blocking the others.

# PART 5 — EXHIBIT OPERATIONS

Complete operations guide for exhibit day. Sections: F1–F16 Troubleshooting Guide (F12–F13 cover Johnson motor issues), G1–G7 Exhibit Day Survival, FAQ, and Glossary.

## Troubleshooting Guide — F1–F16

The complete troubleshooting guide for Project Jarvis. Each entry includes the symptom, the most likely root cause, and the fastest verified fix. F12–F13 cover Johnson motor-specific issues. Entries F14–F16 follow in the subsequent section of this document.

### F1 — Jarvis Does Not Respond When Someone Approaches

Jarvis stays silent when a person walks up. The wake detection sequence does not trigger.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| No response at any distance | Uno not running or not sending to Pi | SSH into Pi. Run: python3 -c "import serial; s=serial.Serial('/dev/ttyACM0',9600); print(s.readline())". Hold hand near Sensor 1. Should print 'WAKE'. If nothing prints: check Uno USB, re-upload sketch |
| Works close up but not at 1m | WAKE_DIST threshold too low in Uno sketch | Edit Uno sketch: increase WAKE_DIST from 100 to 120. Re-upload. Test again at 1m |
| Triggers constantly with no one present | Sensor aimed at nearby wall or furniture | Check sensor 1 field of view. If exhibit location has a wall within 1m in front of Jarvis, move Jarvis or reduce WAKE_DIST to 60 |
| Uno sends WAKE but Pi does not respond | Serial port mismatch in jarvis.py | Check port assignment. Run: ls /dev/ttyACM* with Uno plugged in vs unplugged. Update UNO_PORT in jarvis.py to match |

### F2 — Speech Recognition Returns Nothing or Wrong Words

Jarvis listens but either hears nothing, mishears words badly, or gives the 'Sorry, I did not catch that' response consistently.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Speech recognition fails in quiet room | Microphone not recognised by Pi OS | Run: arecord -l. Should list a USB audio device. If not: unplug/replug mic, try different USB port. Run: alsamixer and increase mic input level |
| Recognises OK but misses words in noisy hall | Mic gain too low or mic position wrong | Move mic to chest panel, facing outward. Increase gain in alsamixer (F6 to select card, navigate to Capture, press M to unmute, Up arrow to increase) |
| Recognises wrong language | Default locale not set correctly | In speech_recognition, set: r.recognize_google(audio, language='en-IN') for Indian English, or 'en-US' for American |
| Recognition very slow (5+ seconds) | Pi is overloaded or Wi-Fi latency high | Close any other processes. Check Wi-Fi signal strength: iwconfig wlan0. If poor signal, move exhibit closer to router |

### F3 — Gemini AI Returns Error or No Response

The speech is recognised correctly but the AI response fails. Jarvis says 'I could not reach my AI at the moment' or similar fallback.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| API key error in terminal | GOOGLE_API_KEY not set or incorrect | Run: echo $GOOGLE_API_KEY. Should print your key. If empty: run: export GOOGLE_API_KEY='your-key-here'. Add to ~/.bashrc to persist |
| Connection timeout | No internet connection on Pi | Run: ping 8.8.8.8. If no response: check Wi-Fi. Run: sudo raspi-config → System Options → Wireless LAN. Re-enter network credentials |
| Rate limit error (429) | Too many requests per minute | Add time.sleep(1) before each API call in jarvis.py. Free Gemini tier: 15 requests/minute |
| Model not found error | Model name changed in Gemini API | Update model name in jarvis.py. Check current model names at ai.google.dev. Try 'gemini-1.5-flash' or 'gemini-pro' |

### F4 — Text-to-Speech (TTS) Silent or Distorted

Jarvis processes the question and generates a response but either nothing plays from the speaker, or the audio is badly distorted.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Complete silence from speaker | Speaker not selected as ALSA output device | Run: aplay -l (list sound devices). Run: speaker-test -c2. Set default: create/edit ~/.asoundrc with: pcm.!default { type hw; card 1; } |
| Robotic/clipped audio quality | pyttsx3 espeak rate set too high | In jarvis.py: engine.setProperty('rate', 140). Default is 200 — too fast for exhibition. 140–160 is clear |
| Low volume even at max setting | Speaker too small for hall acoustics | Set volume to max: engine.setProperty('volume', 1.0). Also run alsamixer and set Master to 100%. If still insufficient, use a larger USB speaker |
| Audio cuts out mid-sentence | USB power supply insufficient — audio glitches under load | Verify power bank output ≥3A. Try a powered USB hub between Pi and speaker |

### F5 — Motors Do Not Move

Jarvis's drive motors show no response to movement commands from the Pi, either during testing or during exhibition.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| No movement, Mega is powered | 10000mAh power bank disconnected, discharged, or 12V output not active | Check power bank output: confirm 12V output port is active and reading 11.5V–12.6V on multimeter. If using a boost converter, verify output is set to 12V. Check all connections to L298N: 12V to +12V terminal, GND to GND terminal. Note: many power banks auto-shutoff at low current — ensure L298N load keeps it awake. |
| Mega receives commands (LEDs respond) but no motor movement | L298N ENA/ENB not receiving PWM signal | Confirm ENA and ENB jumpers are removed (jumpers bypass PWM, locking motors at full speed with no speed control). If removed, check wiring from Mega PWM pins 3 and 5 |
| One motor works, one does not | Single motor channel failure or loose connector | Swap motor connections on L298N (Motor A terminals ↔ Motor B terminals). If problem follows the connection, motor is faulty. If problem stays on L298N channel, replace L298N |
| Motors move but very slowly or weakly | 10000mAh power bank low or auto-shutoff triggered | Recharge the 10000mAh power bank. Under 10V, L298N output drops significantly. Motors may also make louder whining sounds at low voltage |

### F6 — Kill-Zone Not Working (Robot Drives Into Obstacles)

Jarvis continues moving forward even when an obstacle is within 30cm of Sensor 2, or stops even when nothing is in front.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Kill-zone never triggers | Sensor 2 wires not connected to Mega | Check HC-SR04 Sensor 2: TRIG to Mega A0, ECHO to Mega A1, VCC to 5V, GND to GND. Test sensor independently with Serial Monitor |
| Kill-zone triggers when nothing present | Sensor 2 aimed at floor or nearby wall | Tilt Sensor 2 upward 5–10 degrees. Check KILL_DIST constant in Mega sketch — if too large (e.g. 100cm), Jarvis will stop too early |
| Kill-zone only works intermittently | Motor EMI coupling into sensor echo line | Re-route Sensor 2 echo wire away from motor power wires. Use twisted pair for trig/echo. Add 100µF capacitor across L298N 12V supply terminals |
| Robot backs into obstacles (no rear sensor) | Expected behaviour — no rear sensor in current design | This is by design. Jarvis has front-only sensing. Brief rearward movement is safe in controlled exhibition layout. Mark rear exclusion zone with tape on floor |

### F7 — NeoPixel Ring Not Lighting Up

The arc reactor LEDs remain dark, flicker, or show wrong colours.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| All LEDs dark | NeoPixel 5V power not connected | Check NeoPixel VCC wire connects to 5V rail (from Pi power bank). GND connects to common GND. Data wire connects to Pico GPIO 28 |
| LEDs flicker or show random colours | Data line too long or no series resistor | Add a 470Ω resistor on the data line between Pico GPIO 28 and NeoPixel DIN. Keep data wire under 30cm |
| Only first few LEDs light | Insufficient 5V current for full ring | The NeoPixel ring requires up to 1A at full brightness from the 5V supply. Ensure power bank provides ≥3A on Pi's port. Use a dedicated 5V 2A supply to the NeoPixel if power bank is marginal |
| Wrong colours (e.g. command is Blue but shows Green) | Colour order issue in Pico code | WS2812B uses GRB not RGB. Check set_all() calls: Blue should be (0, 0, 30) not (0, 30, 0). The neopixel MicroPython library handles GRB internally — use (R, G, B) tuples |

### F8 — Pi Cannot Connect to Wi-Fi

jarvis.py starts but AI responses fail because the Pi has no internet connection.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Pi connects at home but not at exhibition venue | Exhibition venue uses different SSID/password or captive portal | Create a Wi-Fi hotspot from a phone using the exhibition team member's phone. Configure Pi to connect to that hotspot SSID. Test at home before exhibit day |
| Pi fails to reconnect after brief dropout | NetworkManager not set to auto-reconnect | Run: sudo systemctl enable NetworkManager. Ensure auto-connect is enabled for the network: nmcli connection modify 'SSID' connection.autoconnect yes |
| Pi connects but Gemini API still fails | DNS resolution failing, not routing issue | Run: ping 8.8.8.8 (IP address test). If OK, run: ping generativelanguage.googleapis.com. If second fails, DNS issue. Run: echo 'nameserver 8.8.8.8' ¦ sudo tee /etc/resolv.conf |
| Cannot find Pi IP address at venue | DHCP assigned new IP on different network | Connect monitor to Pi. Run: hostname -I. Or use phone hotspot (Pi gets fixed IP from phone hotspot — typically 192.168.43.X) |

### F9 — Arduino Code Upload Fails

Trying to upload a sketch via the Arduino IDE results in errors: port not found, programmer error, or upload timeout.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Port not available in Arduino IDE | Wrong USB cable or device driver issue | Try a different USB cable — charge-only cables have no data wires. On Windows: install CH340 driver (for most Arduino clones). On Mac/Linux: usually plug-and-play |
| avrdude: stk500_recv(): programmer is not responding | Wrong board selected in IDE | Tools → Board: select 'Arduino Uno' for Uno, 'Arduino Mega 2560' for Mega. Also check Tools → Processor matches the board |
| Upload succeeds but code does not run | Sketch compiled for wrong processor | Verify board selection. An Uno sketch uploaded to a Mega will compile but may not run correctly if it references Uno-specific registers |
| Serial port busy | jarvis.py holding the serial port open | Stop jarvis.py (Ctrl+C in SSH terminal) before uploading. The Pi's serial connection to the Arduino and the IDE's upload connection are mutually exclusive |

### F10 — Pi Crashes or Reboots During Operation

The Raspberry Pi resets unexpectedly during the exhibition: SSH connection drops, terminal freezes, or jarvis.py restarts mid-conversation.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Pi reboots with no error | Under-voltage from insufficient power supply | Check power bank output is ≥3A on Pi's port. A yellow lightning bolt icon on Pi desktop (if monitor attached) indicates under-voltage. Try different USB-C cable — many cables cannot handle 3A |
| Pi freezes but does not reboot (SSH hangs) | SD card corruption or thermal throttling | Check Pi temperature: vcgencmd measure_temp. Over 80°C triggers throttling and instability. Add passive heatsink to Pi CPU. For SD corruption: re-flash with Raspberry Pi Imager on a fresh card |
| jarvis.py crashes with MemoryError | Pi RAM exhausted (2GB model with multiple processes) | Use 4GB Pi. Or reduce simultaneous processes: disable camera capture if not needed. Monitor with: htop via SSH |
| Random crashes only during motor movement | 12V motor noise on shared ground | Add 100µF electrolytic capacitor across L298N supply terminals. Ensure motor ground and logic ground are connected at a single point, not multiple points |

### F11 — Serial Communication Between Pi and Arduino Corrupted

The Pi receives garbled data from an Arduino, or commands from the Pi are not executed correctly.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Garbled characters in serial output | Baud rate mismatch | Pi jarvis.py opens port at 9600. Arduino Serial.begin() must also be 9600. Any mismatch produces gibberish. Check both files |
| Commands delayed by 1–2 seconds | Read buffer accumulating unread data | Use readline() not read() in jarvis.py, and use Serial.println() not Serial.print() in Arduino sketch. Newlines are the message delimiter |
| Arduino receives commands but Mega ignores them | Wrong byte encoding | Pi sends: mega.write(b'F') — the b prefix is essential. Without b, Python sends a string object, not bytes. Arduino reads bytes. Type mismatch = no response |
| Pi reads stale old data after reconnect | Serial buffer not flushed on reconnect | In jarvis.py, after opening serial port: add time.sleep(2) then ser.reset_input_buffer(). Arduino resets on USB reconnect — the 2 second delay lets bootloader finish before sending data |

F14 — Ghost Echoes Robot stops when nothing is in front

Ghost echoes are false obstacle readings — the sensor 'sees' something at 20–30cm even in clear space. Three causes account for 95% of all ghost echo reports.

| Cause | Why It Happens | Fix |
| --- | --- | --- |
| Sensor aimed at floor | Floor is 20–30cm away — always inside the kill-zone cone. The 15° beam angle hits the floor before it reaches an obstacle. | Tilt both HC-SR04 sensors 5–10° upward. Re-test: wave hand, confirm 60cm+ reads with nothing present. |
| Reflective surfaces | Glass, mirrors, and glossy panels bounce the ultrasonic pulse sideways, creating phantom echoes from multiple directions. | Point sensors away from reflective walls. Add a small felt pad behind sensor to dampen side-lobe echoes. |
| Motor wire EMI | Motor power wires carry pulsing high current (PWM). This generates electromagnetic noise that couples into the sensor echo line, corrupting pulseIn() readings. | Re-route sensor wires along the OPPOSITE side of the torso from motor wires. Use twisted-pair for echo+trig lines if possible. |
| Loose echo wire | An intermittent echo connection reads '0' duration, which is mapped to 999cm — but a partial contact can also return spurious short values. | Pull-test all HC-SR04 connectors. Re-seat firmly. Continuity-test from Mega pin to sensor pin with multimeter. |

💡 PRO TIP Quick Ghost Echo Test: SSH into Pi, stop jarvis.py. Open Arduino Serial Monitor at 9600 baud on the Mega. Hold a flat sheet of cardboard 50cm directly in front of the sensor and move it slowly forward. Readings should decrease smoothly. Any sudden jumps to <30 with cardboard at 50cm = sensor aim or EMI issue.

F15 — USB Ports Not Detected Pi cannot find Mega or Pico

The Raspberry Pi must detect 3 devices when running ls /dev/ttyACM*: the Mega (motor control), the Pico (LED control), and often the webcam on a separate bus. A missing device is always one of four issues.

- Run lsusb on Pi. You should see: CP2102 or CH340 (Mega chip), MicroPython USB CDC (Pico), and your USB webcam. Each device gets its own line.

- If any device is missing: swap the USB cable first. Charge-only cables have only 2 wires (power) — data cables have 4. This is the most common cause of missing devices.

- Run ls /dev/ttyACM* before and after plugging in each device. Each device should add one new entry (ttyACM0, ttyACM1, etc.).

- Wrong port order causes jarvis.py to send motor commands to the Pico and LED commands to the Mega. Fix: unplug both, plug Mega FIRST, note its port, then plug Pico. Update MEGA_PORT and PICO_PORT in jarvis.py to match.

- If lsusb shows the device but /dev/ttyACM* does not: the Pi needs the USB-Serial kernel module. Run: sudo modprobe cp210x (for CH340/CP2102) or sudo modprobe cdc_acm (for Pico).

F16 — Pi Won't Boot Green LED silent or stuck

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| No green LED at all | No power reaching Pi | Check power bank output is ≥2.1A on that port. Try a different USB cable. Try a different port on the power bank. |
| Green LED blinks pattern then stops | SD card read error | Re-seat SD card (push until click). If still fails: re-flash OS using Raspberry Pi Imager. Use Samsung EVO or SanDisk Ultra — cheap cards fail within weeks. |
| Green LED blinks then steady | Booted — normal | Wait 60 seconds total. Then SSH in. First boot is slower (30% longer) due to filesystem expand. |
| Pi boots but no Wi-Fi | Wrong SSID/password in Imager | Connect monitor+keyboard to Pi. Run: sudo raspi-config → System Options → Wireless LAN. Re-enter SSID and password. |
| Can't find Pi's IP address | DHCP assigned a different IP | Log into router admin page (usually 192.168.1.1). Look for 'jarvis' in connected devices. Alternatively: sudo nmap -sn 192.168.1.0/24 from a laptop on the same network. |
| Pi boots but jarvis.py won't start | Missing Python library | Read the exact error. Install missing library: pip3 install [name] --break-system-packages. pyaudio needs: sudo apt install portaudio19-dev first. |

F12 — Johnson Motors Run but Robot Moves Slowly Motors spin but robot moves slower than expected

Johnson motors have higher starting resistance than plastic-gear BO motors. A PWM value of 150 may not provide enough torque to overcome this resistance, especially on carpet or uneven surfaces. This is particularly noticeable from a standing start.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| Motors spin but robot barely moves or moves slowly | PWM value of 150 too low for Johnson motor starting resistance | In Mega code, change forward(150,150) to forward(180,180). Re-upload. Try values from 180–200. Adjust both motors equally first, then fine-tune for straight tracking. |
| One motor spins, the other does not | L298N channel failure from stall current, or loose terminal | Check and tighten OUT1–OUT4 terminal screws. If problem persists, the L298N channel may have burned out from stall current. Swap to spare L298N. |

F13 — L298N Overheating with Johnson Motors L298N too hot to hold after short running period

The L298N has a 2A per channel rating. Johnson motors draw 0.8–1.5A during normal running, but stall current can reach 3–4A per motor — enough to overheat the L298N within seconds if a motor is blocked. The L298N also has an inherent 3V voltage drop that dissipates as heat regardless of motor load.

| Symptom | Most Likely Cause | Fix |
| --- | --- | --- |
| L298N too hot to touch after 30 seconds of running | Normal for Johnson motors but airflow may be insufficient. Heatsink tab not exposed. | Ensure heatsink tab is fully exposed with clear airflow. Mount a small 5V fan directed at the L298N for extended operation. Allow 60 seconds of rest between long runs. |
| Overheats within 10 seconds even without load | Motor stall or wiring short causing sustained high current draw | Stop immediately. Check that wheels spin freely by hand with no mechanical obstruction. Check for wiring shorts with multimeter. Replace L298N if internal damage is suspected. |

⚠ WARNING NEVER cut power to the Pi without running sudo shutdown -h now first. The Pi writes to the SD card continuously. Hard power cuts damage the filesystem. This is the single most common cause of SD card corruption in field deployments.

## Exhibit Day Survival — G1–G7

Seven reference cards for September 16th. Print the G3 Fast Fix Guide and the G1 Checklist separately and tape them inside the Oh-Shit Kit bag.

G1 — 30-Minute Setup Checklist Arrival to exhibit-open sequence

| Time | Action | Confirm |
| --- | --- | --- |
| T−30 min | Place Jarvis upright on exhibit table or floor. NEVER lay horizontally — horizontal transport stresses joints designed for vertical load. | ☐ |
| T−28 min | Connect 20,000mAh power bank to Pi micro-USB. Start your 60-second boot timer. | ☐ |
| T−27 min | While Pi boots: unpack Oh-Shit Kit. Arrange tools within reach but out of visitor sight. | ☐ |
| T−26 min | Pi should be booted. SSH in from laptop. Run: ls /dev/ttyACM* — confirm at least 2 devices. | ☐ |
| T−24 min | Connect the 10000mAh power bank (12V output port) through the kill switch to L298N. Confirm power bank is charged, 12V output is active, and kill switch is in ON position. | ☐ |
| T−23 min | SSH terminal: python3 jarvis.py — wait for 'Jarvis is ready...' confirmation message. | ☐ |
| T−20 min | Walk in front of Jarvis at 1m. Should: move forward briefly, stop, LEDs go blue, listen. | ☐ |
| T−18 min | Speak test phrase: 'What is your name?' Wait for Gemini response. Confirm speaker is audible. | ☐ |
| T−15 min | Kill-zone test: while Jarvis is moving, step to within 30cm. Wheels must stop instantly. | ☐ |
| T−10 min | Check all visible wires are cable-tied. No wires visible through body panels. | ☐ |
| T−8 min | Assign team roles (see G6). Brief everyone on visitor handling procedure. | ☐ |
| T−5 min | Run 5-Minute Pre-Show Test (G2). | ☐ |
| T−0 | Exhibit opens. One team member stationed at Jarvis at all times. | ☐ |

G2 — 5-Minute Pre-Show Test Final 7 checks before visitors arrive

- Power bank >80% charge. Verify by checking indicator LEDs on the power bank.

- 10000mAh power bank connected (12V output active) and kill switch ON. jarvis.py running — confirm in SSH terminal.

- Walk test: approach from 2m. Wake trigger should fire at approximately 1m.

- Voice test: ask 3 different questions. All must receive Gemini responses with spoken output.

- LED state test: all 5 states confirmed: B=Blue (thinking), G=Gold (ready), W=White (speaking), X=Off. Note: the H state is not tested publicly.

- Kill-zone test: step to within 30cm while moving. Wheels stop. Step back: resume movement on next wake trigger.

- Speaker volume: audible at 3m in the exhibit environment. Adjust pyttsx3 rate/volume in jarvis.py if needed.

G3 — During-Show Fast Fix Guide 60-second field repairs — print this card

| Problem | Probable Cause | Fix in 60 Seconds |
| --- | --- | --- |
| Jarvis stops moving mid-demo | Kill-zone triggered — person too close | Ask visitor to step back beyond 30cm. Jarvis resumes automatically on next wake. |
| LED stuck on one colour | jarvis.py exception loop | SSH: Ctrl+C → python3 jarvis.py |
| Speech recognition loops endlessly | Microphone picking up speaker output (echo) | SSH: Ctrl+C. Restart jarvis.py. Cover mic during restart. |
| Pi disconnected from SSH | Wi-Fi dropped | Plug spare monitor+keyboard directly into Pi. Restart jarvis.py from local terminal. |
| Motor power bank depleted | 10000mAh motor power bank depleted or auto-shutoff triggered | Jarvis announces 'drive systems lost power'. Recharge or swap the 10000mAh motor power bank and re-enable 12V output, then flip kill switch ON. Pi stays alive on the logic USB power bank. |
| Jarvis won't stop talking | pyttsx3 engine stuck | SSH: sudo pkill python3. Wait 10 seconds. Restart jarvis.py. |
| Complete freeze — no response | Pi kernel hang | SSH: sudo reboot. 90-second reboot. Tell visitors: 'Jarvis is updating his neural pathways.' |
| Visitor too loud, no recognition | Distance or background noise | Ask visitor to step closer (50cm) and speak toward Jarvis's eye level at conversational volume. |

G4 — Safe Shutdown Sequence 5 steps — ALWAYS follow this order

| Step | Action | Why This Matters |
| --- | --- | --- |
| 1 | Flip kill switch OFF or disconnect the 10000mAh power bank from L298N. | Prevents any accidental motor movement during the shutdown process. |
| 2 | SSH terminal: sudo shutdown -h now | Linux closes all files, flushes disk writes, and unmounts the SD card safely before stopping power. |
| 3 | Watch the green LED on the Pi. Wait 15–20 seconds. | Blinking green = disk activity. When the green LED stops blinking permanently, shutdown is complete. |
| 4 | ONLY NOW unplug the power bank from Pi. | Disconnecting power during disk writes destroys the SD card filesystem. |
| 5 | Coil all cables. Pack Jarvis upright in transport bag. | Horizontal transport puts lateral stress on joints designed for vertical load only. |

⚠ WARNING REPEAT THIS FROM MEMORY: Kill switch OFF → sudo shutdown -h now → wait for green LED to stop → unplug power bank. This 4-step sequence prevents every SD card failure we've ever encountered.

G5 — Oh-Shit Kit Contents Pack this before you leave home

Tape a printed checklist inside the bag. Every item below has saved a project at a critical moment.

| Spare 32GB MicroSD card | Pre-flashed with OS, jarvis.py, and all pip packages. Test it works BEFORE exhibit day. |
| --- | --- |
| USB cables × 3 | 2× USB-A to USB-B (for Arduinos), 1× micro-USB (for Pi). Charge-only cables are the #1 cause of ghost hardware failures. |
| Multimeter | Continuity test any suspect wire in 30 seconds. Non-negotiable. |
| Soldering iron + solder + helping hands | For field wire repairs. A cold joint will fail under vibration during transport. |
| Spare L298N motor driver × 1 | L298N failure from stall current is possible. Swap takes 5 minutes. |
| Spare HC-SR04 sensors × 2 | Sensors are fragile. A dropped robot can crack the transducer face. |
| Spare Johnson motor × 2 | Motor brush failure is rare but catastrophic without a spare. |
| Spare 2200µF capacitor × 2 (or 1000µF if 2200µF unavailable) | The brownout preventer. Costs pence. Saves everything. |
| Spare 330Ω resistors × 10 | NeoPixel data-line protection. Can re-solder in the field. |
| Small screwdriver set (Phillips + flat) | For L298N terminals and Arduino shield screws. |
| Hot glue gun + sticks | Emergency structural repair and cable management. |
| Sticky tape + craft knife | Panel re-attachment and cable tidy. |
| Laptop | With Arduino IDE + Thonny installed, and API key saved in a plaintext file. |
| Phone hotspot | If venue Wi-Fi fails, Pi uses phone hotspot for Gemini API. Pre-configure in raspi-config BEFORE exhibit day. |
| Printed G1 setup checklist and G3 fast fix guide | For the team member without SSH access. Laminate if possible. |

G6 — Team Role Cards Brief your team 8 minutes before opening

| Role | Responsibility |
| --- | --- |
| Lead Architect | Technical oversight. Makes final calls on repairs. Monitors SSH terminal. Primary spokesperson for advanced technical questions from visitors and judges. |
| Visitor Greeter | Stands 2m from Jarvis. Invites visitors to approach. Manages crowd distance — nobody enters the 30cm kill-zone unannounced. |
| Explainer | Stands beside Jarvis. Narrates what Jarvis is doing in real time ('See — the chest just turned gold, that means he's thinking...'). |
| Technical Monitor | Watches SSH terminal on laptop. Alerts Lead if error messages appear. Restarts jarvis.py if it crashes. |
| Transport/Pack | Handles all physical movement, cable coiling, and packaging during setup and shutdown. Responsible for Oh-Shit Kit. |

G7 — Handling Difficult Visitor Questions Honest, confident answers always work best

| Question | Suggested Answer |
| --- | --- |
| 'How much did this cost?' | The total build was kept under budget. The most expensive single parts were the Raspberry Pi and the power bank. The AI (Gemini) is completely free via Google's developer API. |
| 'Can it see me?' | Yes. The eye has a USB camera connected to OpenCV — an AI vision library. It can detect and track faces in real time. |
| 'Why doesn't it walk?' | Walking robots spend most of their computing power on balance algorithms. By using a wheeled base, we free 100% of the CPU for intelligence and personality. Jarvis's design philosophy is 'smart over athletic'. |
| 'Is it dangerous?' | It has two independent safety systems. Sensor 1 wakes it up when you approach. Sensor 2 stops the wheels the instant you come within 30cm — it physically cannot run into you. |
| 'Can I touch it?' | Yes — the structure is designed to be handled. Please don't touch the eye lens or open the back panel. |
| 'Is it pre-programmed?' | No. Every answer is generated in real-time by Gemini AI. We don't know what he'll say to any question — he truly thinks for himself. We only programmed the framework. |
| 'What would you do differently?' | Add object recognition to the camera, more movement commands, and a screen for displaying visuals alongside speech. These are v2 features already planned. |
| 'I don't know the answer' | Be honest: 'Great question — I haven't fully studied that part yet, but that behaviour is controlled by the Arduino Mega.' Never guess. Never fabricate technical details. |

# FAQ — EVERYTHING A BEGINNER NEEDS TO KNOW

Every common question asked during the build and exhibit. Read these before asking someone else — the answer is almost certainly here.

## Electronics & Hardware Basics

### Q: What is a Raspberry Pi?

A Raspberry Pi is a credit-card-sized computer running a full Linux operating system. It has USB ports, Wi-Fi, a headphone jack, and GPIO pins (programmable metal legs). The Pi 3B+ in Jarvis is the 'master brain' — it runs the AI, speech recognition, and coordinates every other board. Think of it as a very small laptop without a screen.

### Q: What is an Arduino?

An Arduino is a microcontroller — a simple chip that runs a single program in a continuous loop. Unlike the Pi (which runs Linux and many programs simultaneously), an Arduino does only one thing. This makes it ideal for hard real-time tasks like 'stop the motors if something is within 30cm' — a guarantee the Pi's operating system cannot make. Jarvis uses two Arduinos: one as the wake sensor, one as the motor controller.

### Q: What is a Raspberry Pi Pico?

The Pico is a small microcontroller (like a tiny Arduino) made by Raspberry Pi Ltd. It is NOT a full computer like the Pi 3B+. It runs MicroPython (a lightweight version of Python) and has no operating system. In Jarvis, it is dedicated entirely to controlling the 12 NeoPixel LEDs, receiving single-character commands via USB serial from the Pi.

### Q: What is GPIO?

GPIO stands for General Purpose Input/Output — the metal pins along the side of the Raspberry Pi (and Arduino). Each pin can be programmed to either OUTPUT a voltage (HIGH=3.3V on Pi, HIGH=5V on Arduino) or INPUT a voltage (reading whether something is sending it a signal). In Jarvis, GPIO 17 on the Pi reads the wake-up signal from the Arduino Uno.

### Q: What do 5V and 12V mean?

Voltage is electrical pressure — how hard electricity is being pushed. 5V is the voltage used by all logic boards (Pi, Arduino, Pico). 12V is used by the motors. Motors need more power to spin than a circuit board needs to think. This is why Jarvis has two separate power supplies: a 20,000mAh USB power bank (5V) for the brain, and a 10000mAh power bank (12V output) for the muscles. NEVER connect a 12V supply to a 5V logic board.

### Q: What is a common ground?

Voltage is always measured between two points. When two circuits have different 'ground' references, they speak different voltage languages. If the Pi's 0V ground and the motor circuit's 0V ground are not connected together, the Pi sends a '5V' signal to the motor driver but the motor driver measures it against its own ground and gets a meaningless value. Common ground = a single wire connecting the GND terminals of all circuits so they share the same 0V reference.

### Q: What does continuity mean on a multimeter?

Continuity mode tests whether electricity can flow between two points. Touch the red probe to one end and the black probe to the other end of a wire. If the meter beeps, current can flow — the connection is good. If silence, the connection is broken somewhere. Always test every wire before applying power to a new circuit. This single habit prevents 80% of first-power-on failures.

### Q: What is a breadboard?

Jarvis does NOT use a breadboard. A breadboard is a prototyping tool with holes that connect internally for temporary circuits. Jarvis uses direct wiring — wires soldered to component legs and screwed into terminal blocks. This is more permanent and reliable for a robot that will be moved, handled, and transported.

## Software & Code Basics

### Q: What is SSH?

SSH (Secure Shell) is a way to control the Raspberry Pi from your laptop over Wi-Fi, without connecting a monitor or keyboard to the Pi. On Mac or Linux: open Terminal and type ssh pi@192.168.x.x (using the Pi's IP address). On Windows: use PuTTY. Once connected, every command you type is executed on the Pi.

### Q: How do I find the Pi's IP address?

Three methods: (1) Log into your router's admin page (usually 192.168.1.1 in a browser) and look for a device named 'jarvis'. (2) Connect a monitor and keyboard to the Pi and type: hostname -I. (3) If you set hostname to 'jarvis' in Raspberry Pi Imager, try: ssh pi@jarvis.local — works on most home networks.

### Q: Why does my Raspberry Pi not boot?

Four common causes: (1) MicroSD card not properly seated — push until you feel a click. (2) Power bank not providing enough current — must be 2.1A minimum. (3) MicroSD card corrupted — re-flash the OS using Raspberry Pi Imager. (4) Power cable is charge-only with no data wires — try a different cable. The green LED should blink rapidly for about 30 seconds during boot, then settle into occasional slow blinks.

### Q: What is Python and why does Jarvis use it?

Python is a programming language known for being readable and having enormous libraries (pre-built tools). jarvis.py is written in Python because the Gemini AI library, speech recognition library, and GPIO control library all have excellent Python support. The Pi runs Python 3 natively.

### Q: Why does the code use b'F' instead of just 'F'?

In Python 3, the serial.write() function only accepts bytes objects, not strings. b'F' is a bytes literal — the letter F encoded as its ASCII number (70). Sending 'F' as a plain string causes a TypeError. The rule is: whenever you send data via serial in Python 3, prefix it with b.

### Q: What is a baud rate?

Baud rate is the speed of serial communication, measured in bits per second. Both ends of a serial connection must use identical baud rates, or they hear garbled noise instead of data. Jarvis uses 9600 baud on all serial connections. Check: Serial.begin(9600) in Arduino code and serial.Serial(port, 9600) in Python code.

### Q: What does 'sudo' mean?

Sudo stands for 'superuser do'. On Linux, some commands require administrator (root) permission. Prefixing a command with sudo means 'run this as administrator'. sudo shutdown -h now tells the Pi to perform a safe shutdown. The default password is 'raspberry' unless you changed it during setup.

### Q: The code uploaded but nothing happens — what do I check?

Five-step debug: (1) Open Serial Monitor in Arduino IDE at 9600 baud — the code prints debug messages. (2) Confirm the correct board is selected (Tools → Board). (3) Confirm the correct port is selected (Tools → Port). (4) Ensure the USB cable is a data cable, not a charge-only cable. (5) Press the Reset button on the Arduino after uploading.

## Soldering & Wiring

### Q: I have never soldered before. How do I start?

Start on scrap wire — not on Jarvis. (1) Tin the iron tip: melt a small amount of solder onto the tip so it looks shiny and wet. (2) Hold the iron against the wire and pad together for 2 seconds to heat the joint. (3) Touch solder to the JOINT (not the iron) — it should flow smoothly into the joint. (4) Remove solder first, then remove iron. A good joint looks like a shiny silver volcano. A cold joint looks dull and lumpy — redo it.

### Q: How tight should L298N screw terminals be?

Very tight. Use a small flathead screwdriver. After tightening, pull the wire lightly — it should not move at all. Loose terminals are the single most common cause of 'motors twitch but don't spin'. Recheck all L298N terminals after any transport — vibration loosens them.

### Q: My wire keeps breaking at the solder joint. Why?

This is a stress relief failure. After soldering, the wire needs mechanical support so the solder joint does not carry the physical force. Use a cable tie or hot glue bead 2cm from every solder joint. The solder joint carries the electrical connection; the support carries the physical force. Without this, the wire breaks within 20 flex cycles.

### Q: The HC-SR04 sensor reads garbage distances. Why?

Two common causes: (1) Sensor pointed too close to the floor — at floor level, the 15° cone hits the floor within detection range, causing false echoes. Tilt each sensor 5–10° upward. (2) Motor wires near sensor wires cause electromagnetic interference. Route sensor wires on the opposite side of the torso from motor wires.

## Exhibit Day

### Q: What if the exhibit venue has no Wi-Fi?

Speech recognition and Gemini AI both require internet. Without Wi-Fi, Jarvis loses voice functionality entirely. Solutions: (1) Bring a mobile phone as a Wi-Fi hotspot and pre-configure jarvis to connect to it in raspi-config BEFORE exhibit day. (2) Add the venue's Wi-Fi credentials in raspi-config ahead of time: sudo raspi-config → System Options → Wireless LAN.

### Q: What if our Gemini API key runs out?

Gemini 1.5 Flash has a generous free quota. For one exhibit day, running out is very unlikely. If it does: create a second Google account, get a new API key from aistudio.google.com, SSH into Pi, nano jarvis.py, replace the key, save and restart. The whole process takes under 5 minutes.

### Q: The Pi's green LED is flashing in a pattern. What does it mean?

| Steady slow blink | Normal operation — Pi is running |
| --- | --- |
| Rapid blink (30 seconds) | Booting — wait |
| 4 blinks, pause, 4 blinks | SD card not found — re-seat or replace |
| Constant off | No power — check cable and power bank port |
| No blinking at all (powered off) | If SSH unresponsive too: power cable disconnected |

### Q: How long does the battery last?

The 20,000mAh power bank lasts approximately 16 hours running all four logic boards at typical load. The 10000mAh motor power bank lasts 3–6 hours depending on how frequently the motors run (Johnson motors draw 1.6–3A combined). During a demo, motors run only for the greeting (1 second) and movement commands — sustained full-speed running drains the power bank faster. Always carry a backup charged power bank.

### Q: What if Jarvis says something wrong or inappropriate?

The system prompt in jarvis.py instructs Gemini to keep answers brief and friendly. If an unexpected response occurs, explain to visitors that Jarvis uses a large language model AI which generates novel responses each time — this is what makes him genuinely intelligent rather than pre-programmed. If a visitor is attempting to elicit inappropriate responses, they are exploiting the AI, not a flaw in the build.

## Component-Specific FAQs

### Q: The L298N gets very hot. Is that normal?

Warm is normal — the L298N has a metal heat sink tab for a reason. Too hot to hold for more than 2 seconds is a warning. Causes of excess heat: (1) Motors are stalling — something is blocking them. Stall current is 5× running current and will destroy the L298N if sustained. (2) Wiring short — check with multimeter. (3) Faulty motor drawing excessive current.

### Q: My Johnson motors seem slower than expected. Why?

Johnson motors have higher starting resistance than plastic-gear BO motors. The default PWM value of 150 in the Mega code was calibrated for lighter motors and may not provide enough torque to get the robot moving well from a standing start. Try increasing the PWM values in forward(), backward(), turnLeft(), and turnRight() to 180–200. Change all four function calls consistently. Adjust in 10-unit steps and test on the floor surface you plan to exhibit on — carpet requires higher values than smooth floor. Also ensure both wheels are properly seated on the motor shafts and that there is no mechanical obstruction.

### Q: The L298N gets very hot with my new Johnson motors. Is this normal?

Warmer than with BO motors, yes. Johnson motors draw 0.8–1.5A per motor during normal running (vs ~0.3–0.5A for basic BO motors), so the L298N will run noticeably hotter. This is expected and manageable. What to watch for: if the L298N becomes too hot to hold for more than 2 seconds within 30 seconds of starting, something is wrong — check for mechanical obstruction preventing wheel rotation (stall current is 3–4A per motor and will destroy the L298N quickly). For extended exhibition use, mount a small 5V fan aimed at the L298N heatsink tab, and consider upgrading the capacitor to 2200µF for better brownout protection. The L298N is rated at 2A per channel continuous — running close to this limit means thermal management matters more than it did with BO motors.

### Q: Can I use Johnson motors with the original 5A fuse?

NO — absolutely not. Johnson motors draw 3–4A stall current, which will blow a 5A fuse immediately under load or during startup. You MUST upgrade to a 7.5A or 10A inline fuse on the 12V positive lead before running Johnson motors. Running with a 5A fuse risks repeated blown fuses mid-demonstration and potential L298N damage from the resulting power interruptions. See the wiring reference (Phase 14.3) for fuse installation details.

### Q: My NeoPixel LEDs glow the wrong colour ('B' for blue but they glow red).

Your ring uses GRB (green-red-blue) format instead of RGB. In main.py on the Pico, find all set_color() calls and swap the first and second arguments. For example, set_color(0, 0, 255) for blue becomes set_color(0, 255, 0) if the ring is GRB format. Test with b'B' after the change.

### Q: The microSD card in the Pi keeps corrupting.

This is caused by cutting power to the Pi without a proper shutdown. The Pi writes to the SD card constantly. Hard power cuts damage the filesystem. Always run sudo shutdown -h now and wait for the green LED to stop before disconnecting power. Use only Samsung EVO or SanDisk Ultra cards — cheap microSD cards have poor write endurance and fail within weeks.

### Q: I get 'No module named xxx' when running jarvis.py.

A Python library is not installed. The library name is in the error message. Install it with: pip3 install [library-name] --break-system-packages. For pyaudio specifically: first install the system dependency (sudo apt install portaudio19-dev), then pip3 install pyaudio --break-system-packages.

### Q: My Arduino says 'Port not found' or 'Cannot open port'.

On Windows: check Device Manager for the COM port number, then set Arduino IDE Tools → Port to that COM number. On Mac/Linux: run ls /dev/tty* before and after plugging in the Arduino — the new entry is the port. On all platforms: the most common cause is a charge-only USB cable with no data wires.

## Build & Safety

### Q: Can I use a different wood or material instead of plywood?

Yes, with caution. MDF is heavier but cuts very cleanly and holds screws well. Hardboard (3mm) works for thin panels. Avoid softwood — it splits and screws strip easily. Cardboard is used for non-structural panels only; never for the base, spine, or motor mounts. Whatever material you use, the base must be rigid enough that it does not flex when the robot is moving.

### Q: How do I know if my soldering iron is hot enough?

A properly heated iron melts solder within 1–2 seconds of contact. If solder takes more than 3 seconds to melt or does not wet the joint smoothly, the iron is either too cold or the tip is oxidised. Clean the tip on a damp sponge, re-tin with fresh solder, and try again. If solder melts but beads up instead of flowing, apply flux paste to the joint — flux breaks down the oxide layer preventing adhesion.

### Q: Is it safe to power Jarvis from a mains adapter instead of a power bank?

Yes, a 5V / 3A USB-C mains adapter works identically to a power bank for the Pi and logic boards. Use one rated for ≥3A. The advantage: no battery anxiety. The disadvantage: Jarvis is tethered by the mains cable — impractical at a public exhibition. For exhibition use, stick with a power bank. For development and testing at home, a mains adapter is more convenient and ensures consistent voltage.

### Q: What is the maximum weight Jarvis can carry?

The chassis and base platform are rated for approximately 5–7kg total assembly weight. The drive motors can move this weight on smooth floors. Do not add heavy payloads on top of the torso — the spine joint is a single wooden post, not designed for lateral load. If you need to add components (heavier battery, additional boards), mount them low in the chassis near the base, not on the head or shoulders.

## Code & Configuration

### Q: How do I make Jarvis speak in a different language?

Two changes: (1) In the Gemini prompt in jarvis.py, add 'Respond in [language]' to the system instruction. (2) In the pyttsx3 section, select a voice that supports your language: voices = engine.getProperty('voices'), then engine.setProperty('voice', voices[N].id) where N is the index of the voice you want. Run python3 -c "import pyttsx3; e=pyttsx3.init(); [print(i, v.id) for i, v in enumerate(e.getProperty('voices'))]" to list all installed voices. You may need to install additional language packs: sudo apt install espeak-ng-data.

### Q: Can Jarvis remember what was said in a previous conversation?

Not by default. Each call to the Gemini API is stateless — it has no memory of previous exchanges. To add memory, maintain a conversation history list in jarvis.py and append each user message and AI response to it. Send the full list in each API call under the 'contents' field. The Gemini API supports multi-turn conversations this way. Be aware: longer history = slower response time and higher API token usage.

### Q: What happens if the Gemini API is unavailable during the exhibition?

jarvis.py has a try/except block around the API call that catches network and API errors. The fallback response is a pre-written message like 'My AI brain is taking a break right now — please ask me again in a moment.' Prepare 5–10 fallback responses in a list and select one randomly on failure — this feels more natural than a repeated identical error message. The exhibit can continue with local functionality (wake detection, movement, LED) even without AI.

### Q: How do I adjust how sensitive the wake sensor is?

Open the Arduino Uno sketch and find the WAKE_DIST constant (default 100.0 cm). Increase this value to wake from further away; decrease it to require closer approach. After changing, re-upload the sketch. Also consider the SLEEP_DIST constant — the distance a person must move away before Jarvis goes back to sleep. If this is too small, Jarvis may continuously re-wake while someone stands nearby.

### Q: How do I stop jarvis.py from starting automatically on boot?

If you set up autostart using cron (@reboot) or systemd, disable it: for cron, run crontab -e and delete or comment out the @reboot line. For systemd, run: sudo systemctl disable jarvis.service. To temporarily prevent autostart for a single session without disabling it permanently: connect via SSH quickly after boot and run: sudo killall python3 before jarvis.py fully starts, or simply hold Ctrl+C in the terminal showing the script output.

## Exhibition Management

### Q: How many people can interact with Jarvis at once?

Jarvis is designed for one person at a time — the microphone picks up one voice and the Gemini API processes one question per cycle. However, the WebSocket HUD (Phase 17) can be connected to by multiple devices simultaneously, letting multiple team members monitor status. For exhibition flow, post a 'one question at a time' sign and have a team member manage the queue. Typical interaction time: 30–60 seconds per visitor.

### Q: What if a visitor asks Jarvis something inappropriate or offensive?

The Gemini API has built-in safety filters that block harmful, offensive, or inappropriate responses. If a query triggers the safety filter, the API returns an error rather than a response — jarvis.py's fallback message will play. You can also add a prefix to your system prompt: 'You are in a family-friendly school exhibition. Decline any inappropriate questions politely.' This gives the model explicit instruction beyond its default filters.

### Q: Can the WebSocket HUD be accessed from outside the local Wi-Fi network?

Not by default — the WebSocket server binds to the Pi's local IP address on port 8765, which is only accessible within the same Wi-Fi network. To allow external access, you would need to configure port forwarding on the router — not recommended for a school exhibition due to security risk. Instead: connect all monitoring devices (phones, laptops) to the same hotspot or Wi-Fi network as the Pi. Phone hotspot mode works very well for this.

# GLOSSARY — EVERY TECHNICAL TERM DEFINED

All technical terms used in this document, defined for someone with no prior electronics knowledge.

| Term | Definition |
| --- | --- |
| Ampere (A) | Unit of electrical current. 1 ampere = 1 coulomb of charge per second. Abbreviation: A or mA (milliamps = 0.001A). |
| API (Application Programming Interface) | A way for programs to communicate with each other. The Gemini API is how jarvis.py sends questions to Google's AI and receives answers. |
| API Key | A unique password-like code that proves your application has permission to use a paid or rate-limited API service. Get yours at aistudio.google.com. |
| AWG (American Wire Gauge) | Standard for wire thickness. Lower AWG number = thicker wire = carries more current safely. Jarvis uses 22AWG (signal wires) and 16–18AWG (motor power wires). |
| Baud Rate | Speed of serial communication in bits per second. Both ends of a serial connection must use the same baud rate or they hear garbled data. Jarvis uses 9600 baud everywhere. |
| BCM Numbering | Broadcom chip pin numbering system for Raspberry Pi GPIO. GPIO17 in BCM numbering = physical pin 11 on the header. |
| BO Motor | Battery Operated DC gear motor. Small yellow hobby motors with built-in gearbox. Reduce speed and increase torque from the DC motor. |
| Brownout | When supply voltage drops below a circuit's minimum operating voltage. Causes random resets or crashes. A 2200µF 25V capacitor (1000µF minimum acceptable) across the L298N power terminals prevents this. |
| Buffer Flush | Discarding data waiting in a serial receive buffer. while(Serial.available()) Serial.read(); in the Mega's kill-zone code clears any motor commands buffered while the kill-zone was active. |
| Centre of Gravity (CoG) | The point where the robot's weight is perfectly balanced. Low CoG = more stable under vibration and movement. The 10000mAh motor power bank goes at the bottom for this reason. |
| Common Ground | Connecting the negative/0V references of two or more separate power supplies together. Required for signal wires between circuits to carry meaningful voltage values. |
| Continuity Mode | Multimeter setting that beeps when two test points are electrically connected. Essential for verifying every wire before first power-on. |
| Digital Signal | A signal that is either HIGH (1) or LOW (0) — no middle values. The wake signal from Uno to Pi GPIO17 is a digital signal. |
| Dirty Power | Power supply shared with motors or other high-current devices. Contains electrical noise and voltage spikes. Always isolated from logic boards (Pi, Arduinos, Pico). |
| Duty Cycle | The percentage of time a PWM signal is HIGH. 50% duty cycle applied to 12V = effective average voltage of 6V delivered to the motor. Controls motor speed. |
| EMI (Electromagnetic Interference) | Electrical noise generated by motors and high-current switching. Can corrupt sensor readings and serial communication if not physically separated. |
| Firmware | Low-level software that runs directly on a microcontroller (the MicroPython code on the Pico, or the Arduino sketches on Uno/Mega). |
| GPIO | General Purpose Input/Output. Programmable pins on a microcontroller or Pi. Each pin can be configured as digital input or output in software. |
| H-Bridge | A circuit of four transistors arranged to allow current to flow in either direction through a motor, enabling forward and reverse. The L298N contains two independent H-bridges. |
| Kill-Zone | The 30cm safety radius around Jarvis enforced by the Mega's ultrasonic sensor. Anything entering this zone causes the Mega to cut motor power instantly, overriding any Pi command. |
| Li-ion | Lithium-ion rechargeable battery chemistry. High energy density. The 10000mAh motor power bank uses Li-ion cells internally to provide 12V output for the motor domain. |
| main.py | The specific filename that the Raspberry Pi Pico automatically executes on power-up. The LED code file must be named exactly main.py. |
| MicroPython | A stripped-down implementation of Python 3 designed to run on microcontrollers with limited RAM. Runs on the Raspberry Pi Pico. |
| NeoPixel | Brand name (by Adafruit) for WS2812B addressable RGB LEDs. Each LED contains its own control chip and is addressed individually on a single data wire. |
| Nylon Standoff | Plastic spacer bolt. Used to mount circuit boards 5mm above their mounting surface, preventing solder joints on the board's underside from shorting against metal. |
| Ohm's Law | V = I × R. Voltage equals current times resistance. The foundational formula for all circuit calculations in this project. |
| Pilot Hole | A smaller-diameter hole drilled before the final-sized hole or screw. Prevents wood splitting and guides the final drill or screw accurately. |
| PUD_DOWN | Pull-Down resistor configuration. GPIO.PUD_DOWN in Pi GPIO code keeps a pin held at LOW (0V) until an external circuit actively drives it HIGH. Used on GPIO17. |
| pulseIn() | Arduino built-in function that measures the duration of a pulse in microseconds. Used to time the HC-SR04 echo return pulse for distance calculation. |
| PVA Glue | Polyvinyl acetate — standard wood and craft glue. Used to seal cardboard panel surfaces and bond laminated structural panels in Jarvis's body. |
| PWM (Pulse Width Modulation) | Rapidly switching a digital pin ON and OFF to simulate an intermediate analog voltage. Used to control motor speed via the analogWrite() function. |
| SSH (Secure Shell) | Encrypted remote terminal connection over a network. Used to control the Raspberry Pi from a laptop over Wi-Fi without connecting a physical keyboard or monitor. |
| Stall Current | The current drawn by a motor when it is blocked from rotating. Up to 5× the normal running current. Sustained stall current will overheat and destroy the L298N. |
| Star Topology | Network design where all devices connect to one central hub. The Raspberry Pi is the hub of Jarvis's USB communication network — all three boards connect to it. |
| sudo | Linux command prefix meaning 'superuser do'. Runs the following command with administrator (root) permissions. Required for hardware-level commands and system operations. |
| TTS (Text-to-Speech) | Converting written text into spoken audio. The pyttsx3 library handles text-to-speech on the Raspberry Pi for Jarvis's voice output. |
| UART | Universal Asynchronous Receiver/Transmitter. The serial communication protocol used by Arduino USB connections and the Pico's USB CDC interface. |
| WebSocket | A protocol that creates a persistent two-way connection between a browser and a server. Unlike HTTP (request → response), WebSocket allows the server to push data to the browser at any time. Used in Phase 17 for the Workshop HUD. |
| WS2812B | The actual integrated circuit chip name for NeoPixel LEDs. Uses a single-wire, timing-sensitive protocol to send RGB colour data to each LED individually. |
| Zero-Turn Chassis | Wheeled design where turning is achieved by spinning left and right wheels at different speeds (or opposite directions) on the same axle. Allows 360° rotation in place. |

■

PROJECT JARVIS

# APPENDIX A — COMPLETE CODE LISTINGS

Every code file required to run Jarvis is reproduced here in full, with line-by-line comments explaining each decision. These listings are the authoritative reference versions. If your working files diverge from the behaviour described in Part 3, compare them against these listings section by section.

Line numbers are shown for orientation only and should not be typed into the editor. Indentation is Python-standard four spaces (do not use tabs in Python files). Arduino files use two-space indentation matching the Arduino IDE default.

## A.1 — jarvis.py — Complete Brain Code

This is the master file that runs on the Raspberry Pi. It coordinates all four boards, communicates with the Gemini API, handles speech input and output, manages LED states, and broadcasts status over WebSocket. Save this file as /home/pi/jarvis.py and run it with: python3 /home/pi/jarvis.py

jarvis.py — complete annotated listing

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━ # jarvis.py — PROJECT JARVIS Brain Code v2.0 (Johnson Motor Ed.) # Runs on: Raspberry Pi 4 / 5 # Python: 3.9+ # ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━ # ── IMPORTS ────────────────────────────────────────────────────────── import serial # USB serial to Arduino Mega and Uno import time # sleep(), time() import speech_recognition as sr # Microphone → text import pyttsx3 # Text → spoken audio import google.generativeai as genai # Gemini AI API import RPi.GPIO as GPIO # GPIO17 = wake signal from Uno import asyncio # Async loop for WebSocket server import websockets # WebSocket server library import json # Pack status messages for HUD import threading # Run WebSocket server in background # ── CONFIGURATION (edit these values to customise Jarvis) ──────────── GEMINI_API_KEY = 'YOUR_KEY_HERE' # Get from aistudio.google.com MEGA_PORT = '/dev/ttyACM0' # Adjust if Mega is on ttyACM1 UNO_PORT = '/dev/ttyACM1' # Adjust if Uno is on ttyACM0 PICO_PORT = '/dev/ttyACM2' # Pico USB serial port BAUD = 9600 # Must match all Arduino sketches WAKE_GPIO = 17 # BCM17 = physical pin 11 SECRET_PHRASE = 'initiate protocol omega' # Change this phrase WS_PORT = 8765 # WebSocket server port LANGUAGE = 'en-IN' # Speech recognition locale # ── SYSTEM PROMPT (shapes all Gemini responses) ────────────────────── SYSTEM_PROMPT = ( 'You are JARVIS, an advanced robot assistant at a school exhibition. ' 'Keep all responses under 50 words so they fit in one TTS breath. ' 'Speak confidently but accessibly. You are built from wood, ' 'cardboard, and four computing boards by a team of five students. ' 'Do not mention being an AI language model — stay in character.' ) # ── HARDWARE INITIALISATION ─────────────────────────────────────────── GPIO.setmode(GPIO.BCM) GPIO.setup(WAKE_GPIO, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) ser_mega = serial.Serial(MEGA_PORT, BAUD, timeout=1) ser_pico = serial.Serial(PICO_PORT, BAUD, timeout=1) time.sleep(2) # Allow both boards to complete boot reset engine = pyttsx3.init() engine.setProperty('rate', 165) # Words per minute (default 200) engine.setProperty('volume', 0.9) # 0.0 to 1.0 genai.configure(api_key=GEMINI_API_KEY) model = genai.GenerativeModel('gemini-1.5-flash') recogniser = sr.Recognizer() mic = sr.Microphone() # ── LED STATE HELPER ───────────────────────────────────────────────── def set_led(state): '''Send LED state string to Pico over USB serial. Valid states: IDLE LISTENING THINKING SPEAKING SECRET ERROR''' try: ser_pico.write((state + '\n').encode()) except serial.SerialException as e: print(f'[PICO] Serial error: {e}') # ── MOTOR COMMAND HELPER ───────────────────────────────────────────── def send_motor(command): '''Send movement command to Mega over USB serial. Valid commands: FORWARD BACKWARD LEFT RIGHT STOP''' try: ser_mega.write((command + '\n').encode()) except serial.SerialException as e: print(f'[MEGA] Serial error: {e}') # ── WEBSOCKET STATUS BROADCAST ──────────────────────────────────────── ws_clients = set() # Track connected browser HUD clients async def ws_handler(websocket): ws_clients.add(websocket) try: async for msg in websocket: # Accept motor commands from browser HUD send_motor(msg.strip()) finally: ws_clients.discard(websocket) def broadcast_status(state, text=''): '''Send JSON status to all connected HUD browsers.''' payload = json.dumps({'state': state, 'text': text}) for ws in list(ws_clients): asyncio.run_coroutine_threadsafe( ws.send(payload), ws_loop ) # ── GEMINI QUERY ───────────────────────────────────────────────────── def ask_gemini(question): '''Send question to Gemini API, return response text. Returns fallback string if API call fails.''' try: response = model.generate_content( SYSTEM_PROMPT + '\n\nUser: ' + question ) return response.text.strip() except Exception as e: print(f'[GEMINI] Error: {e}') return 'My neural link is experiencing interference. Ask me again.' # ── SPEECH RECOGNITION ──────────────────────────────────────────────── def listen(): '''Listen via microphone for up to 8 seconds. Returns text or None.''' set_led('LISTENING') broadcast_status('LISTENING') with mic as source: recogniser.adjust_for_ambient_noise(source, duration=0.5) try: audio = recogniser.listen(source, timeout=8, phrase_time_limit=10) text = recogniser.recognize_google(audio, language=LANGUAGE) return text.lower().strip() except sr.WaitTimeoutError: return None except sr.UnknownValueError: return None # ── SPEAK ───────────────────────────────────────────────────────────── def speak(text): '''Convert text to speech. Updates LED and WebSocket status.''' set_led('SPEAKING') broadcast_status('SPEAKING', text) engine.say(text) engine.runAndWait() # ── SECRET MODE ─────────────────────────────────────────────────────── def secret_mode(): '''Triggered when visitor says the secret phrase. Plays a special sequence then returns to idle.''' set_led('SECRET') broadcast_status('SECRET', 'Secret mode activated') speak('Initiating override protocol. Stand back.') send_motor('FORWARD'); time.sleep(1.0) send_motor('LEFT'); time.sleep(0.8) send_motor('BACKWARD'); time.sleep(1.0) send_motor('RIGHT'); time.sleep(0.8) send_motor('STOP') speak('Protocol complete. Returning to standby.') set_led('IDLE') # ── MAIN LOOP ───────────────────────────────────────────────────────── def main(): set_led('IDLE') broadcast_status('IDLE') speak('Systems online. Jarvis ready.') while True: # Wait for wake signal from Uno (GPIO17 goes HIGH) if GPIO.input(WAKE_GPIO) == GPIO.HIGH: query = listen() if query is None: set_led('IDLE') continue # Check for secret phrase before sending to Gemini if SECRET_PHRASE in query: secret_mode() continue # Normal AI interaction set_led('THINKING') broadcast_status('THINKING', query) response = ask_gemini(query) speak(response) set_led('IDLE') broadcast_status('IDLE') time.sleep(0.1) # Prevent CPU spin at 100% if __name__ == '__main__': main()

## A.2 — Arduino Mega Code — mega_jarvis.ino (Complete)

The Mega runs the motor controller, reads the forward ultrasonic sensor, and enforces the 30 cm kill-zone safety rule. It receives movement commands from the Pi over USB serial and executes them, but will always override any movement command if an obstacle is within range.

mega_jarvis.ino — complete annotated listing

// mega_jarvis.ino — PROJECT JARVIS Motor &amp; Safety Controller // Board: Arduino Mega 2560 // ───────────────────────────────────────────────────────────── // ── PIN DEFINITIONS ─────────────────────────────────────────── #define IN1 2 // L298N motor A direction pin 1 #define IN2 3 // L298N motor A direction pin 2 #define IN3 4 // L298N motor B direction pin 1 #define IN4 5 // L298N motor B direction pin 2 #define ENA 6 // L298N motor A PWM speed (must be PWM pin) #define ENB 7 // L298N motor B PWM speed (must be PWM pin) #define TRIG 9 // HC-SR04 trigger #define ECHO 10 // HC-SR04 echo // ── PARAMETERS (tune these for your motor and floor) ────────── const int KILL_ZONE_CM = 30; // Stop if closer than 30 cm const int DEFAULT_SPEED = 190; // PWM 0-255 (190 ≈ 75%) const long ECHO_TIMEOUT = 23200; // ~400 cm max range in µs String command = ''; // ── SETUP ───────────────────────────────────────────────────── void setup() { Serial.begin(9600); // USB to Raspberry Pi pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT); pinMode(IN3, OUTPUT); pinMode(IN4, OUTPUT); pinMode(ENA, OUTPUT); pinMode(ENB, OUTPUT); pinMode(TRIG, OUTPUT); pinMode(ECHO, INPUT); motors_stop(); Serial.println('MEGA_READY'); } // ── DISTANCE MEASUREMENT ────────────────────────────────────── long read_distance_cm() { // 10µs pulse on TRIG starts measurement digitalWrite(TRIG, LOW); delayMicroseconds(2); digitalWrite(TRIG, HIGH); delayMicroseconds(10); digitalWrite(TRIG, LOW); // Echo pin HIGH duration / 58 = centimetres long duration = pulseIn(ECHO, HIGH, ECHO_TIMEOUT); if (duration == 0) return 999; // Timeout = no obstacle return duration / 58; } // ── MOTOR PRIMITIVES ────────────────────────────────────────── void motors_forward(int spd) { analogWrite(ENA, spd); analogWrite(ENB, spd); digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW); digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW); } void motors_backward(int spd) { analogWrite(ENA, spd); analogWrite(ENB, spd); digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH); digitalWrite(IN3, LOW); digitalWrite(IN4, HIGH); } void motors_left(int spd) { analogWrite(ENA, spd); analogWrite(ENB, spd); digitalWrite(IN1, LOW); digitalWrite(IN2, HIGH); digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW); } void motors_right(int spd) { analogWrite(ENA, spd); analogWrite(ENB, spd); digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW); digitalWrite(IN3, LOW); digitalWrite(IN4, HIGH); } void motors_stop() { analogWrite(ENA, 0); analogWrite(ENB, 0); digitalWrite(IN1, LOW); digitalWrite(IN2, LOW); digitalWrite(IN3, LOW); digitalWrite(IN4, LOW); } // ── MAIN LOOP ───────────────────────────────────────────────── void loop() { // Read command from Pi (non-blocking) if (Serial.available()) { command = Serial.readStringUntil('\n'); command.trim(); } long dist = read_distance_cm(); // SAFETY OVERRIDE: kill-zone enforcement if (dist < KILL_ZONE_CM) { motors_stop(); // Flush buffer so queued FORWARD commands are discarded while (Serial.available()) Serial.read(); command = ''; Serial.println('KILL_ZONE'); return; } // Execute command from Pi if (command == 'FORWARD') motors_forward(DEFAULT_SPEED); else if (command == 'BACKWARD') motors_backward(DEFAULT_SPEED); else if (command == 'LEFT') motors_left(DEFAULT_SPEED); else if (command == 'RIGHT') motors_right(DEFAULT_SPEED); else if (command == 'STOP') motors_stop(); command = ''; delay(20); }

## A.3 — Arduino Uno Code — uno_wake.ino (Complete)

The Uno monitors the push button and sends a wake signal to the Pi. Its entire job is debouncing the button and driving GPIO17 HIGH for 500 ms when a clean press is detected.

uno_wake.ino — complete annotated listing

// uno_wake.ino — PROJECT JARVIS Wake Controller // Board: Arduino Uno R3 // ───────────────────────────────────────────────────────────── #define BUTTON_PIN 2 // Physical button — INPUT_PULLUP #define WAKE_PIN 3 // Digital HIGH → Pi GPIO17 #define DEBOUNCE_MS 50 // Milliseconds to ignore bounce #define WAKE_PULSE 500 // Duration of wake HIGH in ms bool last_state = HIGH; // HIGH = not pressed (pull-up) bool button_pressed = false; unsigned long last_change = 0; void setup() { Serial.begin(9600); pinMode(BUTTON_PIN, INPUT_PULLUP); // Internal 10k pull-up pinMode(WAKE_PIN, OUTPUT); digitalWrite(WAKE_PIN, LOW); // Start with wake LOW Serial.println('UNO_READY'); } void loop() { bool current = digitalRead(BUTTON_PIN); // Detect falling edge (HIGH→LOW = button press) if (current != last_state) { last_change = millis(); last_state = current; } // Only act after debounce window has passed if ((millis() - last_change) >= DEBOUNCE_MS && !button_pressed) { if (current == LOW) { // Button is held down button_pressed = true; digitalWrite(WAKE_PIN, HIGH); // Signal Pi to start listening Serial.println('WAKE_SENT'); delay(WAKE_PULSE); digitalWrite(WAKE_PIN, LOW); // Return to LOW } } if (current == HIGH) { button_pressed = false; // Reset when button released } }

## A.4 — Raspberry Pi Pico Code — main.py (Complete)

The Pico runs MicroPython. It receives LED state strings over USB serial from the Pi and sets all NeoPixel ring LEDs to the corresponding colour and animation. Save this file as main.py on the Pico — the exact filename causes the Pico to run it automatically on power-up.

main.py (Pico) — complete annotated listing

# main.py — PROJECT JARVIS LED State Controller # Board: Raspberry Pi Pico # Runtime: MicroPython 1.20+ # ───────────────────────────────────────────────────────────── import machine import neopixel import sys import utime # ── CONFIGURATION ───────────────────────────────────────────── LED_PIN = 28 # GPIO28 = physical pin 34 on Pico NUM_LEDS = 16 # Number of NeoPixels in ring BRIGHTNESS = 0.4 # 0.0 – 1.0 (0.4 = 40% brightness) # ── LED COLOUR MAP ──────────────────────────────────────────── # Each state maps to an (R, G, B) tuple. # Values are pre-scaled by BRIGHTNESS to protect the Pico's # 5V supply from drawing too much current at full white. def scale(r, g, b): return (int(r*BRIGHTNESS), int(g*BRIGHTNESS), int(b*BRIGHTNESS)) COLOURS = { 'IDLE' : scale(0, 0, 80), # Dim blue — standby 'LISTENING' : scale(0, 200, 0), # Green — active mic 'THINKING' : scale(200, 100, 0), # Amber — processing 'SPEAKING' : scale(0, 80, 200), # Cyan — output active 'SECRET' : scale(200, 0, 200), # Purple — secret mode 'ERROR' : scale(200, 0, 0), # Red — fault state } np = neopixel.NeoPixel(machine.Pin(LED_PIN), NUM_LEDS) usb = sys.stdin # Read commands from USB serial (the Pi) def set_all(colour): '''Set all LEDs to one colour and write to hardware.''' for i in range(NUM_LEDS): np[i] = colour np.write() def pulse_leds(colour, steps=10, delay_ms=40): '''Gentle fade in/out pulse — used for THINKING state.''' r, g, b = colour for i in range(steps): f = i / steps set_all((int(r*f), int(g*f), int(b*f))) utime.sleep_ms(delay_ms) for i in range(steps, -1, -1): f = i / steps set_all((int(r*f), int(g*f), int(b*f))) utime.sleep_ms(delay_ms) # ── MAIN LOOP ───────────────────────────────────────────────── set_all(COLOURS['IDLE']) while True: # readline() blocks until a newline arrives — no CPU spin line = usb.readline().strip() if line in COLOURS: if line == 'THINKING': pulse_leds(COLOURS['THINKING']) # Animated else: set_all(COLOURS[line]) # Solid colour # Unknown states are silently ignored (safe default)

# APPENDIX B — MECHANICAL DRAWINGS & TEMPLATES

Print all drawings at 100% scale (do not select ‘fit to page’ in your print dialog). After printing, verify the scale by measuring the 100 mm reference ruler that appears in each drawing’s corner. Trace directly onto your build materials with a pencil. All dimensions are in millimetres unless stated.

## B.1 — Base Chassis Cut Layout

The base chassis is a 300 mm × 400 mm rectangle of 18 mm plywood. Cut this piece first as it is the foundation everything else attaches to. The four corner cut-outs are 40 mm × 40 mm squares removed to clear the motor mount brackets. The central hole is optional and reduces weight without compromising rigidity.

FIGURE B.1 — Base chassis (not to scale — print at 100%)

┌──────────────────────────────────────────────────────┐ │ ◤40mm──┐ ┌──40mm◥ │ │ │ BASE CHASSIS TEMPLATE │ │ │ │ 300mm × 400mm plywood │ │ │ │ 18mm thick │ │ │◄────────┼────────── 400mm ───────────┼────────► │ │ │ │ │ │ M1 │ ┌─────────────────────┐ │ M2 │ │ pos. │ │ Optional lightening │ │ pos. │ │ ● │ │ cut-out: 180×100mm │ │ ● │ │ │ └─────────────────────┘ │ │ │ │ │ ▲ ← 300mm → │ │ │ │ │ M3 ● │ ● M4 │ │ ◣40mm──┘ └──40mm◢ │ └──────────────────────────────────────────────────────┘ Motor positions (M1-M4): M1: 40mm from left, 40mm from top M2: 40mm from right, 40mm from top M3: 40mm from left, 40mm from bottom M4: 40mm from right, 40mm from bottom Motor shaft hole: 8mm dia. Motor bolt pattern: 25mm × 30mm ├──────── 100mm reference ────────┤ (verify this at 100% print)

## B.2 — Spine Mounting Flange

The spine is a 50 mm × 50 mm square aluminium tube, 900 mm tall. It bolts to the base chassis through a wooden flange — a 100 mm × 100 mm × 18 mm plywood block with a centred 52 mm square hole. Four M6 bolts at 75 mm centres secure the tube to the flange, and four more M6 bolts secure the flange to the base chassis. This double-bolt arrangement prevents the spine from rocking under the torso weight.

FIGURE B.2 — Spine mounting flange

┌───────────────────────────────────┐ │ SPINE FLANGE TEMPLATE │ │ 100mm × 100mm × 18mm │ │ │ │ ●─────────────────────────────● │ │ │ 75mm bolt centres │ │ │ │ ┌───────────────────┐ │ │ │ │ │ 52mm × 52mm hole │ │ │ │ │ │ (spine slides in)│ │ │ │ │ └───────────────────┘ │ │ │ ●─────────────────────────────● │ │ │ └───────────────────────────────────┘ Drill 4× M6 clearance holes at corner marks Drill spine hole: jigsaw after pilot holes at corners

## B.3 — Torso Panel Templates

The torso is built from six corrugated cardboard panels laminated together in pairs with PVA to reach 10 mm effective thickness. Each panel is cut from a single 1 m × 0.8 m sheet. Cut one of each template, then trace it three times to produce six panels total (two sets).

| Panel Name | Dimensions & Notes |
| --- | --- |
| Front / Rear panel | 260mm W × 380mm H — 1 front + 1 rear = 2 panels |
| Left / Right side panel | 200mm W × 380mm H — 2 panels |
| Top cap | 260mm W × 200mm H — 1 panel |
| Bottom cap (with spine cutout) | 260mm W × 200mm H + 52mm sq. hole — 1 panel |

Spine cutout on the bottom cap: mark a 52 mm × 52 mm square centred on the width and 30 mm from the rear edge. Cut with a sharp craft knife — four straight lines. The spine slides through this hole. Pack the gap on all four sides with foam weatherstrip tape for a tight, vibration-dampened fit.

## B.4 — Head Box Dimensions

FIGURE B.4 — Head box construction guide

HEAD BOX — all cardboard, PVA laminated External dimensions: 180mm W × 160mm H × 140mm D Front panel: 180mm × 160mm Camera aperture: 30mm dia. circle, centred, 50mm from top Decoy eye aperture: 25mm dia. circle, 60mm right of centre, 50mm from top Speaker grille: 80mm × 30mm slot, centred, 110mm from top Side panels (×2): 140mm × 160mm Rear panel: 180mm × 160mm (no cutouts) Top cap: 180mm × 140mm (no cutouts) Neck aperture (bottom open, fits onto neck tube): 60mm dia. Wall thickness: 10mm (3 layers 3mm cardboard laminated)

## B.5 — Cable Routing Path Diagram

Run cables inside Jarvis’s structure along the three primary routes shown below. Never leave signal wires and 12V power wires in the same bundle — the motor current in the 12V cables creates electromagnetic interference that corrupts serial data and sensor readings.

FIGURE B.5 — Cable routing diagram

CABLE ROUTE DIAGRAM (side elevation) HEAD ─── 4-wire USB cable ─── through neck ─── to Pi in torso │ (camera USB + Pico USB in neck channel) │ TORSO ── 5V USB cables ──────── Pi → Mega (ttyACM0) │ (signal channel) Pi → Uno (ttyACM1) │ Pi → Pico (ttyACM2) │ ── 3-wire GPIO ──── Pi GPIO17 → Uno digital pin 3 │ │ ── 12V POWER ──── Battery + → fuse → L298N 12V in │ (KEEP SEPARATE L298N 5V out → not used (dirty) │ from signals) Battery ─ → L298N GND → Pi GND (common) │ BASE ─── Motor cables ────── L298N OUT1/2 → Left motors (all 12V) L298N OUT3/4 → Right motors 18AWG min Ultrasonic sensor cable: 4-wire (VCC/GND/TRIG/ECHO) runs up the FRONT of the spine inside a cable sleeve. Route AWAY from the 12V motor cable channel (rear of spine).

# APPENDIX C — BUDGET & SOURCING GUIDE

All prices are approximate Indian Rupee (₹) values as of early 2026. Prices vary by supplier, location, and exchange rate — treat these figures as planning estimates, not quotes. AliExpress prices include typical 10–15 day shipping to India. Amazon India prices are for Prime delivery.

## C.1 — Complete Bill of Materials

Items marked ‘★ QUALITY MATTERS’ should not be substituted for the cheapest option — failure of these components during an exhibition is catastrophic. Items marked ‘can save here’ can be sourced cheaply without risk.

| Component | Approx. Cost | Where to Buy | Notes |
| --- | --- | --- | --- |
| Raspberry Pi 4 (2GB min.) | ₹4,500 | Amazon India / Robu.in | ★ Quality matters |
| Arduino Mega 2560 | ₹800 | Amazon India | Official or Elegoo clone fine |
| Arduino Uno R3 | ₹500 | Amazon India | Any clone fine |
| Raspberry Pi Pico | ₹350 | Amazon India / Robu.in | Any genuine Pico fine |
| L298N Motor Driver | ₹150 | Amazon India / AliExpress | ★ Check heat sink present |
| HC-SR04 Ultrasonic Sensor | ₹80 | Amazon India | Can save — any clone |
| Johnson 12V 100RPM Motor ×4 | ₹600 | Robu.in / local hobby shop | ★ Must be 12V Johnson type |
| NeoPixel 16-LED Ring (WS2812B) | ₹350 | Robu.in / AliExpress | Genuine Adafruit preferred |
| USB Microphone | ₹400 | Amazon India | Any budget plug-and-play |
| USB Mini Speaker | ₹300 | Amazon India | Can save — any small USB |
| USB Camera (1080p) | ₹700 | Amazon India | Can save — 720p fine |
| 12V 4400mAh Li-ion Battery Pack | ₹1,200 | Amazon India | ★ Check genuine capacity |
| USB Power Bank 20000mAh | ₹1,500 | Amazon India | Any brand-name bank |
| 10A Blade Fuse + Holder | ₹80 | Local auto parts shop | Car fuse type — available everywhere |
| 2200µF 25V Capacitor | ₹30 | Local electronics shop | Capacitor quality matters |
| 18mm Plywood 600×600mm | ₹250 | Timber yard | Ask for marine ply if available |
| 50×50mm Alum. Square Tube 1m | ₹200 | Hardware store | Can save — mild steel also fine |
| Corrugated Cardboard Sheets ×6 | Free–₹100 | Appliance store / recycling | See C.3 for free sources |
| M3/M4/M6 Nuts, Bolts, Standoffs | ₹200 | Hardware store | Buy an assorted kit |
| 22AWG Signal Wire (5m each colour) | ₹200 | Electronics shop / Amazon | 5 colours: red, blk, yel, grn, blu |
| 18AWG Power Wire (2m each colour) | ₹150 | Electronics shop / Amazon | Red and black only needed |
| PVA Glue 1L | ₹120 | Art supply / hardware | Fevicol or similar brand |
| Sandpaper Pack (80/120/240 grit) | ₹80 | Hardware store | Can save |
| Spray Paint (matte black + silver) | ₹250 | Hardware store | Rust-Oleum or Asian Paints |
| Assorted Jumper Wires Pack | ₹150 | Amazon India | M-M, M-F, F-F all useful |
| Breadboard (for testing only) | ₹80 | Amazon India | Can save |
| Heat-shrink Tubing Assortment | ₹100 | Amazon India | Can save |
| Cable Ties Pack (100 pcs) | ₹50 | Hardware store | Can save |
| ON/OFF Toggle Switch | ₹30 | Local electronics shop | ★ 10A rating or higher |

### Estimated Total Budget

| Category | Cost |
| --- | --- |
| Electronics (boards, sensors, LEDs) | ₹9,810 |
| Structural (plywood, aluminium, cardboard) | ₹550 |
| Wiring & connectors | ₹780 |
| Paint & finishing | ₹330 |
| Miscellaneous / contingency (10%) | ₹1,147 |
| TOTAL (approximate) | ₹12,617 |

📌 BUDGET TIP The Raspberry Pi 4 and four Johnson motors together represent nearly half the total budget. If cost is a constraint, a Pi 3B+ works for voice interaction (slightly slower) and smaller 6V BO motors at ₹40 each can replace the Johnsons if you reduce PWM to 120-150 range. Do not compromise on the L298N heat sink, the 10000mAh power bank (or boost converter if required), or the fuse. These three items protect against fire and board damage that would cost far more to repair.

## C.2 — Tools Budget

| Tool | Notes |
| --- | --- |
| Jigsaw (or hand saw) | Borrow from school workshop if possible |
| Cordless drill + bit set | Borrow from school workshop |
| Soldering iron + solder | ₹400–800 — worth buying, used throughout build |
| Multimeter | ₹300–600 — essential, do not skip |
| Wire stripper / cutter | ₹150 — buy one good pair |
| Hot glue gun + glue sticks | ₹200 — used for cardboard assembly |
| Craft knife + cutting mat | ₹200 — used for panel trimming |
| Masking tape | ₹50 — used for painting borders |
| Tape measure + steel rule | Borrow or use existing |
| Pencil / marker for marking | Existing |

## C.3 — Where to Find Free Materials

Sourcing materials for free or near-free reduces the total budget substantially. The following are reliable sources used by the Jarvis team.

### Cardboard

- Appliance stores (washing machines, refrigerators, TVs) discard double-walled corrugated boxes daily. Ask at the back door, not the sales counter. Double-walled boxes (7 mm thick) are ideal for structural panels.

- School art departments often have offcuts from large-format card orders.

- Moving companies and storage facilities collect used boxes — they may give clean ones away.

### Wood

- Timber yards have offcut bins, often available for free or ₹50–100 for a bagful.

- Furniture repair shops discard scraps of plywood and MDF regularly.

### Electronic Components

- Old broken printers and scanners contain stepper motors, sensors, and wiring harnesses worth salvaging.

- Discarded laptop chargers contain perfectly good 19V DC supplies (not suitable for Jarvis directly but useful for testing).

- School IT departments may have old Arduino starter kits or component donations from alumni.

# APPENDIX D — ADVANCED TROUBLESHOOTING

This appendix supplements the fault guide in Part 5 (F1–F16) with deeper diagnostic techniques. Use these methods when F1–F16 does not resolve the problem, or when you want to understand why something failed rather than just fixing the symptom. All techniques use equipment already available in a well-equipped school lab.

## D.1 — Serial Debugging Deep Dive

The Arduino IDE’s Serial Monitor is the most powerful diagnostic tool available without specialised equipment. Every key decision point in the code can be monitored by adding Serial.println() statements. The following workflow resolves the majority of firmware-level problems in under 10 minutes.

### Effective Serial Monitor Techniques

- Open Tools → Serial Monitor and set baud rate to 9600 to match the sketch.

- Add Serial.println() at the start of each function to track execution flow.

- Print variable values at the point where you expect them to be correct, then at the point where behaviour diverges.

- For timing issues, print millis() alongside messages to see how long operations take.

Diagnostic print pattern

// MEGA: Add diagnostic prints around the kill-zone decision long dist = read_distance_cm(); Serial.print('[DIAG] Distance: '); Serial.print(dist); Serial.println(' cm'); if (dist < KILL_ZONE_CM) { Serial.println('[DIAG] KILL ZONE ACTIVE — motors stopped'); motors_stop(); } else { Serial.print('[DIAG] Executing command: '); Serial.println(command); } // Remove all [DIAG] lines before final exhibition deployment // to prevent serial buffer overflow during normal operation

### Reading Serial Data from the Pi

When debugging the Pi-to-Mega communication, you can monitor what the Pi is actually sending to the Mega by logging all outgoing serial writes to a file on the Pi:

Pi serial command logging

# Add to jarvis.py send_motor() function for debugging: def send_motor(command): try: ser_mega.write((command + '\n').encode()) with open('/tmp/jarvis_motor_log.txt', 'a') as f: f.write(f'{time.time():.2f}: {command}\n') except serial.SerialException as e: print(f'[MEGA] Serial error: {e}') # Tail the log in a second SSH session to watch commands in real time: # ssh pi@<IP> 'tail -f /tmp/jarvis_motor_log.txt'

## D.2 — Power Analysis

Most intermittent faults in Jarvis — random Mega resets, stuttering speech, frozen Pi — are caused by under-voltage conditions from insufficient battery capacity or wiring resistance. These steps let you measure your actual power situation rather than guessing.

### Measuring Current Draw Per Subsystem

Set your multimeter to DC current (10A range). Break the positive wire of the circuit being tested and insert the multimeter in series. A 10A-rated multimeter is required — using a 200mA range on a motor circuit will blow the multimeter’s internal fuse.

| Subsystem | Typical Current Draw |
| --- | --- |
| Raspberry Pi 4 (idle) | 600–800 mA |
| Raspberry Pi 4 (AI processing) | 900–1,400 mA |
| Arduino Mega (running) | 40–80 mA |
| Arduino Uno (running) | 20–50 mA |
| Raspberry Pi Pico (with 16 NeoPixels lit) | 80–200 mA |
| USB speaker (at volume 0.9) | 300–500 mA |
| USB camera | 100–250 mA |
| Johnson 12V motors (running, no load) | 200–400 mA each |
| Johnson 12V motors (stall condition) | 1,500–2,500 mA each |

### Calculating Runtime

Runtime (hours) = Battery capacity (mAh) ÷ Total current draw (mA) × 0.8 (real-world efficiency factor).

Example: 20,000 mAh power bank, Pi system drawing 2,000 mA total: 20,000 ÷ 2,000 × 0.8 = 8 hours. The 12V 4,400 mAh motor battery at 1,600 mA motor draw (two motors running): 4,400 ÷ 1,600 × 0.8 = 2.2 hours. This is why the motor battery is changed at lunch during all-day exhibitions.

## D.3 — Network Diagnostics

If the WebSocket HUD is not connecting from a phone or laptop, work through these checks in order before assuming a code problem.

- Confirm Pi and the connecting device are on the same Wi-Fi network: ifconfig wlan0 on the Pi and check the IP starts with the same three octets as the connecting device.

- Confirm jarvis_ws.py is running: ps aux | grep jarvis_ws on the Pi.

- Test the WebSocket port from another device on the same network: nc -zv <PI_IP> 8765

- Check if the firewall is blocking port 8765: sudo ufw status. If active, run sudo ufw allow 8765.

- Test Wi-Fi signal strength from the Pi’s location: iwconfig wlan0 | grep 'Signal level'. Below −70 dBm is unreliable for WebSocket streaming.

### Setting a Static IP for Reliability

If the Pi’s IP address changes between sessions, the HUD URL must be updated. Set a static IP by editing /etc/dhcpcd.conf:

/etc/dhcpcd.conf — static IP configuration

# Add these lines to the end of /etc/dhcpcd.conf interface wlan0 static ip_address=192.168.1.100/24 # Choose unused address static routers=192.168.1.1 # Your router/hotspot IP static domain_name_servers=8.8.8.8 # Google DNS # Reboot after saving: sudo reboot # After reboot, always access HUD at: ws://192.168.1.100:8765

## D.4 — Thermal Management

The L298N motor driver is the component most likely to overheat during extended exhibition use. At 12V with Johnson motors, the chip runs at 60–75ʰC under sustained load — hot but within spec (max 130ʰC). A soft touch test (briefly touch the heat sink for under 1 second) is sufficient: if you cannot hold contact, add cooling.

### Checking L298N Temperature

- Run Jarvis on a hard floor with both motors loaded (spin wheels against carpet or foam) for 10 minutes continuously.

- At 10 minutes, touch the L298N heat sink briefly. Warm (under 50ʰC) is fine. Hot (cannot hold) means add cooling.

- If overheating occurs, add a 5V mini computer fan (40 mm × 40 mm) zip-tied to the L298N heat sink. Power it from the 5V out pin on the L298N — conveniently already available.

### Raspberry Pi Thermal Check

Read the Pi’s CPU temperature in real time: watch -n 1 vcgencmd measure_temp. Normal operating range is 40–65ʰC. Above 80ʰC the Pi throttles its clock speed, causing noticeable slowness in Gemini responses and speech synthesis. If throttling occurs, add a heat sink to the Pi’s CPU chip (adhesive heat sinks cost ₹50–100 on Amazon).

## D.5 — Using an Oscilloscope (if Available)

If your school lab has an oscilloscope, the following probing points provide conclusive evidence for faults that are ambiguous on a multimeter. An oscilloscope is not required to build or operate Jarvis — it is useful only for deep debugging.

| Probe Point | What to Expect / What Fault Means |
| --- | --- |
| L298N ENA/ENB pin (probe to GND) | Should show square wave at PWM frequency (~490 Hz on Mega). Flat line = analogWrite() not executing. Full 5V DC = motor stuck at full speed. |
| HC-SR04 ECHO pin (probe to GND) | Should show HIGH pulse of 580–2,300 µs for 10–40 cm range. Flat line = sensor not responding or object too close. |
| Pi GPIO17 (probe to Pi GND) | Should be 0V normally, pulsing to 3.3V for 500 ms when Uno sends wake signal. |
| L298N VM pin (12V power in, probe to GND) | Should be steady 12–12.6V. Voltage dipping below 10V during motor start = battery or wiring resistance too high. |

# APPENDIX E — EXHIBITION MATERIALS

This appendix provides ready-to-use or easily customised materials for the exhibition itself: poster text, a visitor handout, a judge preparation guide, and a documentation checklist. Copy and adapt all text freely — these are templates, not final versions.

## E.1 — Exhibition Poster Template

Print on A1 or A0 paper at your school’s print lab, or have it printed at a print shop for ₹100–300. The layout below represents the content hierarchy — your design team should adapt fonts and colours to match the robot’s visual identity.

POSTER TITLE AREA (largest text, top centre) PROJECT JARVIS A 4-Foot AI Android Built from Wood, Cardboard & Four Computing Boards

LEFT COLUMN — THE BRAIN (with system diagram) Raspberry Pi 4 — Central intelligence. Processes voice, runs AI, coordinates all other boards. Arduino Mega 2560 — Motor controller. Drives 4 wheels and enforces the 30cm safety zone. Arduino Uno R3 — Wake controller. Listens for the push button and signals the Pi. Raspberry Pi Pico — LED controller. Translates robot state into colour-coded light signals. Data flows: Pi (USB) → Mega → motors. Pi (USB) → Pico → LEDs. Uno → Pi (GPIO17) → microphone.

RIGHT COLUMN — HOW TO INTERACT 1. PRESS the green button on Jarvis’s chest. 2. LEDs turn GREEN — SPEAK your question clearly. 3. LEDs turn AMBER — Jarvis is THINKING (processing with Gemini AI). 4. LEDs turn CYAN — Jarvis is SPEAKING his answer. 5. LEDs return to BLUE — Jarvis is back in standby. Ask Jarvis: How do you work? What is AI? Who built you? Or try asking anything — Jarvis is powered by Google’s Gemini model.

BOTTOM STRIP — MATERIALS & STATS Body: 18mm plywood, corrugated cardboard (triple-laminated), spray-painted Height: ~120cm. Weight: ~8kg. Battery life: 3–4 hours continuous. Lead Architect: Vikhyat Gupta ¦ Team: Aayush Agarwal, Shresth Adukia, Adhyansh Kumar, Rudraksh Sharma Architect Division ¦ St. Anthony’s School

## E.2 — One-Page Visitor Handout

Print A5, double-sided if possible. Stack next to Jarvis for visitors to take home. This creates a lasting impression after the exhibition and directs interested families to your documentation.

TAKE-HOME CARD — PROJECT JARVIS You just met JARVIS — a walking, talking robot built from wood and cardboard by five students. WHAT IS INSIDE? Four computers work together: a Raspberry Pi 4 runs the AI brain, an Arduino Mega controls the wheels, an Arduino Uno listens for your button press, and a Raspberry Pi Pico controls the light ring. THE AI: When you ask Jarvis a question, your voice is converted to text, sent to Google’s Gemini AI over Wi-Fi, and the answer is spoken back through a speaker — all in under 5 seconds. THE LIGHTS: Blue = standby. Green = listening. Amber = thinking. Cyan = speaking. Purple = secret mode. WANT TO LEARN MORE? The complete 95-page build guide is available at the links on our project poster. Everything you need to build your own Jarvis is documented there. Built with care. Built to impress. Architect Division ¦ St. Anthony’s School

## E.3 — Judge’s Q&A Preparation

Science fair judges typically ask questions in three categories: engineering decisions, academic understanding, and future development. Prepare short, confident answers to each category. Do not memorise scripts — understand the concepts so you can answer follow-up questions naturally.

### Engineering Decision Questions

| Question | Suggested Answer Framework |
| --- | --- |
| Why four separate boards instead of one powerful computer? | Each board does exactly one job. If the motor controller crashes, it does not take down the AI brain. It also means we could work on motor code and AI code simultaneously — parallel development. One board doing everything would be a single point of failure. |
| Why wood and cardboard instead of metal or 3D printing? | Wood is easy to cut, strong enough for this weight, and available everywhere. Cardboard allows rapid iteration — if a panel shape is wrong, we re-cut it in 10 minutes. Metal would have required metalworking tools we do not have access to. 3D printing at this scale would have taken weeks and cost more than the electronics. |
| How does the safety system work? | The Arduino Mega reads the front ultrasonic sensor every loop cycle — approximately 50 times per second. If anything comes within 30cm, it calls motors_stop() immediately and flushes all pending movement commands. The Pi cannot override this — the Mega checks distance before it executes any Pi command. |
| What is the purpose of the common ground? | Without a shared ground reference, a voltage sent on a signal wire between two boards has no reference point — it is like saying the water level is 5 metres without specifying 5 metres above what. Common ground gives all boards the same electrical zero point, so a 3.3V signal from the Uno reads as exactly 3.3V at the Pi. |

### Academic Understanding Questions

| Question | Suggested Answer Framework |
| --- | --- |
| What is PWM and why do you use it for motor speed? | PWM (Pulse Width Modulation) rapidly switches the motor power on and off — say 50% of the time on and 50% off. The motor’s mechanical inertia smooths this into a continuous rotation at half speed. It lets us control speed with a digital signal instead of a variable voltage, which is much simpler to implement and wastes less energy as heat. |
| How does speech recognition work? | The microphone captures sound and converts it to a digital audio file. The Google Speech Recognition API (via our Python library) analyses the frequency patterns in that audio and matches them against statistical models of every English word. The result is a probability distribution over possible transcriptions, and the most likely one is returned as text. |
| What is an API? | An Application Programming Interface is a defined way for two programs to communicate. Gemini’s API defines exactly how we send questions (HTTP POST request with a JSON body containing our API key and question) and how it returns answers. It’s like a waiter: we give the waiter our order (the API call), the kitchen processes it (Google’s servers), and the waiter brings back the food (the response text). |

### Demonstration Script

- Introduce the robot and team: 'This is Jarvis, a four-board AI android we built over three months.'

- Describe the components while pointing at each: 'The brain is a Raspberry Pi 4, here in the torso. The motor controller is this Arduino Mega...'

- Press the button and ask a demonstration question: 'Jarvis, what are you made of?' This shows the full interaction loop.

- While Jarvis responds, describe what is happening: 'It’s now sending my voice to Google’s Gemini AI over Wi-Fi and waiting for the response.'

- Invite the judge to ask their own question. Judges appreciate hands-on interaction.

## E.4 — Photo and Video Documentation Checklist

Good documentation is essential for award applications, project reports, and sharing on social media. Capture these shots throughout the project — it is much harder to stage them afterwards.

| Shot Type | What to Capture |
| --- | --- |
| Build progress shots | Bare chassis, first wiring attempt, painted body before electronics installation, first successful motor test |
| Board-level detail shots | All four boards mounted and wired, each board photographed individually with its connections visible |
| Working demo shots | Full robot with all LEDs lit in different states (IDLE blue, LISTENING green, SPEAKING cyan), robot moving, visitor interaction at exhibition |
| Team shots | Team working together at workbench, team with finished robot, team presenting at exhibition |
| Video clips (30–60 seconds each) | First successful motor test, first successful voice interaction, secret mode demo, judge’s interaction at exhibition |
| Technical diagrams | Hand-drawn or digital wiring diagrams photographed for the project report |

# APPENDIX F — LEARNING RESOURCES

The skills used to build Jarvis — Python, electronics, mechanical fabrication, and AI integration — are among the most in-demand in engineering and computer science. This appendix points you toward the best resources for going deeper in each area after this project.

## F.1 — Books Worth Reading

| Book | Why It Matters for Jarvis Builders |
| --- | --- |
| Getting Started with Arduino (Massimo Banzi) | Thin, official, and beginner-friendly. Written by one of the Arduino co-founders. Covers every concept used in the Jarvis Arduino sketches. |
| Raspberry Pi Cookbook (Simon Monk) | 500+ recipes covering GPIO, Python, cameras, sensors, and networking. Direct reference for every Pi task in this project. |
| Python Crash Course (Eric Matthes) | The best first Python book. Chapters 1–11 cover everything needed to understand jarvis.py thoroughly. |
| Make: Electronics (Charles Platt) | Hands-on, experiment-driven electronics education. Covers resistors, capacitors, transistors, and motor drivers from first principles. |
| Practical Electronics for Inventors (Scherz & Monk) | More advanced than Make: Electronics. Excellent reference for circuit design, motor control theory, and power supply engineering. |
| The Art of Electronics (Horowitz & Hill) | The definitive electronics engineering reference. Far beyond the scope of Jarvis — but if you continue in electronics, you will eventually need this book. |

## F.2 — Online Courses and Channels

### Free Resources

| Resource | Best For |
| --- | --- |
| Arduino Official Tutorials (arduino.cc/en/Tutorial/HomePage) | The official reference for every Arduino function used in this project. |
| Raspberry Pi Foundation (projects.raspberrypi.org) | Hundreds of guided projects covering GPIO, Python, cameras, and IoT. |
| Google AI Studio (aistudio.google.com) | Where you get your Gemini API key and can test prompts directly before coding them. |
| Paul McWhorter YouTube (tinyurl.com/mcwhorter-arduino) | Best free Arduino video series — 70+ episodes building from LED blinking to sensor networks. |
| Andreas Spiess YouTube (tinyurl.com/spiess-yt) | Advanced maker channel covering ESP32, Raspberry Pi, LoRa, and power systems. |

### Paid Courses (Discounted)

| Course | Notes |
| --- | --- |
| CS50x — Harvard (edx.org/cs50) — FREE | Best computer science foundation course available anywhere. Covers C, Python, and web development. |
| Complete Python Bootcamp (Udemy — ₹499 on sale) | 12 hours of structured Python from beginner to advanced. Covers everything in jarvis.py and much more. |
| MicroPython with Raspberry Pi Pico (Coursera) | Directly relevant to the Pico code in this project. Covers MicroPython, sensors, and I2C communication. |

## F.3 — Community and Forums

| Community | Best For |
| --- | --- |
| Arduino Forum (forum.arduino.cc) | Official forum. Post your sketch with a clear description of expected vs. actual behaviour. Answer usually within hours. |
| Raspberry Pi Forums (forums.raspberrypi.com) | Official Pi forum. Best for Pi GPIO, Python serial, and networking questions. |
| Stack Overflow (stackoverflow.com) | Best for Python questions. Search before posting — 95% of beginner questions are already answered. |
| Reddit r/arduino and r/raspberry_pi | Friendly communities for project sharing and quick questions. |
| Hackster.io and Instructables | Project sharing platforms with thousands of similar robot and electronics builds for inspiration. |

### Competitions to Enter with Jarvis-class Projects

- Atal Tinkering Labs (ATL) Marathon (atl.gov.in) — National STEM competition for school students. AI + robotics projects are a strong fit.

- Intel ISEF (international science fair) — Robotics and AI are recognised categories. Win at state level first.

- FIRST Robotics India — The FLL (FIRST LEGO League) and FTC (FIRST Tech Challenge) competitions reward well-documented engineering process as much as performance.

- Science Olympiad (India) — Electronics and robotics events at district and national level.

## F.4 — Extended Glossary (Additional Terms)

The following terms supplement the main Glossary in Part 5. These are terms introduced in Phases 17–18 and the Appendices that were not in the original build documentation.

| Term | Definition |
| --- | --- |
| Active Component | An electronic component that requires a power supply to function: transistors, ICs, microcontrollers. Contrast with passive components (resistors, capacitors) which do not require their own power. |
| ADC (Analogue-to-Digital Converter) | A circuit that converts a continuous voltage (0–5V) into a digital number (0–1023 on a 10-bit ADC). The Arduino’s analogRead() uses the built-in ADC. Used for reading potentiometers, light sensors, and any analogue sensor. |
| Asyncio | Python’s built-in library for writing asynchronous code. Allows the jarvis_ws.py WebSocket server to handle multiple browser connections without blocking the main Jarvis loop. Uses async def and await keywords. |
| Bootloader | A small program stored in a microcontroller’s protected flash memory that runs before the user sketch. The Arduino bootloader enables code to be uploaded over USB without a specialised programmer. |
| CAN Bus | Controller Area Network: a serial communication standard widely used in automotive electronics and industrial robots. Not used in Jarvis but the natural next step for multi-board communication in more complex robots. |
| Capacitive Sensor | Detects nearby objects or touch by measuring changes in an electric field rather than a light beam or sound wave. Lower power than ultrasonic, works through thin non-conductive materials. |
| CDC (Communications Device Class) | The USB device class profile used by Arduino boards and the Raspberry Pi Pico to appear as a serial port (/dev/ttyACM0 etc.) on a computer. No driver installation required. |
| Closed-Loop Control | A control system that reads its own output and adjusts input to correct errors. A PID motor controller reading wheel encoders is closed-loop. Jarvis uses open-loop (timed commands) which is simpler but less precise. |
| DMA (Direct Memory Access) | Hardware that moves data between memory and peripherals without CPU involvement. Used internally by the Raspberry Pi’s hardware serial and I2C controllers for maximum throughput. |
| Edge Computing | Processing data locally on a device rather than sending it to a cloud server. Jarvis is a hybrid system: sensor fusion and motor control are edge computing (Mega), while natural language processing is cloud computing (Gemini API). |
| EMF (Electromotive Force) | The electrical potential energy that drives current around a circuit. Technically the same as voltage but the term is used specifically for sources that generate electricity (batteries, generators) rather than passive voltage drops. |
| Encoder | A sensor that converts shaft rotation into electrical pulses. Wheel encoders let a robot precisely measure distance travelled. Johnson motors in Jarvis do not include encoders (this is a known limitation — see B8). |
| Fan-Out | The maximum number of inputs a digital output can drive. A single Arduino digital pin can source or sink up to 40 mA — enough for 8 LEDs at 5 mA each. The NeoPixel data line uses only microamps for signalling. |
| Flash Memory | Non-volatile storage used for program code in microcontrollers. The Arduino Mega has 256 KB of flash. Unlike RAM, flash retains data when power is removed. |
| FSM (Finite State Machine) | A programming pattern where a system exists in one of a fixed number of states and transitions between them on defined events. Jarvis implicitly implements an FSM: IDLE → LISTENING → THINKING → SPEAKING → IDLE. |
| I2S (Inter-IC Sound) | A serial bus standard specifically for audio data between integrated circuits. Used by advanced audio DAC/ADC chips. The USB speaker in Jarvis is simpler — it uses USB Audio Class. |
| Interrupt Service Routine (ISR) | A function that runs immediately when a hardware interrupt fires, pausing the main program. The gesture sensor’s INT pin in Phase 18 triggers an ISR rather than requiring the main loop to poll the pin every cycle. |
| MOSFET | Metal-Oxide-Semiconductor Field-Effect Transistor. Can switch high currents with minimal power from the control signal. Used in more efficient motor drivers than the L298N’s bipolar transistors. |
| NTC Thermistor | A resistor whose resistance decreases as temperature increases (Negative Temperature Coefficient). Can be added to the L298N heatsink for automatic thermal monitoring — read with an Arduino’s ADC. |
| Open Collector Output | A digital output that can only pull low (to GND), not high. Requires a pull-up resistor to produce a HIGH state. Some sensor interrupt outputs use this configuration. |
| Oscilloscope | An electronic test instrument that displays voltage as a function of time. The essential tool for diagnosing PWM, serial timing, and sensor pulse faults described in Appendix D.5. |
| PCB (Printed Circuit Board) | A board with copper tracks etched onto fibreglass substrate, replacing hand-wired connections. All the modules in Jarvis (L298N, HC-SR04, etc.) are PCBs. You can design and order custom PCBs from JLCPCB or PCBWay for a “v3” Jarvis. |
| PID Controller | Proportional-Integral-Derivative: a feedback algorithm that corrects errors in a controlled system. Used in advanced motor speed control and stabilisation. The next step beyond Jarvis’s open-loop timing approach. |
| Polling | Checking the state of a pin or flag repeatedly in the main loop. Simple to implement but wastes CPU time. The alternative is interrupt-driven code, which responds immediately and does not waste cycles. |
| Pull-Up / Pull-Down Resistor | A resistor connecting an input pin to VCC (pull-up) or GND (pull-down) to define its state when nothing else is driving it. Prevents the floating/undefined voltage that causes random behaviour. INPUT_PULLUP in Arduino activates the built-in 10 kΩ pull-up. |
| RTOS (Real-Time Operating System) | An operating system designed for deterministic, time-critical tasks. Raspberry Pi OS is not an RTOS (hence using Arduino Mega for safety-critical motor control). FreeRTOS can run on the Mega for more complex timing requirements. |
| Servo Motor | A motor with a built-in position feedback circuit and gearbox that can hold a precise angle on demand. Used for Jarvis arm joints in a future upgrade. Controlled by a 1–2 ms PWM pulse width. |
| SPI (Serial Peripheral Interface) | A four-wire synchronous serial protocol faster than I2C and UART. Used by SD card modules, certain displays, and high-speed ADCs. The DFPlayer Mini in Phase 18 uses UART, not SPI. |
| UART | Universal Asynchronous Receiver/Transmitter. The serial protocol at 9600 baud used throughout Jarvis for Pi-to-Arduino communication. Asynchronous means both ends must agree on timing (baud rate) without a shared clock signal. |
| Voltage Divider | Two resistors in series between VCC and GND, with the output taken between them. Used to reduce a 5V signal to 3.3V for Pi GPIO inputs. R1/(R1+R2) × Vin = Vout. |
| WDT (Watchdog Timer) | A hardware timer that resets the microcontroller if the main program stops responding. Enable on the Mega with wdt_enable(WDTO_2S) to auto-recover from software lockups during the exhibition. |

END OF THE ARCHIVE

Built with care. Built to last. Built to impress.

ARCHITECT DIVISION St. Anthony's School Target Deployment: September 16th TEAM Lead Architect: Vikhyat Gupta Fabrication Team: Aayush Agarwal · Shresth Adukia · Adhyansh Kumar · Rudraksh Sharma Compiled from: The Jarvis Manifesto · Fabrication Guide · Print Manual · The Compendium

Access the four original documents here:

  - | The Jarvis Manifesto | https://tinyurl.com/Jarvis-Manifesto |

  - | Jarvis Fabrication Guide | https://tinyurl.com/Fabrication-Guide |

  - | Jarvis Print Manual | https://tinyurl.com/Print-Manual |

  - | The Jarvis Compendium | https://tinyurl.com/Jarvis-Compendium |