PROJECT JARVIS

THE JARVIS COMPENDIUM

The Complete Reference Encyclopedia for Building, Coding,

Wiring, Troubleshooting & Exhibiting Project Jarvis

Architect Division  ·  St. Anthony's School  ·  Target: September 16th

Lead Architect: Vikhyat Gupta

Fabrication Team: Aayush Agarwal · Shresth Adukia · Adhyansh Kumar · Rudraksh Sharma

Anyone holding this book has everything needed to build, wire, code & exhibit Jarvis.

# SECTION A — FOUNDATIONS: Electricity Explained Simply

Who needs this? Everyone. You cannot wire a robot without understanding these basics.

## A1 — What Are Voltage, Current & Resistance?

Question: Why does 12V hurt motors but 3.3V doesn't? What is the difference between volts and amps?

### The Water Pipe Analogy (The Best Way to Understand Electricity)

Voltage (V) = Water Pressure. Think of a tall water tower. The higher the tower, the more pressure is pushing water through the pipe. 5 volts = a small water tower. 12 volts = a much taller one. Voltage is the 'push' that drives electricity.

Current (A / Amps) = Flow Rate. How much water is actually flowing per second. A thin pipe can only carry a trickle even with huge pressure. Current is how many electrons actually move.

Resistance (Ω / Ohms) = Pipe Narrowness. A resistor is a narrow section of pipe that limits flow. It converts electrical energy into heat.

### Ohm's Law — The Single Most Useful Formula in Electronics

V = I × R    (Voltage = Current × Resistance)

Rearranged: I = V ÷ R and R = V ÷ I

Jarvis Example: Decoy eye LED needs 20mA (0.02A) from Pi's 3.3V pin. Pi pin = 3.3V, LED needs ~1.8V, so resistor must drop 1.5V. R = 1.5V ÷ 0.02A = 75Ω. We use 330Ω to be safe (less current = less heat, LED still glows).

⬛ WHY THIS MATTERS Every component has a voltage and current limit. Exceed the voltage = instant death. Exceed the current = slow death (heat damage). Ohm's Law lets you calculate exactly what resistor to use to protect any component. The Pi's GPIO pins can only deliver 3.3V at max 16mA per pin. That's enough to light an LED but NOT to drive a motor — that's why we need the L298N motor driver.

| Component | Voltage | Current Draw | Power Source |
| --- | --- | --- | --- |
| Raspberry Pi 3B+ | 5V | ~700mA typical | USB Power Bank |
| Arduino Uno | 5V | ~50mA + sensor | USB Power Bank (via Pi) |
| Arduino Mega | 5V | ~100mA + motors | USB Power Bank (via Pi) |
| Pi Pico | 5V (VBUS) | ~30mA + LEDs | USB Power Bank (via Pi) |
| BO Motors (each) | 6–12V | 200mA running, 1A stall | 12V Li-ion Battery |
| NeoPixel ring (12) | 5V | Up to 720mA full white | Pico VBUS |
| HC-SR04 Sensor | 5V | 15mA | Arduino 5V pin |
| L298N Driver | 5–35V (VIN) | 36mA logic + motor load | 12V Battery |

## A2 — Common Ground: The Single Most Misunderstood Concept

Question: Why do the 5V system and 12V system need their GND wires connected?

The short answer: Voltage is not a fixed thing — it's a difference. 5V means '5 volts higher than GND'. If the Pi and Mega don't share the same GND wire, they're speaking different languages and the signal wires carry garbage.

### The Walkie-Talkie Analogy

Imagine two walkie-talkies. One is tuned to a reference of 0 Hz and transmits at 5 Hz above that. The other thinks 0 Hz is actually 3 Hz — so it hears your 5 Hz signal as 2 Hz. Complete confusion. Common ground sets both radios to the same '0'. Now a signal of 5V HIGH on the Uno's Pin 2 is recognized as HIGH by the Pi's GPIO 17.

In Jarvis: Two separate power supplies are used (12V battery and 5V power bank). They never share a positive (+) rail. But their GND (negative/0V) wires MUST be connected at one point. This one wire is the 'Common Ground'.

⬛ ⚠ DANGER Never connect 12V positive to any 5V component. It will destroy the chip instantly. Always connect GND of 12V battery to GND of 5V power system. Without this, signal wires carry floating voltages and the robot behaves randomly. Use a multimeter on continuity mode to verify GND connectivity before powering on.

⬛ ✓ QUICK TEST Set multimeter to continuity mode (beep symbol). Place one probe on Mega GND, other probe on L298N GND terminal. Hear beep = common ground connected. No beep = STOP and wire it first. Repeat for: Pi GND pin → Mega GND, Uno GND → Mega GND.

## A3 — Dirty vs Clean Power: Why 12V Never Touches the Pi

Question: Why does Jarvis use two separate power supplies instead of one?

DC motors are electrically 'dirty'. When they start, stop, or stall, they create massive voltage spikes and suck huge bursts of current. This 'electrical noise' travels backwards through the power wire and can crash any microcontroller sharing the same supply.

Clean Power (Brain Juice): The 20,000mAh USB power bank provides perfectly regulated 5V to all brain boards (Pi, Uno, Mega, Pico). No motors anywhere on this circuit.

Dirty Power (Muscle Juice): The 12V Li-ion battery goes ONLY to the L298N motor driver. The motor driver's sole job is to amplify the Mega's tiny 5V control signals into the large 12V/1A power that motors need.

⬛ DUAL POWER ARCHITECTURE 12V Li-ion Battery → L298N Motor Driver → BO Motors 20,000mAh USB Bank → Raspberry Pi → USB hub → Uno + Mega + Pico Common Ground wire connects: 12V battery (–) to Arduino Mega GND NEVER: 12V battery (+) to anything with a Pi, Uno, or Pico

⬛ WHY THIS MATTERS Real-world fact: The 1000µF capacitor mounted on the L298N exists to absorb the motor spikes BEFORE they can even reach the L298N's logic pins. It's an electrical shock absorber. If the 12V battery dies during operation, the 5V brain stays alive. This is why Jarvis can announce 'Lead Architect, my drive systems have lost power' — the Pi is still running on the power bank.

## A4 — Digital vs Analog Signals

Question: What is the difference between a digital pin and an analog pin? Why does PWM matter?

Digital Signals = Light Switch. Either ON (HIGH = 5V) or OFF (LOW = 0V). No middle ground. Jarvis uses digital signals for: motor direction pins (IN1–IN4 on L298N), wake-up signal from Uno to Pi GPIO 17, ultrasonic TRIG pins.

Analog Signals = Dimmer Switch. Can be any value between 0V and the supply voltage. Jarvis's HC-SR04 ECHO pin returns a pulse whose width (time) represents distance. That time measurement is analog in nature.

PWM (Pulse Width Modulation) = Fake Analog via Digital. The Mega's pins ENA (Pin 3) and ENB (Pin 8) are PWM pins. They turn on and off thousands of times per second. If they're ON for half the time, the motor gets half power. analogWrite(motorSpeedLeft, 150) means the pin is HIGH for 150/255 = 59% of each cycle → 59% motor speed.

| Pin Type | What It Does in Jarvis |
| --- | --- |
| Uno Pin 2 → Pi GPIO17 | Digital HIGH/LOW wake-up signal |
| Mega Pins 4-7 | Digital motor direction (IN1–IN4) |
| Mega Pins 3, 8 (PWM) | analogWrite() motor speed control |
| Mega Pin 9/10 | Digital TRIG/ECHO for ultrasonic |
| Pico GP28 | NeoPixel data signal (digital) |
| Pi GPIO 17 | Digital input: reads Uno's wake signal |

## A5 — Pull-Up and Pull-Down Resistors

Question: Why does the Pi code say GPIO.PUD_DOWN? What happens without it?

A GPIO pin that is not connected to anything is called a 'floating pin'. Its value bounces randomly between HIGH and LOW due to stray electrical noise in the air. This would make Jarvis spontaneously wake up with no one around.

PUD_DOWN (Pull-Down) = Default to LOW. A tiny internal resistor connects the pin to GND. This 'pulls' the pin firmly to 0V (LOW) unless an external signal actively drives it HIGH. In Jarvis: GPIO 17 sits at 0V until the Uno actively sends 5V through the wake wire. Clean, reliable.

Line in Pi Code: GPIO.setup(uno_wire, GPIO.IN, pull_up_down=GPIO.PUD_DOWN)  — this activates the internal pull-down resistor on GPIO 17.

⬛ ⚠ DANGER Never leave a GPIO pin floating (configured as INPUT with no pull-up/down). The random bouncing can trigger false wake-ups, or worse, a false HIGH on an output pin could damage the Pi.

## A6 — Electricity Myths That Kill Robots

| MYTH | TRUTH |
| --- | --- |
| 'If it works without soldering it's fine' | Jumper wire connections loosen under vibration. A motor starting causes whole-robot vibration. ALWAYS solder or use screw terminals for motor wiring. |
| '5V and 12V can share a GND wire' | TRUE — they MUST share GND. But never share positive rails. GND is safe to share; positive is deadly. |
| 'More voltage = more powerful robot' | More voltage = more speed AND heat AND risk. Exceeding rated voltage destroys components. 12V motors at 12V is correct. 5V logic at 12V is dead. |
| 'Hot glue holds electronics permanently' | Hot glue fails under motor vibration in minutes. All circuit boards must be on standoffs bolted to plywood mounts. |
| 'USB cables are all the same' | Data USB cables carry 5V and data. Charge-only cables carry 5V but NOT data — Arduino IDE won't see the board. Always use data-capable USB cables. |
| 'I don't need to verify wiring before power' | One wrong wire destroys a £/$15 chip in milliseconds. Always use multimeter continuity mode first. |
| 'Pi shutdown is like a phone shutdown' | Pi runs Linux. Pull power while writing to SD card = corrupted OS = total rebuild needed. Always sudo shutdown -h now. |

# SECTION B — MATERIALS DEEP DIVE: Every Component Explained

Who needs this? Builders and anyone who needs to source replacements or diagnose failures.

## B1 — Plywood: 6mm vs 12mm

What it is: Plywood is multiple thin wood layers glued together with grains alternating 90°. Stronger than solid wood of same thickness. Available at any hardware store.

| Specification | 12mm Plywood | 6mm Plywood |
| --- | --- | --- |
| Used for | Base chassis — must carry total robot weight (2–3kg) | Leg boxes, head box, bracing strips, mounting plates |
| Why this thickness | Rigid enough to bolt motors through without flexing | Lighter weight, easier to cut precise shapes |
| Cutting tool | Jigsaw (go slow) | Jigsaw or craft knife for straight lines |
| Testing if failing | Flex the panel — it should feel like a book, not a credit card | Check edges for delamination (layers peeling apart) |

Alternatives: MDF board of same thickness works but is heavier and absorbs moisture. Solid pine wood works but splinters more. Never use chipboard — it crumbles under screws.

⬛ ⚠ DANGER Never use plywood that shows delamination (layers separating). It will split under load, especially at screw points. Buy new.

## B2 — Double-Wall Corrugated Cardboard

What it is: Corrugated cardboard has a wavy middle layer (the 'flute') sandwiched between flat sheets. Double-wall = two flute layers. This gives excellent rigidity-to-weight ratio.

Reinforcement technique: Three layers laminated with PVA glue, each rotated 90° from the last. Same principle as plywood — alternating grain directions cancel out flex. One layer: floppy. Three alternating layers: near-rigid. Cure time: minimum 2 hours under heavy books.

What it does in Jarvis: All torso panels, arm panels. It is NOT used for structural load — plywood handles that. Cardboard handles geometry (shaping) and cosmetics.

⬛ 💡 PRO TIP Seal all cardboard surfaces with 2 coats of PVA (50:50 water dilution) before spray painting. Unsealed cardboard soaks up paint unevenly and looks rough. Always cut cardboard with a craft knife and metal ruler — scissors compress the flute and weaken the edge.

## B3 — PVC Pipe 25mm

What it is: Standard plumbing PVC pipe, 25mm outer diameter. Used as Jarvis's spine — the central vertical column that everything attaches to.

Why PVC? Lightweight. Strong in compression (vertical loads). Perfectly round so it sits in a drilled hole without gaps. Cheap and available everywhere. Cuts cleanly with a hacksaw.

Critical spec: Spine length = 90cm. Base sits 10cm above floor (motor wheel diameter). Top of spine = shoulder height. Total robot height ≈ 4 feet (122cm) with 20cm head.

How to test if failing: Grip the top of the spine and push sideways. Should have zero flex. If it rocks, the PVC flange plate at the base is loose — re-screw and add epoxy.

⬛ ✓ QUICK TEST With robot assembled, tape a spirit level to the spine. Check front-to-back: bubble must be centered. Rotate 90°. Check left-to-right: bubble must be centered. If off: loosen flange plate, correct angle, retighten, re-epoxy.

## B4 — BO Motors (Yellow Gear Motors)

What they are: Small DC gear motors. 'BO' = Battery Operated. Internal gearbox reduces RPM (speed) while multiplying torque (turning force). Standard yellow motors used in school/hobby robotics.

| Property | Value / Explanation |
| --- | --- |
| Operating voltage | 3–12V (we run at 12V for maximum speed) |
| Running current | ~200mA per motor at no load |
| Stall current | ~1A per motor (motor stopped, full voltage). This is the dangerous state. |
| Stall = danger | Motor stalls when blocked. Current jumps to 1A. L298N gets hot. Prolonged stall = L298N thermal shutdown or motor burnout. |
| Zero-turn ability | Two motors on same axle line — run one forward, one backward = spin on the spot. Jarvis uses this for 360° turns. |
| Signs of death | Motor runs but wheel doesn't spin (broken gear). Runs hot. Buzzes but doesn't turn. Smells burnt. |

How to test: Connect motor directly to 9V battery with two wires. Should spin freely and silently. Grind/squeal = dying gears. No movement = dead motor.

Alternatives: N20 micro gear motors (smaller, higher torque). TT gear motors (similar size, compatible). Any DC motor can work if PWM voltage range matches.

## B5 — Ball Casters and the 1–2mm Clearance Rule

What they are: Small ball-in-socket wheels that can roll in any direction. Used as 'stabilizer legs'. They don't drive — they only prevent the robot from tipping.

The 1–2mm clearance rule: Drive wheels must sit 1–2mm LOWER than the caster ball contact point. When the robot is level, all weight is on the drive wheels. Casters float just above the ground. If Jarvis starts to tip forward or backward, the casters catch it. This is the zero-wobble secret.

How to set clearance: With motors and wheels on a flat surface, measure height H from floor to underside of base. Caster ball center should be at H minus 1mm. If casters are too tall: shim motor mounts up with a 1mm strip of card. If casters are too low: add a washer under each caster bolt.

⬛ ✓ QUICK TEST Place robot on smooth floor. Slide a sheet of paper under each caster. Paper should slide under easily but with slight resistance. If paper slides freely: casters too high, casters won't catch tips. Adjust. If paper won't slide at all: casters too low, they're carrying weight. Adjust.

## B6 — L298N Motor Driver

What it is: A dual H-bridge motor driver. It takes small 5V logic signals from the Mega and uses them to switch much larger 12V/2A currents to the motors. Think of it as a relay controlled by the Arduino.

### Internal H-Bridge Explained

An H-bridge is four transistors arranged in an 'H' shape. By switching different combinations of transistors ON/OFF, current can flow through the motor in either direction = forward or backward movement. The L298N contains two H-bridges (for left and right motor).

| Terminal | What It Does |
| --- | --- |
| VIN / 12V | Input voltage from 12V battery. Powers the motors only. |
| 5V (logic power) | Can receive 5V from Arduino OR provide 5V out (see jumper) |
| GND | Must connect to: 12V battery (–) AND Arduino Mega GND |
| ENA / ENB | Enable pins. Connect to PWM pins (Mega 3 & 8). Controls speed via analogWrite() |
| IN1, IN2 | Direction control for Motor A (left). IN1 HIGH+IN2 LOW = forward. |
| IN3, IN4 | Direction control for Motor B (right). Same logic. |
| OUT1/OUT2 | Motor A output terminals — connect left motor wires |
| OUT3/OUT4 | Motor B output terminals — connect right motor wires |
| 1000µF Capacitor | Mount across VIN and GND. Absorbs motor startup voltage spikes. |

⬛ ⚠ DANGER The 5V enable jumper: if ENA/ENB jumpers are installed, motors run at FULL SPEED always, ignoring PWM. REMOVE both ENA and ENB jumpers before connecting PWM wires. L298N gets hot during operation. This is normal. If too hot to touch for more than 2 seconds, check for stalled motor or wiring short. If red LED on L298N goes off, it has triggered thermal protection. Let it cool 5 minutes.

## B7 — HC-SR04 Ultrasonic Sensor

What it is: Sends a high-frequency sound pulse (40kHz — too high to hear) and measures how long it takes to bounce back from an object. Time → distance.

| Property | Value |
| --- | --- |
| Range | 2cm to 400cm (reliable: 5cm to 200cm) |
| Accuracy | ±3mm |
| Operating voltage | 5V DC |
| Current | 15mA |
| TRIG pin | Send a 10µs HIGH pulse to fire one sound burst |
| ECHO pin | Goes HIGH when burst sent, LOW when echo received. Width = travel time. |
| Distance formula | distance = (pulseIn time / 2) / 29.1 cm |
| Timeout value | 30,000 µs = max wait time. At 343 m/s, 30,000µs = max ~5m range. Returns 0 if nothing detected. |

### Why the pulseIn Timeout Matters

Without a timeout, pulseIn() blocks forever if no echo returns (pointing at open space, or the sound beam misses everything). This freezes the entire Arduino loop. The 30,000µs timeout ensures the loop runs every ~30ms even with nothing in front of the sensor.

Ghost echoes: If the sensor is pointed parallel to the floor, it picks up echoes from the floor tiles. Always tilt the sensor 5–10° upward. Also: add the check (dist > 0) to ignore timeout returns.

⬛ ✓ QUICK TEST In Arduino Serial Monitor at 9600 baud, hold your hand at known distances (10cm, 50cm, 100cm). Reported values should be within ±3cm of actual distance. If reading is always 0 or 999: check 5V and GND connections, then TRIG and ECHO pins.

## B8 — NeoPixel LED Ring (12 LEDs)

What it is: WS2812B addressable LEDs. Each LED has its own tiny driver chip. A single data wire carries color commands for all 12 LEDs in a serial chain.

| Property | Value |
| --- | --- |
| Protocol | Single-wire serial, 800kHz. Timing-critical — no delays allowed during transmission. |
| Power | 5V. Max current: 60mA per LED × 12 = 720mA at full white. In normal operation: ~200mA. |
| Data wire | GP28 on Pico (any GPIO works, we use 28). Must be 3.3V logic — Pi Pico is 3.3V = compatible. |
| Color format | RGB (Red, Green, Blue) values 0–255 each. (255,200,0) = Gold. (0,0,255) = Blue. |
| Without diffuser | You see 12 bright dots, looks cheap. With diffuser film: smooth glowing disc. |

Jarvis color states:

| LED Color | Meaning | Triggered by |
| --- | --- | --- |
| Pulsing Blue | Listening / scanning | Pico receives 'B' |
| Solid Gold | Gemini AI processing | Pico receives 'G' |
| Solid White | Speaking (TTS active) | Pico receives 'W' |
| OFF | Idle / no one nearby | Pico receives 'X' |

⬛ ⚠ DANGER NeoPixels draw up to 720mA at full white. Powering 12 from a Pi GPIO pin (max 16mA) will destroy the Pi. Always power from VBUS (5V regulated USB pin) on the Pico.

## B9 — USB Webcam

What it does: Two jobs: (1) Camera lens feeds frames to OpenCV on the Pi for face-tracking. (2) Built-in microphone captures user's voice for speech recognition.

Why remove the casing? The outer plastic housing is bulky and cannot be mounted flush inside the head eye socket. By removing it (usually two small screws on the back), you get just the PCB + lens + USB cable, which is compact and can be bolted directly inside the head box on a plywood bracket.

Lens alignment is critical: The lens PCB must be centered exactly behind the eye hole. Even 2mm off-center makes the robot look cross-eyed. Use the plywood bracket to achieve precise positioning before final gluing.

⬛ 💡 PRO TIP Confirm USB webcam is detected before mounting: plug into Pi, run 'ls /dev/video*' in terminal. Should show /dev/video0. If nothing appears, webcam is faulty or USB cable is damaged.

## B10 — 12V Li-ion Battery Pack

What it is: Lithium-ion rechargeable battery pack, 12V output. Mounted at the absolute lowest point of the chassis. Low center of gravity = less risk of tipping.

Why 12V specifically? BO motors operate at 6–12V. At 12V they run at maximum speed. The L298N handles up to 35V, so 12V is safe for the driver. Lower voltage = slower robot. Higher voltage = destroyed motors.

⬛ ⚠ DANGER Li-ion batteries can catch fire if short-circuited or overcharged. Always use the included charger only. Install a physical master kill switch inline with the 12V battery positive wire. This allows instant power cut without hunting for the battery.

## B11 — 20,000mAh USB Power Bank (Clean Power)

Minimum spec required: 20,000mAh capacity AND 2.1A output on at least one USB port. Why 2.1A? The Pi alone can draw 700mA. Uno + Mega + Pico add ~200mA more. At peak with USB bus loaded, total draw ≈ 1.2A. 2.1A port leaves 0.9A headroom — critical to prevent brownout.

Runtime estimate: At 1.2A average draw, a 20,000mAh (= 20Ah) bank lasts approximately 16 hours. For a 6-hour exhibit day, this is more than sufficient.

⬛ ⚠ DANGER Some budget power banks auto-shut off if current draw is below a threshold (designed to stop slow-charging). Test: plug in Pi and let it idle for 10 minutes. If power bank shuts off: it has auto-shutoff. Buy a better one.

## B12 — MicroSD Cards: Why Samsung or SanDisk Only

Why these brands? The Pi's OS does thousands of tiny read/write operations per second. Cheap microSD cards from unknown brands have slow write speeds and fail after a few weeks of heavy use. Samsung EVO and SanDisk Ultra are rated for continuous high-speed writes.

Required spec: 32GB minimum, Class 10 / UHS-I (U1) or faster. The Raspberry Pi OS image is ~4GB; 32GB leaves 28GB for Python packages, OpenCV, Gemini library, and data.

⬛ ⚠ DANGER Always keep a backup image of the configured Pi SD card. After fully setting up jarvis.py and all packages, use Raspberry Pi Imager on your laptop to create a backup image file. SD card corruption happens when power is cut to the Pi during a write operation. Always sudo shutdown -h now. Never pull the plug.

## B13 — 1000µF Capacitor: Brownout Protection

What it is: An electrolytic capacitor — essentially a tiny rechargeable battery that charges and discharges in milliseconds. Mounted directly across the L298N VIN (+) and GND (–) terminals.

The problem it solves: When motors start, they briefly draw 5–10× their normal current (inrush current). This causes a momentary voltage sag on the power line. The capacitor stores charge and releases it instantly to fill that sag — like a speed bump absorber. Without it, motor startups cause voltage dips that can reset the Arduino Mega.

⬛ ⚠ DANGER Electrolytic capacitors are POLARIZED — they have a + and – leg. The longer leg is positive. The negative leg has a white stripe on the capacitor body. Connect backwards = capacitor explodes. Always double-check polarity before powering on.

## B14 — Resistors: Ohm's Law in Practice

Why 330Ω for LEDs? Pi GPIO pins output 3.3V. A typical LED has a forward voltage of ~2V and max current of 20mA. Voltage drop across resistor = 3.3V − 2V = 1.3V. Required R = 1.3V ÷ 0.02A = 65Ω. We use 330Ω which limits to just 4mA — the LED still glows well but runs cool, extending life.

### Resistor Color Code (4-Band)

| Color | Value | Color | Value |
| --- | --- | --- | --- |
| Black | 0 | Green | 5 |
| Brown | 1 | Blue | 6 |
| Red | 2 | Violet | 7 |
| Orange | 3 | Grey | 8 |
| Yellow | 4 | White | 9 |

Reading a 330Ω resistor: Orange(3), Orange(3), Brown(×10) = 330Ω. Gold 4th band = ±5% tolerance.

# SECTION C — TOOL SCHOOL: How to Use Every Tool Safely

Rule #1: Read the 'Common Mistakes' for every tool before picking it up. Most injuries happen in the first 5 minutes of using a tool incorrectly.

## C1 — Jigsaw

What it does: Cuts curves and straight lines through wood and cardboard. The blade moves up and down rapidly while the tool glides forward.

| How to use correctly | For straight cuts: clamp a metal ruler as a guide and run the jigsaw base plate against it. Mark cut line first. Stay 1mm outside the line; sand to the final line. |
| --- | --- |
| Blade types | Fine-tooth blades for plywood (less splintering). Coarse blades for cardboard. Never force the blade — let it cut at its own speed. |
| Safety | Clamp workpiece firmly so it cannot move. Keep both hands on the tool. Never cut toward yourself. Wear safety glasses. |
| Common mistakes | Cutting too fast = blade wanders off line. Not clamping = workpiece vibrates and ruins the cut. Using a blunt blade = rough, splintered edges. |

## C2 — Power Drill

| How to use correctly | Mark the hole position with a pencil. Always start with a 2–3mm pilot hole before the final size. This prevents wood splitting. Drill perpendicular — use a drill guide or watch from two angles. |
| --- | --- |
| Pilot holes | Always pre-drill before screwing into plywood edges. Plywood splits along grain without a pilot hole. Pilot hole = 75% of screw shaft diameter. |
| Screw vs bolt | Screw: goes directly into wood, self-anchoring. Bolt: goes through both pieces, needs a nut on the other side. Use bolts for high-stress joints (leg-to-chassis, motor mounts). |
| Safety | Secure workpiece. Never hold with your hand near the drill bit. Start at slow speed, increase gradually. |
| Common mistakes | No pilot hole → split wood. Drilling too fast → overheating → burning wood. Wrong bit size → stripped screw hole. |

## C3 — Soldering Iron

Temperature: Set to 350°C for standard electronics solder (60/40 tin-lead). 370°C for lead-free solder.

| Good joint | Shiny, smooth volcano shape. Flows around both the wire and the pad. Sets in ~2 seconds. |
| --- | --- |
| Cold joint | Dull, grey, lumpy. Looks like a used chewing gum. High resistance = signal noise, intermittent connection. REDO IT. |
| How to solder | Tin the tip first (apply a little solder to clean it). Hold iron on the joint for 2 seconds to heat it. Then apply solder to the JOINT (not the iron). Remove solder, then iron. Let cool — do NOT blow or move. |
| Safety | Soldering iron tip reaches 350°C — instant third-degree burn. Never put down a hot iron without the stand. Solder in ventilated area — flux fumes are toxic. |
| Common mistakes | Touching iron to solder first → solder drips, no proper joint. Moving joint before cooled → cold joint. Too much solder → bridge between pads → short circuit. |

## C4 — Multimeter

Most important tool in your kit. If you only learn one tool, learn this. It tells you what's actually happening in a circuit.

| Mode | How to Use | Use for Jarvis |
| --- | --- | --- |
| Continuity (🔊) | Red to one end, black to other. Beep = connected. No beep = broken or disconnected. | Verify every wire connection before power-on. Check common ground. |
| DC Voltage (V—) | Red to positive point, black to GND. Read the voltage. | Check 12V battery level. Verify 5V pins on Arduino. |
| Resistance (Ω) | Component out of circuit. Red and black to each leg. | Verify resistor values. Check motor coil resistance (should be 5–20Ω). |

⬛ ⚠ DANGER NEVER use the current (A) measurement mode on Jarvis circuits. You must break the circuit to measure current. Using voltage probes to try to measure current = short circuit = destroyed multimeter and possibly destroyed board. Always start with the HIGHEST range on the dial, then decrease. Starting low on a high voltage = pegged meter = possible damage.

## C5 — Wire Strippers

Wire gauge guide: 22–24 AWG for signal wires (sensor, data). 18–20 AWG for power wires under 1A. 16 AWG for motor wires. The notch on the stripper must match the wire gauge — too tight = cut the copper conductors. Too loose = insulation won't strip cleanly.

## C6 — Heat Gun & Heat Shrink Tubing

What it does: Heat shrink tubing slips over a wire junction and shrinks down tightly when heated. Creates an insulated, waterproof, mechanically strong cover. Always use on soldered wire-to-wire joints.

How to use: Slip heat shrink BEFORE soldering (cannot get it past a solder joint). Solder the joint. Slide the tube over. Apply heat gun 2–3cm away, moving constantly. Tube shrinks in 3–5 seconds.

## C7 — Clamps

Types: G-clamps for small pieces. F-clamps for wide boards. Spring clamps for light pressure during glue drying. At minimum pressure — you want firm contact, not crushing force. Over-clamping bows plywood.

## C8 — Craft Knife

How to use correctly: Always cut against a metal ruler — never freehand. Score first with light pressure, deepen in 2–3 passes. Change blade the moment it feels like it's dragging rather than slicing. A sharp blade is SAFER than a dull one (requires less force, more control).

⬛ ⚠ DANGER Always cut AWAY from yourself. Secure the workpiece. Never hold the ruler with fingers in the blade's path — use the non-cutting edge of the ruler as a guide fence.

## C9 — Spirit Level

Two-direction rule: Always check from BOTH front-to-back AND side-to-side. A pipe can be perfectly level in one plane and 5° off in the other. Particularly critical for the PVC spine — use the plumb-line method (weight on string) as backup.

## C10 — Sandpaper

| Grit | Use For |
| --- | --- |
| 80-grit (coarse) | Removing material, rough shaping — rarely needed |
| 120-grit (medium) | First sanding of cut plywood edges |
| 240-grit (fine) | Final sanding before painting |
| Block vs hand | Use a sanding block (sandpaper wrapped around a flat block) for flat surfaces. It distributes pressure evenly and prevents finger-shaped grooves. Hand-sand only for curves. |

# SECTION D — CODE EXPLAINED: Every Line, Every Reason

Who needs this? Software team and anyone debugging or modifying behavior. Every single line explained.

## D1 — Arduino Uno: The Bouncer

Mission: Never sleeps. Watches the room. If a human steps within 1 metre, sends HIGH signal to Pi GPIO 17 to wake up the AI system.

| Code Line | Explanation |
| --- | --- |
| int trigPin = 9; | The Arduino pin physically connected to the HC-SR04 TRIG (transmit) wire. Pin 9 chosen because it's a regular digital output pin. |
| int echoPin = 10; | Connected to HC-SR04 ECHO (receive) wire. Reads the return signal duration. |
| int wakePin = 2; | Connected to Raspberry Pi GPIO Pin 17. Goes HIGH to wake the Pi. |
| pinMode(trigPin, OUTPUT); | Tells Arduino: I will SEND signals on this pin (to trigger the ultrasonic burst). |
| pinMode(echoPin, INPUT); | Tells Arduino: I will RECEIVE signals on this pin (the echo return). |
| pinMode(wakePin, OUTPUT); | Tells Arduino: I will SEND the wake signal from this pin to the Pi. |
| Serial.begin(9600); | Opens serial communication at 9600 baud for debugging (optional — useful in Serial Monitor). |
| digitalWrite(trigPin, LOW); delayMicroseconds(2); | Resets the trigger pin LOW first. Ensures the TRIG line is clean before we send the pulse. |
| digitalWrite(trigPin, HIGH); delayMicroseconds(10); | Sends a 10-microsecond HIGH pulse — this is exactly what HC-SR04 needs to fire one ultrasonic burst. |
| digitalWrite(trigPin, LOW); | Returns TRIG to LOW immediately after the pulse. |
| duration = pulseIn(echoPin, HIGH, 30000); | Measures how long ECHO pin stays HIGH. The 30,000µs timeout = max wait. If no echo in 30ms, returns 0 (nothing detected or out of range). |
| distance = (duration == 0) ? 999 : (duration / 2) / 29.1; | Converts time to distance. Sound travels at 343m/s = 0.0343 cm/µs = 1cm per 29.1µs round trip (÷2 for one way). duration==0 guard returns 999 (nothing there) instead of a meaningless 0. |
| if (distance > 0 && distance < 100) | Triggers wake only when something is between 1cm and 100cm. The >0 check filters out the timeout=0 case. |
| digitalWrite(wakePin, HIGH); | Sends 5V on the wake wire to Pi GPIO 17. Pi code sees this as GPIO.HIGH = someone is there! |
| delay(500); | Checks twice per second. Enough to catch someone approaching. Faster would waste power; slower would miss quick movements. |

## D2 — Arduino Mega: The Muscle

Mission: Receive movement commands from Pi via USB serial. ALWAYS check the kill-zone sensor FIRST on every loop. If object <30cm: stop everything and ignore Pi. Safety overrides all.

| Code Line | Explanation |
| --- | --- |
| const int trigPin=9, echoPin=10; | Kill-zone sensor pins. Same HC-SR04 wiring as Uno but on Mega's physical pins 9 and 10. |
| const int leftMotorForward=4, leftMotorBackward=5; | L298N IN1 and IN2. Both LOW = stop. Forward HIGH+LOW. Backward LOW+HIGH. |
| const int rightMotorForward=6, rightMotorBackward=7; | L298N IN3 and IN4. Same logic for right motor. |
| const int motorSpeedLeft=3, motorSpeedRight=8; | PWM pins connected to L298N ENA and ENB. analogWrite() values here control speed. Both are hardware PWM pins on Mega. |
| Serial.begin(9600); | Opens USB serial at 9600 baud. Pi sends movement commands here. MUST match Pi code's Serial('/dev/ttyACM0', 9600). |
| dur = pulseIn(echoPin, HIGH, 30000); | Reads kill-zone sensor. 30000µs timeout prevents blocking. |
| dist = (dur == 0) ? 999 : (dur/2)/29.1; | Same conversion as Uno. 999 = no object. Anything from 1–30 triggers emergency stop. |
| if (dist > 0 && dist < 30) { stopMotors(); ... return; } | THE HARDWARE OVERRIDE. If anything is within 30cm: stop motors, flush the serial buffer (discard any Pi commands), wait 100ms, then RETURN — skips the entire rest of the loop. Pi is completely ignored. |
| while(Serial.available()) Serial.read(); | BUFFER FLUSH. When the kill-zone triggers, any movement commands sitting in the serial buffer are discarded. Without this, Jarvis would lurch forward the instant the obstacle leaves. |
| char cmd = Serial.read(); | Reads one character from the Pi. F=forward, B=backward, L=left, R=right, S=stop. |
| switch(cmd) | Clean way to handle multiple commands. Default case: stopMotors() — any unrecognized command is treated as stop for safety. |
| forward(150,150) | Two arguments: left speed, right speed. 150/255 = ~59% speed. If robot drifts, adjust these two numbers. |
| analogWrite(motorSpeedLeft, l); | PWM duty cycle. 0 = stop, 255 = full speed. Values 80–200 give good controllable speeds. |

## D3 — Raspberry Pi Pico: The Artist

Mission: Wait for a single letter from the Pi via USB serial. Translate it into the correct LED color on the NeoPixel ring.

| Code Line | Explanation |
| --- | --- |
| import machine, neopixel, sys, time | machine: hardware access. neopixel: built-in MicroPython library for WS2812B LEDs. sys: system functions. time: delays for LED animations. |
| NUM_LEDS = 12 | Our ring has 12 LEDs. Change this if using a different ring size. |
| LED_PIN = 28 | GP28 on Pico (physical pin 34). Connected to NeoPixel DIN wire. |
| np = neopixel.NeoPixel(machine.Pin(LED_PIN), NUM_LEDS) | Creates the LED strip object. machine.Pin(28) = the data output pin. 12 = number of LEDs. |
| def set_color(r, g, b): | Function that sets ALL 12 LEDs to the same color. r,g,b are each 0–255. |
| for i in range(NUM_LEDS): np[i] = (r, g, b) | Sets each LED's color value in the buffer. Does NOT light up yet. |
| np.write() | CRITICAL: This command sends all the buffered colors to the actual LEDs. Without this line, nothing changes on the ring. |
| while True: | Infinite loop — Pico runs this forever after power-on. |
| letter = sys.stdin.read(1) | BLOCKING CALL — waits here until exactly one character arrives via USB serial from the Pi. This is why the Pico doesn't need any complex waiting logic. |
| if letter == 'B': set_color(0, 0, 255) | Blue = Listening state. Pure blue (R=0, G=0, B=255). |
| elif letter == 'G': set_color(255, 200, 0) | Gold = Thinking state. (R=255, G=200, B=0). Adjust G value to change gold hue. |
| elif letter == 'X': set_color(0, 0, 0) | OFF — idle state. All LEDs dark. |

## D4 — Raspberry Pi: The Master Brain (jarvis.py)

Mission: Coordinate everything. Wait for wake signal. Approach visitor. Listen. Think with Gemini. Speak answer. Handle the secret.

| Code Line | Explanation |
| --- | --- |
| import RPi.GPIO as GPIO, serial, time, glob | GPIO: Raspberry Pi pin control. serial: USB communication with Mega and Pico. time: delays. glob: finds USB device ports automatically. |
| import speech_recognition as sr | Python library that uses Google's online speech-to-text API to convert mic audio into text. |
| import google.generativeai as genai | The official Google Gemini AI library. Sends text prompts and receives AI-generated text responses. |
| import pyttsx3 | Text-to-speech library. Converts AI text responses into spoken audio through the speaker. |
| GEMINI_API_KEY = 'YOUR_KEY_HERE' | Your personal API key from aistudio.google.com. Without this, Gemini won't respond. |
| WAKE_PIN = 17 | Raspberry Pi GPIO 17. Listens for the HIGH signal from Arduino Uno's Pin 2. |
| GPIO.setmode(GPIO.BCM) | Uses BCM (Broadcom chip) pin numbering. GPIO 17 in BCM = physical pin 11. BCM is the standard numbering for Pi projects. |
| GPIO.setup(WAKE_PIN, GPIO.IN, pull_up_down=GPIO.PUD_DOWN) | GPIO 17 as INPUT with internal pull-down. Normally LOW (0V). Goes HIGH only when Uno sends 5V. |
| def find_serial_port(desc): | Scans /dev/ttyACM* and /dev/ttyUSB* to find connected Arduino/Pico automatically. Better than hardcoding ttyACM0 because the order can change. |
| mega = serial.Serial(mega_port, 9600, ...) | Opens USB serial to Mega at 9600 baud. Sending b'F' makes Mega move forward. |
| pico = serial.Serial(pico_port, 9600, ...) | Opens USB serial to Pico at 9600 baud. Sending b'B' makes LEDs go blue. |
| model = genai.GenerativeModel('gemini-1.5-flash') | Uses Gemini 1.5 Flash — fast response, free tier, ideal for exhibit use. |
| recognizer.adjust_for_ambient_noise(source, duration=1) | Listens to the room for 1 second and calibrates the silence threshold. CRITICAL for exhibit halls with crowd noise. Increase to duration=2 in noisy environments. |
| audio = recognizer.listen(source, timeout=5, phrase_time_limit=5) | Waits up to 5 seconds for speech to start. Stops recording after 5 seconds of speech. Prevents Jarvis from listening forever. |
| answer = ask_gemini(user_input) | Sends the recognized speech to Gemini AI and gets a response text back. |
| engine.say(answer); engine.runAndWait() | Converts Gemini's text response to spoken audio through the speaker. runAndWait() blocks until speaking is finished. |
| except KeyboardInterrupt: | Catches Ctrl+C in the terminal. Runs cleanup code (GPIO.cleanup, close serial ports) before exiting gracefully. |
| GPIO.cleanup() | Resets all GPIO pins to safe state. Always required before Pi Python program exits. Skipping it leaves pins in undefined state. |

## D5 — Serial Communication: How the Boards Talk

| Concept | Explanation |
| --- | --- |
| Baud rate 9600 | 9600 bits per second. Both ends must be set to the same rate. Different rates = garbage data. 9600 is slow but reliable for simple single-character commands. |
| USB Star Topology | Pi is the center hub. Pi → Mega (ttyACM0), Pi → Pico (ttyACM1), Pi → Webcam (video0). Each device has its own dedicated cable. No daisy-chaining. This prevents interference between devices. |
| b'F' vs 'F' | In Python 3, serial.write() requires bytes. b'F' is a bytes literal = the ASCII code for 'F' (70). Sending 'F' as a string would cause an error. |
| Serial.read() on Arduino | Returns a char (character). 'F', 'B', 'L', 'R' commands are single characters to keep the protocol simple and fast. |
| Port detection | Pi assigns /dev/ttyACM0 to the first USB serial device detected after boot. Order depends on which device was plugged in first. Use find_serial_port() in jarvis.py or plug devices in a consistent order. |

# SECTION E — WIRING ATLAS: Every Connection Documented

RULE #1: Work with ALL power disconnected. Only connect power in Phase 15 of the build manual. Use multimeter continuity check on every wire before powering on.

## E1 — Arduino Uno Wiring (The Bouncer)

| Uno Pin | Connects To | Wire Color | Why This Pin |
| --- | --- | --- | --- |
| 5V | HC-SR04 Sensor 1 VCC | Red | Powers the sensor. Uno 5V pin provides regulated 5V from USB power bank. |
| GND | HC-SR04 Sensor 1 GND | Black | Ground reference for sensor. |
| GND | Pi GND (any GND pin) | Black | Common ground — CRITICAL for wake signal to work. |
| Pin 9 | HC-SR04 Sensor 1 TRIG | Yellow | Digital OUT. Sends 10µs pulse to fire ultrasonic burst. |
| Pin 10 | HC-SR04 Sensor 1 ECHO | Green | Digital IN. Reads echo return time duration. |
| Pin 2 | Pi GPIO 17 (Pin 11) | Orange | Digital OUT. Goes HIGH when distance < 100cm to wake the Pi. |

⬛ WHY THIS MATTERS Pin 2 on Uno is used for the wake signal because it is also capable of hardware interrupt on Uno — if we ever upgrade the code to interrupt-driven wake-up, Pin 2 is already the right pin.

## E2 — Arduino Mega Wiring (The Muscle)

| Mega Pin | Connects To | Wire Color | Why This Pin |
| --- | --- | --- | --- |
| 5V | HC-SR04 Sensor 2 VCC | Red | Powers kill-zone sensor. |
| GND | HC-SR04 Sensor 2 GND | Black | Sensor ground. |
| GND | L298N GND terminal | Black | CRITICAL common ground — without this motor signals don't work. |
| Pin 9 | HC-SR04 Sensor 2 TRIG | Yellow | Digital OUT. Fires kill-zone sensor pulse. |
| Pin 10 | HC-SR04 Sensor 2 ECHO | Green | Digital IN. Reads kill-zone echo. |
| Pin 3 (PWM) | L298N ENA terminal | Blue | PWM OUT. Speed control for left motor. |
| Pin 4 | L298N IN1 terminal | White | Digital OUT. Left motor forward direction. |
| Pin 5 | L298N IN2 terminal | Grey | Digital OUT. Left motor backward direction. |
| Pin 6 | L298N IN3 terminal | Purple | Digital OUT. Right motor forward direction. |
| Pin 7 | L298N IN4 terminal | Brown | Digital OUT. Right motor backward direction. |
| Pin 8 (PWM) | L298N ENB terminal | Blue | PWM OUT. Speed control for right motor. |
| USB-B port | Pi USB-A port | USB cable | Serial communication — Pi sends movement commands here. |

⬛ ⚠ DANGER ENA and ENB jumpers on L298N MUST BE REMOVED before connecting Mega PWM wires. With jumpers: motors run full speed always. Without jumpers: PWM control works.

## E3 — Raspberry Pi Pico Wiring (The Artist)

| Pico Pin | Connects To | Wire Color | Why This Pin |
| --- | --- | --- | --- |
| VBUS (Pin 40) | NeoPixel ring VCC/5V | Red | VBUS = 5V from USB. NOT the 3.3V regulated output. NeoPixels need 5V not 3.3V. |
| GND (Pin 38) | NeoPixel ring GND | Black | Ground. |
| GP28 (Pin 34) | NeoPixel ring DIN (Data In) | Yellow | Data line. 3.3V signal from GP28 is enough for WS2812B protocol. Any GP pin works; 28 is chosen for torso routing. |
| Micro-USB port | Pi USB-A port | USB cable | Power (5V from Pi's USB port) AND serial data (LED commands). |

## E4 — Raspberry Pi 3B+ Wiring

| Pi Connection | Connects To | Notes |
| --- | --- | --- |
| GPIO 17 (Pin 11) | Uno Pin 2 (wake wire) | INPUT. Reads HIGH = someone within 1m. Pull-down enabled in code. |
| GPIO GND (Pin 9) | Uno GND | Common ground — allows wake signal to work correctly. |
| GPIO (any GND) | 3.3V for decoy eye LED via 330Ω resistor | Powers the decoy eye glow LED in the head. |
| USB Port 1 | Arduino Mega (USB-B) | Motor commands: b'F', b'B', b'L', b'R', b'S' |
| USB Port 2 | Raspberry Pi Pico (micro-USB) | LED commands: b'B', b'G', b'W', b'X' |
| USB Port 3 | USB Webcam | Camera feed to OpenCV + microphone for speech recognition. |
| 3.5mm audio jack | External speaker | Plays pyttsx3 TTS audio output. |
| Micro-USB power | 20,000mAh power bank | 5V 2.1A minimum. This powers ALL brain boards via USB. |
| MicroSD slot (underside) | 32GB MicroSD | Running Raspberry Pi OS (Legacy Bullseye 32-bit). |

## E5 — L298N Motor Driver Complete Wiring

| L298N Terminal | Connects To | Wire | Notes |
| --- | --- | --- | --- |
| VIN / 12V | 12V battery positive (+) | Red (heavy gauge) | Motor supply. 12V input for motor power. |
| GND (power GND) | 12V battery negative (–) AND Mega GND | Black | BOTH 12V battery GND AND Mega GND must connect here. This creates common ground. |
| ENA | Mega Pin 3 (PWM) | Blue | Remove ENA jumper first. This pin now controls left motor speed. |
| IN1 | Mega Pin 4 | White | Left motor: HIGH = forward direction |
| IN2 | Mega Pin 5 | Grey | Left motor: HIGH = backward direction |
| IN3 | Mega Pin 6 | Purple | Right motor: HIGH = forward direction |
| IN4 | Mega Pin 7 | Brown | Right motor: HIGH = backward direction |
| ENB | Mega Pin 8 (PWM) | Blue | Remove ENB jumper first. Right motor speed control. |
| OUT1, OUT2 | Left BO Motor | Red/Black | Polarity sets direction. Swap if motor runs backward. |
| OUT3, OUT4 | Right BO Motor | Red/Black | Same — swap wires if wrong direction. |
| 1000µF capacitor | Across VIN (+) and GND | — | Longer leg (+) to VIN. Shorter leg (–) to GND. Brownout protection. |

## E6 — Power Architecture Summary

⬛ COMPLETE POWER MAP — Read Before Wiring Anything DIRTY POWER CIRCUIT (12V — NEVER touch brain boards): 12V Battery (+) → L298N VIN → L298N OUT1/2/3/4 → BO Motors → 12V Battery (–) 12V Battery (–) also connects to: Mega GND + L298N GND [= Common Ground] CLEAN POWER CIRCUIT (5V — brain boards only): USB Power Bank (5V 2.1A) → Pi Micro-USB Pi USB Port 1 → Mega (5V logic power + serial data) Pi USB Port 2 → Pico (5V VBUS + serial data + NeoPixel power) Pi USB Port 3 → USB Webcam (5V power + data) COMMON GROUND BRIDGE (single wire connecting the two circuits): 12V Battery (–) = L298N GND = Mega GND [all on same terminal] The 5V USB circuit shares GND through the Mega's USB connection

# SECTION F — TROUBLESHOOTING: Fix Anything Fast

How to use this section: Read the symptom. Follow the numbered steps IN ORDER — each step assumes previous steps have been checked.

### F1 — Robot Won't Move At All

- Is the 12V battery connected to L298N VIN? Check red wire from battery (+) to L298N.

- Is the 12V battery charged? Test voltage with multimeter — should read 11–12.6V.

- Is jarvis.py running on the Pi? Check SSH terminal for 'Jarvis is ready...' message.

- Is Mega connected to Pi via USB? Run 'ls /dev/ttyACM*' — should show ttyACM0.

- Test motor command manually: python3, import serial, mega=serial.Serial('/dev/ttyACM0',9600), mega.write(b'F'). Motors should run.

- If still no movement: open Serial Monitor on Mega, set 9600 baud, send 'F'. If motors run: problem is Pi→Mega communication. If motors don't run: problem is Mega→L298N wiring.

- Check ENA/ENB jumpers are REMOVED from L298N.

- Check L298N GND connects to both 12V battery (–) AND Mega GND.

### F2 — Motors Twitch But Don't Spin

- Classic symptom of insufficient current. 12V battery is weak or connection is loose.

- Check 12V battery voltage: should be >11V under load. If <10V: charge the battery.

- Tighten all screw terminal connections on L298N. Loose terminal = high resistance = voltage drop = twitch.

- Check the 1000µF capacitor is correctly installed (+ to VIN, – to GND). Reversed capacitor causes this symptom AND can burst.

- Test each motor independently: disconnect from L298N, connect directly to 9V battery. If motor spins freely: L298N problem. If motor only twitches on 9V too: motor is dying.

### F3 — One Wheel Spins Slower

- This is a motor speed calibration issue (very common — no two motors are identical).

- In Mega code, find: analogWrite(motorSpeedLeft, 150); analogWrite(motorSpeedRight, 150);

- If drifting LEFT (left wheel slower): increase motorSpeedLeft by 5–10 (try 160). OR decrease motorSpeedRight.

- If drifting RIGHT: increase motorSpeedRight OR decrease motorSpeedLeft.

- Adjust in increments of 5. Re-upload code and test each time.

- Also check motor mounting: if one motor's axle is slightly tilted, it creates more ground friction → slower effective speed. Inspect motor bolts.

### F4 — No Wake-Up Response (Nothing happens when approaching)

- Check orange wire: Uno Pin 2 → Pi GPIO 17 (physical pin 11). Continuity check.

- Check common ground: Uno GND → Pi GND (any GND pin). Without this, the wake signal doesn't have a reference.

- Is jarvis.py running? The wake detection loop only runs when jarvis.py is active.

- Test Uno in Serial Monitor: wave hand in front of Sensor 1. 'Wake!' messages should appear.

- Test GPIO directly: on Pi, run 'gpio readall' (or use Python: GPIO.input(17)). Trigger Uno manually: should read 1 when active.

- In jarvis.py, add: print(GPIO.input(WAKE_PIN)) inside the while loop to debug the pin reading.

- Check Sensor 1 wiring: 5V → VCC, GND → GND, Pin9 → TRIG, Pin10 → ECHO. A sensor with VCC and GND swapped will be dead.

### F5 — LEDs Don't Respond

- Is Pico file named main.py exactly? 'Main.py', 'MAIN.py', etc. will NOT auto-run. Reconnect via Thonny and verify filename.

- Is Pico detected by Pi? Run 'ls /dev/ttyACM*'. Pico should appear.

- Test LED manually: python3, import serial, pico=serial.Serial('/dev/ttyACM1', 9600), pico.write(b'B'). Ring should go blue.

- Check VBUS (pin 40) → NeoPixel VCC. VBUS is 5V. DO NOT use 3V3 pin — LEDs need 5V.

- Check GP28 → NeoPixel DIN. Loose connection here = no LED response.

- In Thonny, open Serial Monitor with Pico connected. Type B and press Enter — should light up blue. If yes: problem is Pi→Pico serial. If no: problem is Pico code or wiring.

### F6 — Wrong LED Colors

- LED color is defined in Pico code as (R, G, B) tuples. If colors seem swapped, your NeoPixel ring might use GRB format instead of RGB.

- Test: send 'B' command (should be pure blue = R0,G0,B255). If it glows red instead: swap R and G values in all set_color() calls in Pico code.

- Gold looks wrong: adjust the (255, 200, 0) values. More orange: reduce G. More yellow: increase G.

- LEDs too bright: reduce the max values (e.g., (0, 0, 150) for dimmer blue).

### F7 — No Speech Recognition

- Is the USB webcam detected? Run 'lsusb' on Pi — webcam should appear in the list.

- Is the microphone working? Run 'arecord -l' — should list a USB audio device.

- Run this test: python3 -c "import speech_recognition as sr; r=sr.Recognizer(); m=sr.Microphone(); print('Mic OK')"

- If Microphone() fails: run 'sudo apt install portaudio19-dev' and reinstall pyaudio.

- In jarvis_routine(), is adjust_for_ambient_noise() running? Without it in a noisy exhibit hall, the silence threshold is wrong and Jarvis never detects speech starting.

- Test internet connection: recognizer.recognize_google() sends audio to Google's servers. 'wifi network' → ping google.com.

### F8 — Speech Recognition Picks Up Everything (Never stops listening)

- Exhibit hall is too noisy. Increase ambient noise calibration: change duration=1 to duration=2 in adjust_for_ambient_noise().

- Add phrase_time_limit: already set to 5 seconds in the code. This forces Jarvis to stop listening after 5 seconds regardless.

- Increase energy threshold manually: recognizer.energy_threshold = 400 (add this line before listen()). Default is ~300.

- Position the webcam microphone to face slightly downward — this reduces room echo pickup.

- As a last resort: stand closer to the robot when speaking (30–50cm from microphone).

### F9 — AI (Gemini) Doesn't Respond

- Is the API key in jarvis.py? Look for GEMINI_API_KEY = 'YOUR_KEY_HERE' — if it still says 'YOUR_KEY_HERE' the AI won't work.

- Is the Pi connected to the internet? Run 'ping 8.8.8.8'. If no response: fix Wi-Fi.

- Did you flash the Pi OS with correct Wi-Fi credentials? Check Wi-Fi SSID and password in raspi-config.

- Test Gemini manually: python3, import google.generativeai as genai, genai.configure(api_key='YOUR_KEY'), m=genai.GenerativeModel('gemini-1.5-flash'), print(m.generate_content('hello').text)

- If Gemini quota exceeded: free tier has usage limits. Use 'gemini-1.5-flash' (not 'gemini-pro') which has the most generous free quota.

### F10 — Random Shutdowns or Reboots

- Classic brownout symptom. Check the 1000µF capacitor on L298N is correctly installed (+) to VIN.

- Is the power bank 2.1A output? Some banks are 1A on some ports. Use the 2.1A port (usually marked with a lightning bolt or '2.1A' label).

- Is the USB cable from power bank to Pi a data cable (not charge-only)? Charge-only cables sometimes have higher resistance — voltage drop under load causes reboot.

- Check for bad USB contact: wiggle the power bank cable gently while Pi is running. If it reboots: replace the cable.

- On Pi: run 'dmesg | grep -i volt' after reboot — look for 'under-voltage detected' messages confirming the cause.

### F11 — Kill-Zone Not Stopping (Robot doesn't stop when hand in front)

- Is Sensor 2 (kill-zone) connected to Mega, not Uno? Common mix-up.

- Is the sensor pointing forward? It must face the direction of robot travel.

- Open Mega Serial Monitor. Add: Serial.println(dist); after the distance calculation. Walk in front: distance should decrease and hit <30 to trigger stop.

- Check sensor tilt: if pointing at floor, it reads floor distance (~20cm) constantly and the kill-zone triggers all the time. Point slightly upward.

- Verify the return; statement exists after stopMotors() in the kill-zone block. Without return, code continues and motors restart.

### F12 — Ghost Echoes (Robot stops with nothing in front)

- Most common cause: sensor pointed at floor. Floor distance ≈ 20–30cm = always inside kill-zone. Tilt sensor 5–10° upward.

- Multiple reflective surfaces (glass, mirrors) create false echoes. Point sensor away from reflective walls.

- Narrow environments: echoes bouncing off walls. Add a minimum range check: if (dist < 5) treat as invalid.

- Electronic interference: keep sensor wires away from motor wires. Motor wires carry pulsing high current → EMI → false echo readings. Route sensor wires along the far side of the torso from motor wires.

### F13 — USB Ports Not Detected

- Run 'lsusb' on Pi. Should show: webcam, Arduino Mega (CP2102 or CH340 USB-Serial), Pico (MicroPython USB).

- Missing device: swap USB cable (charge-only cables don't transmit data).

- Arduino not detected: may need CH340 or CP2102 driver on laptop (for initial code upload). Pi usually detects without extra drivers.

- Run 'ls /dev/ttyACM*'. If Mega and Pico both show as ttyACM0 and ttyACM1: unplug both, plug Mega first, note its port, then plug Pico.

- Pico not detected as serial: confirm code is saved as main.py (not boot.py). Also verify MicroPython firmware is installed (BOOTSEL procedure).

### F14 — Pi Won't Boot

- No green LED activity at all: Pi isn't getting power. Check power bank output, USB cable, and try a different USB port.

- Green LED blinks then stops: SD card problem. Try re-flashing the OS image. Use a new Samsung or SanDisk card.

- Pi boots but no Wi-Fi: check SSID/password entered correctly in Imager settings. Run 'sudo raspi-config' to re-enter Wi-Fi.

- Pi boots but SSH connection refused: confirm SSH was enabled in Imager gear icon settings before flashing.

- Can't find Pi's IP address: log into your router's admin page and look for 'jarvis' in connected devices.

### F15 — Battery Dies Too Fast

- 12V battery draining fast: motors are running at maximum speed the whole time (150/255 ≈ 60% duty cycle × 2A peak = significant drain). Lower PWM values if full speed isn't needed.

- 5V power bank draining fast: OpenCV face tracking + Gemini API calls use significant Pi CPU. If exhibit is 6+ hours, bring a second power bank.

- Check if webcam is running OpenCV face tracking continuously even when no one is talking. In the current code, OpenCV runs only in jarvis_routine(). If you added continuous tracking: consider only activating after wake signal.

- Ensure pico.write(b'X') (LEDs OFF) runs during idle state. LEDs at full white = 720mA = big drain.

# SECTION G — EXHIBIT DAY SURVIVAL

September 16th. This is what all the work was for. Follow this section exactly.

## G1 — 30-Minute Setup Checklist

T-30 minutes: Arrive at location

- T-30: Place Jarvis upright on exhibit table/floor. NEVER lay horizontally during transport.

- T-28: Connect 20,000mAh power bank to Pi micro-USB. Start the 60-second boot timer.

- T-27: While Pi boots: unpack Oh-Shit kit. Arrange tools within reach but out of visitor sight.

- T-26: Pi should be booted. SSH in from laptop. Run: ls /dev/ttyACM* — confirm 3 devices.

- T-24: Connect 12V battery to L298N. Check kill-zone sensor is pointing forward.

- T-23: In SSH terminal: python3 jarvis.py. Wait for 'Jarvis is ready...' confirmation.

- T-20: Walk in front of Jarvis at 1-metre distance. Should: move forward briefly, stop, LEDs go blue, listen.

- T-18: Speak test phrase: 'What is your name?' Wait for Gemini response. Confirm speaker is audible at exhibit volume.

- T-15: Test kill-zone: while Jarvis is moving, step to within 30cm. Should stop instantly.

- T-10: Check all visible wires are cable-tied and no wires are visible through the body panels.

- T-8: Assign team roles (see G6). Brief everyone on visitor handling.

- T-5: Run the 5-minute pre-show test (see G2).

- T-0: Exhibit opens. One team member stationed at Jarvis at all times.

## G2 — 5-Minute Pre-Show Test

Run this immediately before exhibit opens:

- Power bank > 80% charge. Check indicator LEDs.

- 12V battery connected. Motor kill-switch in ON position.

- SSH connection live. jarvis.py running.

- Walk test: approach from 2m. Wake triggers at ~1m.

- Voice test: ask 3 different questions. All get Gemini responses.

- LED test: all 5 states confirmed (B=Blue, G=Gold, W=White, H=Pink, X=Off).

- Kill-zone test: confirm stop response.

- Speaker volume: audible at 3m in exhibit environment.

- All team members know their role (see G6).

## G3 — During-Show Troubleshooting (Fast Fix Guide)

| Problem | Fix in 60 Seconds |
| --- | --- |
| Jarvis stops moving mid-demo | Person standing within 30cm — step back. Kill-zone triggered. |
| LED stuck on one color | SSH: Ctrl+C to stop jarvis.py. python3 jarvis.py to restart. |
| Speech recognition loops endlessly | SSH: Ctrl+C. Restart jarvis.py. Cover mic with hand during restart. |
| Pi disconnected from SSH | Pi Wi-Fi dropped. Plug spare monitor+keyboard into Pi directly and restart jarvis.py. |
| Motor battery dies | Jarvis announces 'drive systems lost power'. Connect backup 12V battery via kill switch. Pi stays alive. |
| Jarvis won't stop talking | pyttsx3 stuck. SSH: sudo pkill python3. Wait 10s. Restart jarvis.py. |
| Complete freeze (no response) | SSH: sudo reboot. 90 second reboot. Explain to visitors: 'Jarvis is updating his neural pathways.' |
| Visitor too loud, no recognition | Ask visitor to step closer (50cm) and speak clearly toward Jarvis's eye level. |

## G4 — Safe Shutdown Sequence (WHY this order)

Never pull power directly from a running Pi.

- Step 1: Unplug 12V battery from L298N. WHY: Muscles die first. Prevents any accidental movement during shutdown. Pi continues on power bank.

- Step 2: In SSH terminal, type: sudo shutdown -h now. Press Enter. WHY: This tells the Pi's Linux OS to safely close all files, flush disk writes, and unmount the SD card before stopping power.

- Step 3: Wait 15–20 seconds. Watch the green LED on the Pi. WHY: Green LED blinking = disk activity (writing files). When it stops blinking permanently: safe to remove power.

- Step 4: Unplug the power bank from Pi. WHY: Only after the green LED stops. This ensures no data is being written when power is cut.

- Step 5: Coil all cables. Pack Jarvis upright (trolley or crate). WHY: Horizontal transport puts lateral forces on joints designed for vertical load.

## G5 — 'Oh Shit' Kit Contents

Pack this bag BEFORE going to the exhibit. Tape a checklist inside the bag.

- Spare 32GB MicroSD card (pre-flashed with jarvis.py and all packages) — for catastrophic SD corruption

- Spare USB cables (2× USB-A to USB-B, 1× micro-USB) — for cable failures

- Multimeter — for any wiring emergency

- Soldering iron + solder + helping hands — for broken wire joints

- Spare L298N motor driver — if driver burns out

- Spare HC-SR04 sensor × 2 — sensors crack under impact

- Spare BO motor × 2 — if motor dies mid-exhibit

- Spare 1000µF capacitor

- Spare 330Ω resistors × 10

- Small screwdriver set (Phillips + flat)

- Hot glue gun + sticks — for panel repairs

- Sticky tape and craft knife

- Laptop (with Arduino IDE + Thonny installed, and API key saved in a text file)

- Phone hotspot: if exhibit Wi-Fi fails, Pi can use phone hotspot for Gemini API

- Printed copy of this section (G) — for team members without laptop access

## G6 — Team Role Cards

Print and cut these. Give one to each team member before exhibit opens.

| Role | Responsibility |
| --- | --- |
| Lead Architect (Vikhyat) | Technical oversight. Makes final calls on repairs. Monitors SSH terminal. Primary spokesperson for technical questions. |
| Visitor Greeter | Stands 2m from Jarvis. Invites visitors to approach. Keeps crowd from entering 30cm kill-zone. |
| Explainer | Stands beside Jarvis. Explains what Jarvis is doing while it does it ('See — the chest just turned gold, that means he's thinking...') |
| Technical Monitor | Watches laptop SSH terminal. Alerts Lead if error messages appear. Restarts jarvis.py if it crashes. |
| Transport/Pack | Handles all physical movement, cable coiling, packaging during setup and shutdown. |

## G7 — What to Say to Visitors (Sample Script)

⬛ OPENING LINE (when visitor approaches) Visitor looking at Jarvis: 'This is Project Jarvis — a 4-board, AI-powered android. He uses a Raspberry Pi as his brain, Gemini AI to think, and four separate processors to control his movement, emotions, and senses. Walk up to him — he'll greet you.'

⬛ WHEN JARVIS IS LISTENING (blue LEDs) 'See those blue LEDs? That's his listening state. Ask him any question — he understands natural speech.'

⬛ WHEN JARVIS IS THINKING (gold LEDs) 'Gold means he's thinking — that's the Gemini AI generating his answer. He's using the same AI model as Google's AI assistant.'

⬛ WHEN ASKED 'Is it pre-programmed?' 'No. Every answer is generated in real-time by Gemini AI. We don't know what he'll say to any question — he truly thinks for himself. We only programmed the framework; the intelligence is his own.'

## G8 — How to Handle Difficult Questions About Jarvis

| Question | Answer |
| --- | --- |
| 'How much did this cost?' | The total build cost was kept under [your budget]. The most expensive parts were the Raspberry Pi and the power bank. The AI (Gemini) is free via Google's API. |
| 'Can it see me?' | Yes. The eye has a USB camera connected to OpenCV — an AI vision library. It can detect and track faces. It follows the Lead Architect when they move. |
| 'Why doesn't it walk?' | Walking robots spend most of their computing power on balance. By using a wheeled base, we free 100% of the CPU for intelligence and personality. Jarvis's design choice is 'smart over athletic'. |
| 'Is it dangerous?' | It has two safety systems. Sensor 1 wakes it up when you approach. Sensor 2 stops the wheels the moment you come within 30cm — it physically cannot run into you. |
| 'Can I touch it?' | Yes — the structure is designed to be handled. Please don't touch the eye lens (fingerprints affect camera quality) or open the back panel. |
| 'What would you do differently?' | Great question. Honest answer: add object recognition to the camera, add more movement commands, add a screen for displaying visuals alongside speech. These are v2 features. |

# SECTION H — THEORY DEEP DIVES: Understanding the 'Why'

This section is for builders who want to understand the physics and engineering behind Jarvis's design decisions.

## H1 — How Ultrasonic Sensors Actually Work (Physics)

The principle: HC-SR04 contains a piezoelectric transducer — a crystal that vibrates when voltage is applied, creating sound waves at 40kHz. This is above human hearing range (max ~20kHz).

The timing math: Sound travels at 343 m/s in air at 20°C. Formula: distance = speed × time. The echo travels twice the distance (out and back), so: distance = (time × 343) ÷ 2. In centimetres: distance(cm) = (time_µs / 2) / 29.1 — because 343 m/s = 0.0343 cm/µs = 1cm per 29.1µs.

Temperature dependence: Sound speed changes with temperature. At 0°C: 331 m/s. At 20°C: 343 m/s. At 40°C: 355 m/s. This causes ~3% error over a 20°C temperature range — acceptable for Jarvis's purpose.

Beam angle: HC-SR04 has a 15° detection cone. Objects within 15° of straight ahead are detected. This is why floor echoes are a problem when pointed horizontally — the floor is within the cone.

## H2 — PWM Explained: Duty Cycle and Frequency

What PWM really is: Pulse Width Modulation = rapidly switching a digital pin ON and OFF. The proportion of ON time to total cycle time is the duty cycle.

Mega PWM frequency: Arduino Mega hardware PWM runs at ~490Hz on most pins (including pins 3 and 8 used in Jarvis). At 490Hz, the motor receives 490 ON/OFF pulses per second. The motor's inertia averages these out into smooth rotation.

analogWrite values: 0 = 0% duty = 0V average = motor stopped. 127 = 50% duty = 6V average = half speed. 255 = 100% duty = 12V average = full speed. 150 = 59% duty ≈ 7V average = the speed used in Jarvis code.

Why not just use resistance to reduce speed? A resistor wastes energy as heat. A PWM signal turns the motor off between pulses (0V = 0 current = 0 waste). PWM is the efficient way to control DC motor speed.

## H3 — USB Protocol and Why Star Topology

USB basics: USB is a serial communication protocol (Universal Serial Bus). Data travels as binary pulses at 12Mbps (Full Speed USB 2.0). Each device gets its own address on the bus.

Star topology: All devices connect directly to the Pi (the hub), not to each other. Uno ← USB → Pi ← USB → Mega ← USB → Pico. Each cable is isolated. This is critical because:

- No single cable failure brings down the whole network.

- Electromagnetic interference from motor cables doesn't propagate through the USB bus.

- Each device appears as a separate /dev/ttyACM port — simple to identify and debug.

- Short cable runs (inside the torso) minimize signal degradation.

Alternatives and why we rejected them: I2C and SPI require custom wiring and more code complexity. Wireless (Bluetooth/Wi-Fi) between boards adds latency and failure modes. USB is reliable, standard, and the boards already have USB ports.

## H4 — Gemini AI API: How It Processes Prompts

What happens when Jarvis asks Gemini: The jarvis.py code sends an HTTPS request to Google's servers with: (1) your API key for authentication, (2) the system prompt ('You are Jarvis, keep answers brief...'), (3) the user's spoken words. Google's Gemini model processes this and returns text. Total round trip: 1–3 seconds.

Token limits: Gemini measures text in tokens (~4 characters). Gemini 1.5 Flash handles up to 1 million tokens in context. For Jarvis's short exchanges, this is infinite effective limit.

Why Gemini over GPT or Claude? Gemini 1.5 Flash has the most generous free tier suitable for continuous exhibition use. The Python library is simple. API key setup takes 2 minutes at aistudio.google.com.

What the system prompt does: 'You are Jarvis, a helpful robot assistant. Keep answers brief and friendly.' This shapes every response. Jarvis won't give 500-word essays because the prompt tells Gemini to be brief. Change this prompt to change Jarvis's personality.

## H5 — Speech-to-Text: How It Works

Three steps: (1) pyaudio captures raw audio from the USB webcam microphone as a digital waveform. (2) SpeechRecognition library sends this audio data to Google's Cloud Speech-to-Text servers via HTTPS. (3) Google returns the recognized text string.

Why online STT? Offline STT (like Vosk or Whisper) requires significant CPU — the Pi would take 3–8 seconds per utterance. Google Cloud STT is essentially instant (0.5s) because Google's massive servers process it. The internet dependency is the tradeoff.

energy_threshold: The SpeechRecognition library has a threshold below which it treats audio as silence. Too low: picks up room noise as speech. Too high: misses quiet speech. adjust_for_ambient_noise() auto-calibrates this threshold by sampling the background noise level.

## H6 — Why 4 Boards Instead of One Powerful Computer

The architectural philosophy: Each board does exactly one job, perfectly. No board is overloaded. If one board crashes, others continue.

| Board | Its Only Job | Why Not the Pi Alone |
| --- | --- | --- |
| Raspberry Pi | AI coordination, speech, vision | Can't do 5ms kill-zone response. Linux not real-time. |
| Arduino Mega | Motor control + kill-zone | Real-time response in µs. Pi Linux has ~10ms interrupt latency. |
| Arduino Uno | Wake-up sentinel | Never sleeps. Pi consumes too much power to run 24/7. |
| Pi Pico | LED animation | Pi GPIO can't drive NeoPixel protocol at correct timing. Pico handles µs-precise timing. |

Real-time vs. non-real-time: Linux (Pi) is a non-real-time OS. It can be interrupted by other processes at any moment. Arduino is a microcontroller — it runs one loop, no OS, no interruptions. This is why the kill-zone is on the Mega, not the Pi. A 30cm safety stop must respond in <5ms, always. The Pi cannot guarantee that.

# SECTION I — GLOSSARY: Every Term Defined

If you see a word in the documents you don't understand, find it here.

| Term | Definition |
| --- | --- |
| Ampere (A) | Unit of electrical current. 1 ampere = 1 coulomb of charge per second. Abbrev: A or mA (milliamps = 0.001A). |
| Ambient Noise | Background sound in an environment. The adjust_for_ambient_noise() function measures this to set the microphone threshold. |
| analogWrite() | Arduino function for PWM output. Takes values 0–255. 0=off, 255=full voltage. |
| API (Application Programming Interface) | A way for programs to communicate. The Gemini API is how jarvis.py asks Google's AI for answers. |
| API Key | A unique password-like code that proves your app has permission to use an API. Get yours at aistudio.google.com. |
| AWG (American Wire Gauge) | Standard for wire thickness. Lower AWG = thicker wire = carries more current. 22AWG (signal) to 16AWG (motor power). |
| Baud Rate | Speed of serial communication in bits per second. Jarvis uses 9600 baud. Both ends of a serial connection must match. |
| BCM Numbering | Broadcom chip pin numbering system for Raspberry Pi GPIO. GPIO17 in BCM = physical pin 11. Used in RPi.GPIO library. |
| BO Motor | Battery Operated DC gear motor. Small yellow hobby motors with built-in gearbox. |
| Brownout | When supply voltage drops below a circuit's minimum. Causes random resets or crashes. The 1000µF capacitor prevents brownout. |
| Buffer Flush | Discarding data waiting in a serial buffer. while(Serial.available()) Serial.read(); clears pending commands. |
| Center of Gravity (CoG) | The point where the robot's weight is balanced. Low CoG = more stable. That's why the 12V battery goes at the bottom. |
| Common Ground | Connecting the negative/0V references of two separate power supplies. Required for signal wires to carry meaningful values. |
| Continuity Mode | Multimeter setting that beeps when two test points are connected. Essential for verifying wiring before power-on. |
| Corrugated Cardboard | Cardboard with a wavy inner layer. Double-wall = two flute layers. Three laminated layers = structurally rigid. |
| Current | Flow of electric charge, measured in Amperes. The 'water flow rate' of electricity. |
| Delamination | Plywood layers separating. Sign of water damage or structural failure. Never build with delaminated plywood. |
| Digital Signal | A signal that is either HIGH (1) or LOW (0). No middle values. Used for direction pins, wake signals. |
| digitalWrite() | Arduino function. Sets a digital pin HIGH (5V) or LOW (0V). |
| Dirty Power | Power supply shared with motors. Contains electrical noise/spikes. Always isolated from brain boards. |
| Duty Cycle | The percentage of time a PWM signal is HIGH. 50% duty cycle at 12V = effective 6V to the motor. |
| EMI (Electromagnetic Interference) | Electrical noise generated by motors and high-current wires. Can corrupt sensor readings. Route motor wires away from sensor wires. |
| ECHO Pin | HC-SR04 output pin. Stays HIGH for the duration of the echo travel time. |
| Epoxy | Two-part adhesive. Much stronger than wood glue or hot glue. Used for motor mounts and structural joints. |
| ENA / ENB | Enable pins on L298N. Connect to PWM pins for speed control. Remove jumpers before use. |
| Firmware | Low-level software that runs on a microcontroller (like the MicroPython on the Pico or Arduino code on Uno/Mega). |
| GPIO (General Purpose Input/Output) | Programmable pins on a microcontroller or Pi. Can be configured as digital input or output. |
| H-Bridge | Circuit of four transistors that allows motor direction reversal. The L298N contains two H-bridges. |
| HC-SR04 | Model number of the ultrasonic distance sensor used in Jarvis. 4 pins: VCC, GND, TRIG, ECHO. |
| I2C | Serial communication protocol using two wires (SDA + SCL). Not used in Jarvis — USB used instead. |
| IN1, IN2, IN3, IN4 | Direction control inputs on L298N. Combination of HIGH/LOW determines motor forward/backward. |
| Kill-Zone | The 30cm safety radius around Jarvis. If anything enters this zone, the Mega cuts motor power instantly. |
| L298N | Dual H-bridge motor driver integrated circuit. Controls two DC motors from 5V logic signals. |
| LED (Light Emitting Diode) | Component that produces light when current flows through it. Requires a series resistor to limit current. |
| Li-ion | Lithium-ion rechargeable battery chemistry. High energy density. Used for Jarvis's 12V motor battery. |
| Linux | The operating system running on the Raspberry Pi (Raspberry Pi OS = Linux). Non-real-time OS. |
| main.py | The filename the Pico automatically runs on power-up. Must be exactly this name. |
| MicroPython | Stripped-down Python for microcontrollers. Runs on the Raspberry Pi Pico. |
| Motor Driver | Circuit that amplifies low-power control signals into high-power motor current. L298N is our motor driver. |
| Neopixel | Brand name for WS2812B addressable RGB LEDs. Each LED has its own control chip. Single-wire protocol. |
| Nylon Standoff | Plastic spacer bolt. Used to mount circuit boards 5mm above their mounting plate, preventing shorts. |
| Ohm (Ω) | Unit of electrical resistance. Higher ohms = more resistance = less current flow. |
| Ohm's Law | V = I × R. Voltage equals current times resistance. The foundational formula for all circuit calculations. |
| OpenCV | Open Computer Vision library. Used by Pi to process webcam frames, detect faces, track movement. |
| OS (Operating System) | Software that manages hardware (Linux on Pi). Provides file system, networking, process management. |
| Piezoelectric | Material that converts electrical signals to mechanical vibration (sound). Used in HC-SR04. |
| Pilot Hole | A smaller hole drilled before the final sized hole or screw. Prevents wood splitting. |
| PLC | Programmable Logic Controller. Industrial equivalent of Arduino. Not used in Jarvis but same concept. |
| Port | A connection endpoint. USB ports on Pi. Serial port addresses (/dev/ttyACM0) in software. |
| Power Bank | USB rechargeable battery pack. Provides regulated 5V output. Must be 2.1A minimum for Jarvis. |
| PUD_DOWN | Pull-Down resistor. GPIO.PUD_DOWN in Pi code keeps a GPIO pin at LOW until actively driven HIGH. |
| pulseIn() | Arduino function that measures pulse duration in microseconds. Used to read HC-SR04 echo time. |
| PVA Glue | Polyvinyl acetate — standard wood/craft glue (PVA wood glue). Seals cardboard and bonds laminated panels. |
| PVC (Polyvinyl Chloride) | Plastic pipe material. Used as Jarvis's spine. Lightweight, strong, available at hardware stores. |
| PWM (Pulse Width Modulation) | Rapidly switching a digital pin to simulate analog voltage. Used for motor speed control. |
| Raspberry Pi | Single-board Linux computer. Acts as Jarvis's master brain. Runs Python AI code. |
| Raspberry Pi OS | Linux-based operating system for Raspberry Pi. Must use Legacy Bullseye 32-bit for Jarvis. |
| Resistance | Opposition to current flow. Measured in Ohms. Higher resistance = less current at same voltage. |
| Return Statement | In code: return exits the current function immediately. Used in the Mega kill-zone to skip motor commands after an emergency stop. |
| RPM (Revolutions Per Minute) | Motor speed measurement. BO motors' gearbox reduces RPM from 12,000 to ~100–200 RPM for practical wheel movement. |
| Serial Communication | Sending data one bit at a time over a single wire. Arduino USB uses UART serial protocol. |
| SSH (Secure Shell) | Encrypted remote terminal connection. Used to control Pi from a laptop over Wi-Fi. |
| Stall Current | Current drawn by a motor when it's blocked from spinning. Up to 5× running current. Can destroy L298N if sustained. |
| Star Topology | Network design where all devices connect to one central hub. Pi is the hub in Jarvis's USB network. |
| sudo | Linux command prefix that runs a command as administrator. Required for hardware-level commands on Pi. |
| Switch Statement | Code structure (used in Mega) that tests a variable against multiple values. Clean alternative to many if/else statements. |
| Terminal | Command-line interface. Black window where you type Linux commands on Pi or SSH window on laptop. |
| Timeout | A time limit in code. pulseIn(..., 30000) gives up waiting after 30,000µs = prevents hanging forever. |
| Token | Unit of text for AI processing. ~4 characters per token. Gemini measures prompt and response length in tokens. |
| Torso | Main body section of Jarvis. Reinforced cardboard box containing all electronics. |
| TRIG Pin | HC-SR04 input pin. Receives a 10µs HIGH pulse to fire one ultrasonic burst. |
| TTS (Text-to-Speech) | Converting written text to spoken audio. pyttsx3 library handles this on the Pi. |
| UART | Universal Asynchronous Receiver/Transmitter. The serial protocol used by Arduino USB and Pico USB. |
| USB (Universal Serial Bus) | Data+power cable standard. Used for all Pi-to-board connections in Jarvis. |
| Voltage | Electrical pressure difference between two points. 'Height' of the water tower analogy. |
| WS2812B | The actual chip name for NeoPixel LEDs. Uses a single-wire, timing-sensitive protocol. |
| Zero-Turn Chassis | Wheeled design where turning is achieved by spinning wheels at different speeds on the same axle. Allows 360° rotation on the spot. |

END OF THE JARVIS COMPENDIUM

Architect Division · St. Anthony's School · September 16th

Built with care. Built to last. Built to impress.